| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0570 |
| --- | --- |

| Title: | AHG15: reset chroma QP offsets when starting a CTU |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe de Lagrange | Email: | philippe.delagrange@interdigital.com |

| Source: | InterDigital |
| --- | --- |

_____________________________

# Abstract

This contribution, as JVET-Q0126, JVET-Q0267 and JVET-Q0476, exposes the problem #659 reported in JVET bug tracker, namely the propagation of the chroma QP offsets CuQpOffsetCr, CuQpOffsetCr and CuQpOffsetCbCr across the boundaries of regions that should be decodable independently or in parallel.

To solve the problem, this contribution proposes to reset the CuQpOffsetCx variables every CTU (or every CTU row); it states that the reset process potentially creates adverse effects (creating zero-offset “holes” in the chroma QP-offset map), and that resetting the chroma QP-offset more often than necessary is not desirable.

The proposed draft text change consists in moving a 3-lines sentence from sequence header semantics to CTU semantics.

# Problem statement

A problem has been reported in JVET bug tracker ticket #659 with assignment of chroma QP offsets for non-coded CUs, which now have an impact on deblocking after adoption of JVET-P1001.

For leading non-coded CUs in a chroma quantization group, the chroma QP offsets are propagated from previous CUs in coding order. This is illustrated in Figure 1, where CuQpOffsetCx collectively represents CuQpOffsetCb, CuQpOffsetCr and CuQpOffsetCbCr.

Figure 1: Propagation of chroma QP offsets across quantization groups

In VVC draft 7, this propagation can cross Tiles and CTU rows, which is unacceptable. Chroma QP offsets must be reset at least at CTU row start, Tile boundaries and entropy-coding sync points, to enable parallel processing and independent decoding of subpictures as expected in VVC.

Various proposals for the current meeting explain the problem and propose to reset chroma QP offsets at various levels: CQG, CTU, or CTU row. See JVET-Q0126, JVET-Q0267 and JVET-Q0476.

One aspect mentioned in a comment of ticket #659 is reminded here: resetting chroma QP offset potentially creates a “hole”, meaning a zero chroma QP offset for some non-coded CU, breaking the continuity between two areas with specified offsets, as illustrated in Figure 2:

Figure 2: Resetting CuQpOffsetCx may create a “hole”

It is noted that the technical effect of these “holes” is limited to the decision process for deblocking some chroma non-coded block edges.

# Proposal

It is proposed to reset CuQpOffsetCx every CTU. This is asserted to be an acceptable compromise, preventing evil propagation of CuQpOffsetCx while enabling it when not causing problems, avoiding creating too many “holes” with zero chroma QP-offset.

Draft text changes are limited to move a sentence from slice header to CTU semantics, as shown below:

#### 7.4.8.1	General slice header semantics

When present, the value of the slice header syntax element slice_pic_order_cnt_lsb shall be the same in all slice headers of a coded picture.

The variable CuQpDeltaVal, specifying the difference between a luma quantization parameter for the coding unit containing cu_qp_delta_abs and its prediction, is set equal to 0. The variables CuQpOffsetCb, CuQpOffsetCr, and CuQpOffsetCbCr, specifying values to be used when determining the respective values of the Qp′Cb, Qp′Cr, and Qp′CbCr quantization parameters for the coding unit containing cu_chroma_qp_offset_flag, are all set equal to 0.

[…]

#### 7.4.10.2	Coding tree unit semantics

The CTU is the root node of the coding tree structure.

The array IsAvailable[ cIdx ][ x ][ y ] specifying whether the sample at ( x, y ) is available for use in the derivation process for neighbouring block availability as specified in clause 6.4.4 is initialized as follows for cIdx = 0..2, x = x0..x0 + CtbSizeY − 1, and y = y0..y0 + CtbSizeY − 1:

IsAvailable[ cIdx ][ x ][ y ] = FALSE		(154)

The variables CuQpOffsetCb, CuQpOffsetCr, and CuQpOffsetCbCr, specifying values to be used when determining the respective values of the Qp′Cb, Qp′Cr, and Qp′CbCr quantization parameters for the coding unit containing cu_chroma_qp_offset_flag, are all set equal to 0.

[…]

An alternate proposal to reset CuQpOffsetCx even less often is to reset every CTU row, as solution 3 of JVET- Q0126. This can be done by inserting “When the current CTU is the first in a CTU row within a tile, […]” at the beginning of the above-mentioned sentence.

# References

[JVET-P2001]	B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 7)”, JVET-P2001, Geneva, CH, October 2019

[JVET-Q0126]	K. Kawamura, S. Naito, “Initializations and propagation of Chroma QP Offset”, JVET-Q0126, Brussel, BE, January 2020

[JVET-Q0267]	B. Heng, M. Zhou, W. Wan, “AHG16: On Propagation of Chroma CU QP Offsets”, JVET-Q0267, Brussel, BE, January 2020

[JVET-Q0476]	A. K. Ramasubramonian, B. Ray, G. Van der Auwera, M. Karczewicz, “On chroma QP offsets for zero-CBF leading chroma coding blocks”, JVET-Q0476, Brussel, BE, January 2020

# Patent rights declaration(s)

InterDigital does not have any current or pending patent rights relating to the technology described in this contribution.