| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0571 |
| --- | --- |

| Title: | IBC-Mirror mode for screen content coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian Cao, Zhen Qiu, Jun Wang, Fan Liang (SYSU) Yuanfang Yu, Yang Liu (OPPO) | Email: | caoj33@mail2.sysu.edu.cn qiuzh7@mail2.sysu.edu.cn wangj387@mail.sysu.edu.cn isslf@mail.sysu.edu.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | Sun Yat-Sen University, NELVT Guangzhou Research and Industrialization Center, OPPO |  |  |

- Abstract

In this contribution, an IBC-Mirror mode is proposed. Based on the translational motion model of IBC mode, the function of “horizontal and vertical mirror flip” is added to the IBC non-merge mode to further reduce the prediction error and improve the coding efficiency.

The use of IBC-Mirror mode is signaled by two new flags at CU level, including mirrorFlag and mirrorType. The flag named mirrorFlag signals whether to use “mirror flip” function. The flag named mirrorType signals whether to use “vertical mirror flip” (true) or “horizontal mirror flip” (false). Except for IBC non-merge mode, the values of these two flags default to false. The value of mirrorType defaults to false if the value of mirrorFlag is false.

The proposed IBC-Mirror mode is implemented on VTM-7.1. The simulations show BD-rate gains for VTM-7.1 anchor (IBC mode enabled) are:

- In AI, xxx% / xxx % / xxx % for Y, U, V component of SCC 1080p, separately.

- In AI, xxx% / xxx % / xxx % for Y, U, V component of Class F, separately.

- In RA, xxx% / xxx % / xxx % for Y, U, V component of SCC 1080p, separately.

- In RA, xxx% / xxx % / xxx % for Y, U, V component of Class F, separately.

Introduction

Intra block copy (IBC) is proved to be efficient when there are many repeating areas within the same picture. However, IBC is based on translational motion model, which may be not effective for some prediction blocks with mirror characteristic. For example, twenty-six letters often appear in screen content, and most of them have accurate or approximate mirror characteristic, such as ‘A’, ‘B’, ‘C’, ‘D’, ‘E’, ‘H’, ‘I’, ‘K’, ‘M’, ‘O’, ‘T’, ‘U’, ‘V’, ‘W’, ‘X’, ‘Y’.

As shown in Fig,1, to encode a block with content ‘A’ using IBC mode, the block should refer to an encoded block with content ‘A’. If the encoded block beyonds the reference range, it can not be used as the reference block so that IBC mode may cease to be effective.

an encoded block

with content ‘A’

BV

too far to be used

the current block

with content ‘A’

A

A

×

- Fig. 1 An example that IBC mode may cease to be effective.

Using the proposed IBC-Mirror mode can handle this problem. As shown in Fig.2, when encoding the right half (b) of the block, the left half (a) can be flipped horizontally first and then used as the reference block of the right half (b), which can significantly improve the coding efficiency.

(1)            (2)             (3)              (4)

- Fig. 2 An example that the proposed IBC-Mirror mode can be effective. (1) a block with content ‘A’; (2) the left half; (3) the horizontal mirror image of the left half; (4) the right half.

The reference relation of blocks using “horizontal mirror flip” is shown in Fig.3 below.

(1)                  (2)

- Fig. 3 The reference relation using “horizontal mirror flip”. (1) reference block; (2) prediction block.

The reference relation of blocks using “vertical mirror flip” is shown in Fig.4 below.

(1)                  (2)

- Fig. 4 The reference relation using “vertical mirror flip”. (1) reference block; (2) prediction block.

Proposed methods

Encoder aspect:

The function of “horizontal and vertical mirror flip” is added to the IBC non-merge mode. When the Hash-based IBC search can not provide valid BV candidates, original BV search process and the proposed IBC-Mirror judging process will be applied.

The proposed IBC-Mirror judging process is described below.

1. For the current block (size: W*H), derive the reference block based on the valid BV=(x, y) by BV search process.

2. Let OrgBuf and PredBuf (two W*H matrices) store the pixels of the current block and the reference block respectively.

3. Calculate SAD between OrgBuf and PredBuf, written as SAD_org.

4. Use “horizontal mirror flip” for PredBuf and store the changed pixels in PredBuf_hor.

5. Calculate SAD between OrgBuf and PredBuf_hor, written as SAD_hor.

6. Use “vertical mirror flip” for PredBuf and store the changed pixels in PredBuf_ver.

7. Calculate SAD between OrgBuf and PredBuf_ver, written as SAD_ver.

8. Choose the minimum among SAD_org, SAD_hor and SAD_ver. If the minimum is SAD_org, set both mirrorFlag and mirrorType to false. If the minimum is SAD_ver, set both mirrorFlag and mirrorType to true. If the minimum is SAD_hor, set mirrorFlag to true and mirrorType to false.

After BV search process, store the best BV, CU.mirrorFlag and CU.mirrorType. In the stage of motion compensation, derive the reference block based on the best BV first. Then use the corresponding “mirror flip” function according to CU.mirrorFlag and CU.mirrorType. The subsequent process is consistent with the original encoding process.

Decoder aspect:

Obtain the BV of the current block, CU.mirrorFlag and CU.mirrorType. Derive the reference block based on the BV first. Then use the corresponding “mirror flip” function according to CU.mirrorFlag and CU.mirrorType. The subsequent process is consistent with the original decoding process.

Inference of the IBC-Mirror mode:

- Fig. 5 Inference of the IBC-Mirror mode

Experimental Results

The proposed IBC-Mirror mode is implemented on VTM-7.1. QP values are chosen as 22, 27, 32 and 37 in AI and RA case. Other test configurations follow the requirement of reference [1]. Results are shown in the following tables:

- Tab. 1 Experimental results

|  | All Intra Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.1 + IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F |  |  |  |  |  |
| Class SCC 1080p |  |  |  |  |  |

|  | Random Access Main 10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.1 + IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F |  |  |  |  |  |
| Class SCC 1080p |  |  |  |  |  |

Some coding gains for screen content materials are observed from the experimental results. However, the proposed IBC-Mirror mode introduces a lot of coding time. In further study, some fast decision algorithms should be developed and used to reduce coding time.

Conclusions

In this contribution, a new IBC-Mirror mode is proposed and tested. Some coding gains for screen content materials are observed from the experimental results. It is recommended to include this method into the test model for further consideration.

References

[1]	X. X, Y. Sun, Y. Chao and J. Xu, " Description of Core Experiment 8: Screen Content Coding Tools," JVET-L1028, Macao, CN, Oct. 2018.

Patent rights declaration(s)

OPPO and NELVT Guangzhou Research and Industrialization Center may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sun Yat-Sen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).