| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0573 |
| --- | --- |

| Title: | Intra-Affine mode for screen content coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian Cao, Huang Wang, Jun Wang, Fan Liang (SYSU) Yuanfang Yu, Yang Liu (OPPO) | Email: | caoj33@mail2.sysu.edu.cn wangh267@mail2.sysu.edu.cn wangj387@mail.sysu.edu.cn isslf@mail.sysu.edu.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | Sun Yat-Sen University, NELVT Guangzhou Research and Industrialization Center, OPPO |  |  |

- Abstract

In this contribution, intra-affine mode is proposed for screen content coding (SCC). In this mode, affine model is introduced to intra prediction. The use of intra-affine mode is signaled by using the IBC mode flag and the Affine flag. These two flags are both true when the proposed mode is used. The proposed intra-affine mode is implemented on VTM-7. The simulations show BD-rate gains for VTM-7 anchor (IBC mode enabled) are:

- In AI, xxx% / xxx % / xxx % for Y, U, V component of SCC 1080p, separately.

- In AI, xxx% / xxx % / xxx % for Y, U, V component of Class F, separately.

- In RA, xxx% / xxx % / xxx % for Y, U, V component of SCC 1080p, separately.

- In RA, xxx% / xxx % / xxx % for Y, U, V component of Class F, separately.

Introduction

In JVET-M0634 [1], we preliminary proposed a method which combined the affine motion mode with IBC/CPR (CPRAffine). However, the improvement of coding efficiency of CPRAffine was not significant.

In JVET-O0682 [2], we improved the process and proposed the intra-affine mode. However, numbers of availability check at both encoder and decoder aspect were large to ensure that all 4x4 subblocks should point to an available decoded area.

In this contribution, availability check operation is removed at decoder aspect considering that encoding process could be made always guarantee the finally selected subblock BV is valid.

Proposed methods

Encoder aspect:

The encoder performs RD check for blocks whose sizes are equal to or larger than  and finds the best mode among three modes (regular intra mode, IBC mode and intra-affine mode).

As shown in Fig. 1, the process has two main steps: 1) selecting initial BVAffis (block-affine vectors, consist of BV of two control points) and 2) iterative searching for the best BVAffi.

1) Initial BVAffis can be obtained by inheritance and local search.

- Inheritance way means that initial BVAffis can be obtained by reusing the motion information of encoded spatial neighboring blocks (BV in IBC mode and BVAffi in proposed mode).

- Local search way means that initial BVAffis are obtained by searching in the encoded area. The way of searching is similar to the traditional inter prediction. Searching area is limited to the current CTU and searching step length is set to 8 pixels.

2) The way of iterative searching is similar to the affine motion inter prediction. The major difference is that the reference picture is the current picture itselfe.

Subpel interpolation is not used in the proposed intra-affine mode. A 4x4 subblock BV is adjusted to an integral multiple of 16 to indicate a location of a reference subblock.

An additional check is performed to determine whether a reference subblock indicated by a 4x4 subblock BV (adjusted to integer pel precision) is in a reconstructed area of the current picture or not. If not, BV of control point is invalid. The 4x4 subblock BV is derived based on BV of two control points and the location of the subblock, which is the same as the process for inter affine mode. Availability check is performed for all 4x4 subblocks at encoder aspect in the current implementation.

- Fig. 1  The overall algorithm flowchart

Decoder aspect:

4-parameter affine model is used in intra-affine mode. MVD of the two control points is signalled in CU (i.e. in MODE_IBC branch after a flag indicating intra-affine mode). MVPcand for intra-affine is established in a similar way to that for inter affine mode. MVP of the two control points are obtained using MVP_index. CPMV is calculated using MVP and MVD.

If the affine flag and the IBC mode flag are both true, the proposed intra-affine mode can be inferred.

Availability check is removed at decoder aspect considering that encoding process could be made always guarantee the finally selected subblock BV is valid.

- Fig. 2  Inference of the intra-affine mode

Experimental Results

The proposed intra-affine mode is implemented on VTM-7. QP values are chosen as 22, 27, 32 and 37 in AI and RA case. Some other test configurations and test sequences follow the requirement of reference [3]. VTM-6.0 (IBC mode enabled) is used as the anchor. Results are shown in the following tables:

- Tab. 1  Experimental results

|  | All Intra Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7 + IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F |  |  |  |  |  |
| Class SCC 1080p |  |  |  |  |  |

|  | Random Access Main 10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7 + IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F |  |  |  |  |  |
| Class SCC 1080p |  |  |  |  |  |

Conclusions

In this contribution, a new intra-affine mode is proposed and tested. Some coding gains for screen content materials are observed from the experimental results. It is recommended to include this method into the test model for further consideration.

References

[1]	S. Cao, H. Han, J. Wang, F. Liang, Y. Yu and Y. Liu, "Affine motion mode in intra coding," JVET-M0634, Marrakech, MA, Jan. 2019.

[2]	J. Cao, Z. Li, J. Wang, F. Liang, Y. Yu and Y. Liu, "CE8-related: An Intra-Affine mode for screen content coding," JVET-O0682, Gothenburg, SE, July. 2019.

[3]	X. X, Y. Sun, Y. Chao and J. Xu, " Description of Core Experiment 8: Screen Content Coding Tools," JVET-L1028, Macao, CN, Oct. 2018.

Patent rights declaration(s)

OPPO and NELVT Guangzhou Research and Industrialization Center may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sun Yat-Sen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).