| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0577 |
| --- | --- |

| Title: | Crosscheck of JVET-Q0307 (CE4-related: Block-dimension based GEO mode selection) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Lidong Xu, Huan Dou, Yi-jen Chiu 8F Raycom Infotech Park A, Haidian District, Beijing, 100029, China | Tel: Email: | +86-10-82611515 lidong.xu@intel.com huan.dou@intel.com yi-jen.chiu@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

This contribution reports cross-checking results for JVET-Q0307 (CE4-related: Block-dimension based GEO mode selection) which proposes to reduce the number of GEO modes according to block dimensions. Two methods were proposed in JVET-Q0307. Method #1 reduces the per-shape GEO modes from 82 to 32 and defines a table for the list of available GEO modes of each block shape. Method #2 proposes to further reduce the per-shape GEO modes from 32 to 16. The crosschecker verified that the software implementation matches the description and the simulation BD-rate results reportedly match those provided by the proponents.

# Verification

The proposal JVET-Q0037 proposes to reduce the number of GEO modes according to block dimensions. Two methods were proposed in JVET-Q0307. Method #1 reduces the per-shape GEO modes from 82 to 32 and defines a table for the list of available GEO modes of each block shape. Method #2 proposes to further reduce the per-shape GEO modes to 16. Each method is evaluated on top of two CE tests, i.e., “CE4-1 + CE4-2.1” and “CE4-1 + CE4-2.1 + CE4-3.1”. Totally four test cases were evaluated and were compared with two different anchors of CE4 common base and VTM7.0, respectively. It was verified that the implementation in the provided software matches the description, and the BD rates reported in the proposal were reproduced. The encoding and decoding time in below tables are not accurate.

## Test-1: CE4-1+CE4-2.1+Method #1

Table 1 and 2 showed simulation results of Test-1 with the anchors being CE4 common base and VTM7.0, respectively.

Table 1. Performance of Test-1 over CE4 common base

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0_CE4_common_base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.10% | -0.02% | 98% | 100% |
| Class A2 | 0.04% | 0.04% | -0.06% | 98% | 100% |
| Class B | -0.02% | 0.03% | 0.03% | 98% | 99% |
| Class C | 0.02% | 0.20% | 0.13% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.05% | 0.03% | 98% | 100% |
| Class D | 0.02% | -0.05% | -0.03% | 99% | 100% |
| Class F | 0.02% | -0.03% | -0.05% | 99% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0_CE4_common_base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.01% | 0.11% | 96% | 98% |
| Class C | 0.03% | 0.02% | -0.08% | 100% | 98% |
| Class E | 0.23% | -0.25% | 0.12% | 97% | 96% |
| Overall | 0.08% | -0.05% | 0.05% | 97% | 97% |
| Class D | -0.08% | -0.27% | -0.09% | 98% | 101% |
| Class F | 0.00% | -0.02% | -0.16% | 99% | 101% |

Table 2. Performance of Test-1 over VTM7.0

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.20% | -0.20% | 97% | 99% |
| Class A2 | -0.17% | -0.15% | -0.08% | 100% | 99% |
| Class B | -0.16% | -0.08% | -0.14% | 100% | 100% |
| Class C | -0.57% | -0.79% | -0.77% | 102% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.26% | -0.31% | -0.31% | 100% | 100% |
| Class D | -0.41% | -0.71% | -0.65% | 105% | 110% |
| Class F | -0.30% | -0.41% | -0.38% | 99% | 107% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.33% | -0.50% | -0.48% | 99% | 99% |
| Class C | -0.76% | -1.23% | -0.97% | 103% | 101% |
| Class E | -0.94% | -0.43% | -0.76% | 97% | 98% |
| Overall | -0.62% | -0.72% | -0.71% | 100% | 99% |
| Class D | -0.76% | -0.73% | -1.13% | 101% | 114% |
| Class F | -0.50% | -0.93% | -0.46% | 100% | 106% |

## Test-2: CE4-1+CE4-2.1+CE4-3.1+Method #1

Table 3 and 4 showed simulation results of Test-2 with the anchors being CE4 common base and VTM7.0, respectively.

Table 3. Performance of Test-2 over CE4 common base

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0_CE4_common_base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.07% | 0.06% | 98% | 101% |
| Class A2 | 0.06% | 0.10% | -0.03% | 99% | 101% |
| Class B | 0.02% | 0.04% | 0.02% | 99% | 100% |
| Class C | 0.10% | 0.16% | 0.11% | 99% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.06% | 0.04% | 99% | 101% |
| Class D | 0.07% | 0.00% | 0.24% | 99% | 100% |
| Class F | 0.06% | 0.05% | 0.05% | 99% | 101% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0_CE4_common_base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.09% | 0.00% | 0.65% | 95% | 97% |
| Class C | 0.11% | 0.10% | 0.06% | 97% | 99% |
| Class E | 0.28% | -0.09% | 0.38% | 97% | 100% |
| Overall | 0.14% | 0.01% | 0.39% | 96% | 98% |
| Class D | 0.10% | -0.16% | 0.25% | 97% | 102% |
| Class F | 0.03% | -0.11% | 0.10% | 100% | 101% |

Table 4. Performance of Test-2 over VTM7.0

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.16% | -0.12% | 96% | 100% |
| Class A2 | -0.16% | -0.09% | -0.05% | 101% | 99% |
| Class B | -0.12% | -0.06% | -0.15% | 101% | 101% |
| Class C | -0.49% | -0.83% | -0.79% | 101% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.22% | -0.29% | -0.29% | 100% | 101% |
| Class D | -0.36% | -0.67% | -0.38% | 105% | 110% |
| Class F | -0.26% | -0.33% | -0.28% | 99% | 109% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.27% | -0.50% | 0.06% | 98% | 98% |
| Class C | -0.68% | -1.15% | -0.83% | 100% | 102% |
| Class E | -0.89% | -0.28% | -0.51% | 98% | 102% |
| Overall | -0.56% | -0.66% | -0.38% | 99% | 100% |
| Class D | -0.58% | -0.60% | -0.79% | 100% | 116% |
| Class F | -0.48% | -1.01% | -0.20% | 101% | 106% |

## Test-3: CE4-1+CE4-2.1+Method #2

Table 5 and 6 showed simulation results of Test-3 with the anchors being CE4 common base and VTM7.0, respectively.

Table 5. Performance of Test-3 over CE4 common base

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0_CE4_common_base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | 0.12% | 0.18% | 98% | 99% |
| Class A2 | 0.13% | 0.12% | 0.12% | 99% | 101% |
| Class B | 0.10% | 0.15% | 0.18% | 99% | 101% |
| Class C | 0.31% | 0.54% | 0.56% | 100% | 104% |
| Class E |  |  |  |  |  |
| Overall | 0.16% | 0.24% | 0.27% | 99% | 102% |
| Class D | 0.19% | 0.23% | 0.37% | 99% | 102% |
| Class F | 0.14% | 0.18% | 0.15% | 100% | 102% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0_CE4_common_base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.20% | 0.23% | 0.55% | 97% | 99% |
| Class C | 0.49% | 0.69% | 0.70% | 98% | 101% |
| Class E | 0.71% | 0.16% | 0.67% | 97% | 101% |
| Overall | 0.43% | 0.37% | 0.63% | 97% | 100% |
| Class D | 0.30% | 0.03% | 0.76% | 96% | 100% |
| Class F | 0.18% | 0.42% | 0.35% | 99% | 103% |

Table 6. Performance of Test-3 over VTM7.0

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.03% | 0.00% | 96% | 98% |
| Class A2 | -0.09% | -0.07% | 0.10% | 101% | 100% |
| Class B | -0.04% | 0.04% | 0.00% | 102% | 102% |
| Class C | -0.28% | -0.45% | -0.35% | 102% | 106% |
| Class E |  |  |  |  |  |
| Overall | -0.12% | -0.11% | -0.07% | 100% | 102% |
| Class D | -0.24% | -0.44% | -0.25% | 105% | 113% |
| Class F | -0.18% | -0.21% | -0.18% | 100% | 111% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.15% | -0.28% | -0.04% | 100% | 100% |
| Class C | -0.31% | -0.57% | -0.20% | 101% | 104% |
| Class E | -0.47% | -0.04% | -0.22% | 98% | 104% |
| Overall | -0.28% | -0.31% | -0.14% | 100% | 102% |
| Class D | -0.38% | -0.43% | -0.29% | 99% | 113% |
| Class F | -0.33% | -0.50% | 0.04% | 101% | 109% |

## Test-4: CE4-1+CE4-2.1+CE4-3.1+Method #2

Table 7 and 8 showed simulation results of Test-4 with the anchors being CE4 common base and VTM7.0, respectively.

Table 7. Performance of Test-4 over CE4 common base

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0_CE4_common_base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | -0.01% | 0.14% | 98% | 100% |
| Class A2 | 0.17% | 0.20% | 0.09% | 99% | 102% |
| Class B | 0.10% | 0.20% | 0.18% | 98% | 100% |
| Class C | 0.37% | 0.58% | 0.55% | 99% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.19% | 0.26% | 0.25% | 98% | 101% |
| Class D | 0.27% | 0.42% | 0.40% | 98% | 102% |
| Class F | 0.13% | 0.21% | 0.15% | 99% | 101% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0_CE4_common_base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.28% | 0.30% | 0.30% | 96% | 100% |
| Class C | 0.53% | 0.78% | 0.63% | 96% | 101% |
| Class E | 0.70% | 0.00% | 0.67% | 97% | 101% |
| Overall | 0.47% | 0.38% | 0.50% | 96% | 101% |
| Class D | 0.40% | -0.08% | 0.43% | 95% | 97% |
| Class F | 0.14% | 0.22% | 0.21% | 101% | 107% |

Table 8. Performance of Test-4 over VTM7.0

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.10% | -0.04% | 96% | 99% |
| Class A2 | -0.05% | 0.01% | 0.07% | 101% | 100% |
| Class B | -0.04% | 0.10% | 0.01% | 100% | 101% |
| Class C | -0.22% | -0.41% | -0.35% | 100% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.10% | -0.09% | 100% | 102% |
| Class D | -0.16% | -0.25% | -0.22% | 104% | 112% |
| Class F | -0.19% | -0.17% | -0.18% | 99% | 109% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.08% | -0.21% | -0.30% | 99% | 101% |
| Class C | -0.27% | -0.48% | -0.26% | 99% | 105% |
| Class E | -0.48% | -0.20% | -0.22% | 97% | 104% |
| Overall | -0.24% | -0.30% | -0.26% | 99% | 103% |
| Class D | -0.29% | -0.56% | -0.61% | 97% | 110% |
| Class F | -0.37% | -0.69% | -0.09% | 102% | 112% |