| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0582-v1 |
| --- | --- |

| Title: | Harmonization of JVET-Q0451 and JVET-Q0453 for MIP & LFNST design |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo, Shuai Wan, Xinwei Li, Haixin Wang, Fuzheng Yang, Yanzhuo Ma, Yuanfang Yu, Yang Liu | Email: | jyhuo@mail.xidian.edu.cn swan@nwpu.edu.cn xinweili@stu.xidian.edu.cn hxwang2@mail.xidian.edu.cn fzhyang @mail.xidian.edu.cn yzma@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | Xidian University, Northwestern Polytechnical University, Guangdong OPPO Mobile Telecommunications Corp., Ltd. |  |  |

_____________________________

- Abstract

Proposed is a harmonization of the methods proposed in JVET-Q0451 and JVET-Q0453. It is reported that, under the CTC, an average performance for {Y, U, V} is {-0.11%, -0.06%, -0.01%} for AI and {-0.07%, 0.02%, 0.10%} for RA over VTM 7.0.

Introduction

In VVC Draft 7[1], mipSizeId is set equal to 1 for 8x8 block. LFNST is enabled for an MIP block with its width and height greater than or equal to 16 with a fixed configuration of LFNST transform set and LFNST transpose flag.

In JVET-Q0451[2], it is proposed that mipSizeId is set to 2 for 8x8 block;

In JVET-Q0453[3], it is proposed that LFNST transform set and LFNST transpose flag are determined according to an MIP mode index and MIP transpose flag of an MIP block, respectively.

Proposed Methods

In this contribution, a harmonization of JVET-Q0451 and JVET-Q0453 is proposed, that is,

- mipSizeId is set to 2 for 8x8 block;

- LFNST transform set and LFNST transpose flag are determined according to an MIP mode index and MIP transpose flag of an MIP block, respectively.

Experimental results

The experiments were performed based on VTM7.0 software[4] under JVET common test condition [5]. The coding performance is provided in Table 1.

Table 3. Simulation results of the proposed method

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -0.25% | 0.05% | 102% | 100% |
| Class A2 | -0.08% | 0.04% | -0.02% | 101% | 100% |
| Class B | -0.10% | -0.04% | -0.04% | 102% | 100% |
| Class C | -0.06% | -0.01% | -0.08% | 101% | 101% |
| Class E | -0.11% | -0.09% | 0.10% | 100% | 99% |
| Overall | -0.11% | -0.06% | -0.01% | 101% | 100% |
| Class D | -0.05% | -0.03% | -0.06% | 101% | 99% |
| Class F | -0.06% | -0.05% | 0.16% | 100% | 99% |

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | 0.05% | 0.07% | 100% | 100% |
| Class A2 | -0.08% | -0.08% | 0.07% | 101% | 100% |
| Class B | -0.05% | 0.10% | 0.19% | 101% | 100% |
| Class C | -0.03% | -0.01% | 0.02% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | 0.02% | 0.10% | 101% | 100% |
| Class D | -0.05% | -0.22% | -0.07% | 101% | 99% |
| Class F | -0.05% | -0.05% | -0.07% | 100% | 99% |

Conclusions

This contribution presents a harmonization of of JVET-Q0451 and JVET-Q0453 for MIP and LFNST design. The performance of the harmonized method is -0.11% (AI) and -0.07% (RA) over VTM7.0. It is recommended to adopt this method in the next version of VVC draft.

- 5 References

[1] B. Bross, J. Chen, S. Liu, Y.-K. Wang, Verstatile Video Coding (Draft 7), JVET document JVET-P2001, Version 14 , Geneva, Switzerland, October 2019

[2] https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-7.0

[3] J.-Y. Huo, H.-X. Wang, X.-W. Li et al., On mipSizeId modification for 8x8 blocks, JVET document JVET-Q0451, Brussels, Belgium, January 2020

[4] 	J.-Y. Huo, W.-H. Qiao, X.-W. Li et al., Modification of LFNST for MIP coded block, JVET document JVET-Q0453, Brussels, Belgium, January 2020

[5] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET common test conditions and software reference configurations for SDR video, JVET document JVET-N1010, Geneva, Switzerland, March 2019

- 6 Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).