| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0585-v1 |
| --- | --- |

| Title: | CE5-related: cross-check report of JVET-Q0125 on JC-CCALF with Alternative Filter Weight |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. François | Tel: Email: | edouard.francois@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution reports cross-checking results for contribution JVET-Q0125 on JC-CCALF with Alternative Filter Weight. The cross-checking tests were performed for SDR CTCs, in AI, RA and LDB configurations. The results reportedly match those provided by the proponents for the tested configurations. Encoding/Decoding runtimes are reported to be of same order as the ones reported by the proponent.

# Introduction

JVET-Q0125 proposes to use individual JC-CCALF filter weight for each alternative filter. Specifically, instead of using various weight values as in JC-CCALF (see for instance CE5-1.4), only sign changes are applied for 4 alternative filters, e.g., +--+. In the decoder, the current CTU selects the filter weight according to the parsed alternative filter index.

# Experimental Results

Tests were performed using SDR CTCs for AI, RA and LBD configurations.

The cross-checking was achieved using the following computing specifications:

- CPU type:	Xeon

- Hyperthreading:	off

- Turbo mode:	on

- Compiler:	gcc 6.3.1

- OS:	CentOS7

- Decoding options

  - MD5 SEI:	no

  - YUV output:	yes

- SIMD options:	SSE42

No significant encoder/decoder runtime difference was observed compared to the runtimes reported in JVET-Q0125.

For all the tested simulations, a perfect match is obtained compared to the proponent results. Related xls file is attached.

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | -4.06% | -3.61% | 101% | 101% |
| Class A2 | 0.08% | -5.46% | -10.29% | 100% | 100% |
| Class B | 0.09% | -10.73% | -7.77% | 100% | 99% |
| Class C | 0.08% | -3.31% | -3.22% | 101% | 101% |
| Class E | 0.11% | -8.04% | -2.20% | 101% | 101% |
| Overall | 0.10% | -6.65% | -5.56% | 101% | 100% |
| Class D | 0.07% | -3.23% | -2.05% | 101% | 101% |
| Class F | 0.07% | -4.89% | -5.40% | 99% | 101% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.14% | -5.37% | -5.22% | 100% | 103% |
| Class A2 | 0.15% | -9.48% | -14.53% | 101% | 99% |
| Class B | 0.20% | -16.01% | -12.84% | 102% | 103% |
| Class C | 0.08% | -4.08% | -3.49% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.15% | -9.39% | -9.16% | 101% | 102% |
| Class D | 0.05% | -4.37% | -3.41% | 101% | 100% |
| Class F | 0.11% | -3.68% | -3.76% | 101% | 101% |
|  |  |  | Low delay B |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.15% | -19.05% | -17.17% | 100% | 100% |
| Class C | 0.15% | -5.42% | -4.50% | 101% | 101% |
| Class E | -0.01% | -13.14% | -5.83% | 102% | 101% |
| Overall | 0.11% | -13.03% | -10.11% | 101% | 100% |
| Class D | 0.08% | -5.99% | -4.32% | 103% | 101% |
| Class F | 0.01% | -3.95% | -2.88% | 101% | 104% |