| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0586-v1 |
| --- | --- |

| Title: | AHG9/AHG12: A summary of HLS contributions on tiles and slices |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hendry | Email | dr.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution is for information contribution summarizing contributions on tiles and slices (i.e., contributions category 6.20.2).

# On tiles

- JVET-Q0244 aspect #1: signal a flag to specify whether the explicitly signalled tile row is the same as the explicitly signalled tile column. If true, tile row information can be derived from tile column information.

- JVET-Q0244 aspect 6: Propose fixes for out of bound bugs in the derivation of CtbToTileColBd[ ] and CtbToTileRowBd[ ]. The proposed fixes are shown below:

The list CtbToTileColBd[ ctbAddrX ] for ctbAddrX ranging from 0 to PicWidthInCtbsY − 1, inclusive, specifying the conversion from a horizontal CTB address to a left tile column boundary in units of CTBs, is derived as follows:

tileX = 0
for( ctbAddrX = 0; ctbAddrX  <=  PicWidthInCtbsY; ctbAddrX++ ) {
	if( tileX < NumTileColumns – 1  &&  ctbAddrX  = =  tileColBd[ tileX + 1 ] )	(27)
		tileX++
	CtbToTileColBd[ ctbAddrX ] = tileColBd[ tileX ]
}

The list CtbToTileRowBd[ ctbAddrY ] for ctbAddrY ranging from 0 to PicHeightInCtbsY − 1, inclusive, specifying the conversion from a vertical CTB address to a top tile column boundary in units of CTBs, is derived as follows:

tileY = 0
for( ctbAddrY = 0; ctbAddrY  <=  PicHeightInCtbsY; ctbAddrY++ ) {
	if( tileY < NumTileRows – 1  &&  ctbAddrY  = =  tileRowBd[ tileY + 1 ] )	(28)
		tileY++
	CtbToTileRowBd[ ctbAddrY ] = tileRowBd[ tileY ]
}

- JVET-Q0359 aspect 1: propose the following:

- Constrain that the width or the height of each tile shall not larger than the width of the picture and the height of the picture in unit of CTB, respectively.

- Constrain that that the sum of the all tile width and the sum of all tile height should less than or equal to the width or height of the picture in unit of CTB, respectively

- JVET-Q0359 aspect 2: Add the following: for i in the range of 0 to num_slices_in_pic_minus1-1, inclusive , the sum of tile_idx_delta[ i ] shall less than NumTilesInPic.

# On rectangular slices containing one or more tile

- Main concept of signalling rectangular slice:

  - Keep the current signalling method by signalling slice width, slice height, and tile idx delta. If we do this, we’ll need to consider all miscellaneous proposed improvement in 3.

  - Go back to previous signalling method such as for each rectangular slice, signal only the information about the bottom-right tile index (JVET-Q0202). If we do this, we can skip discussing most proposed improvements in 3 below, except 3.h.

- Proposed additional features for rectangular slices:

  - Allow possibility to define rectangular slice in PPS based on CTU height. This will allow defining rectangular slice within subpicture without tiles (JVET-Q0164).

  - To have single_tile_per_slice_flag and use it to condition further signalling of syntax elements related to rect slices. Also condition the presence of single_tile_per_slicle_flag on the value of single_slice_per_subpic_flag (JVET-Q0204, JVET-Q218 aspect 2, and JVET-Q0289 aspect 1)

    - loop_filter_across_tiles_enabled_flag is not present when the value of single_tile_per_slice_flag is equal to 1 (JVET-Q0289 aspect 1)

  - Remove signalling of num_slice_in_pic_minus1 and introduce num_slice_in_subpic_minus1[ i ] (JVET-Q0377 aspect 1).

- Misc proposed changes for rectangular slice signalling:

  - Condition the presence of tile_idx_delta_present_flag on the value of num_slices_in_pic_minus1. It is asserted that tile_idx_delta_present_flag is not necessary to be present when the value of num_slices_in_pic_minus1 is equal to 0 (JVET-Q0218 aspect 1).

  - Add inference value for single_slice_per_subpic_flag when not present to be equal to 0. This is asserted to be needed since the flag is used in the conditioning of syntax elements in PPS and also in the derivation of CTB raster scanning, tile scanning, and subpicture scanning processes. (JVET-Q0218 aspect 3).

  - Replace tile_idx_delta[ i ] with tile_idx_delta_minus1[ i ] and tile_idx_delta_sign[ i ]. tile_idx_delta_sign[ i ] is not present for the first slice (JVET-Q0228 aspect 1).

  - Signal the following two new flags (JVET-Q0230):

- multiple_slice_in_tile_present_flag indicates whether at least one tile is composed of more than slice or not

- multiple_slice_in_tile_flag[i] indicates whether a tile is composed of more than one slice or not.

  - Signal a flag for each rectangular slice in the picture to indicate if the slice height in tiles is the same as the slice width in tiles. If this flag is false, then the slice height is explicitly signalled (JVET-Q0244 aspect 2).

  - Signal rectangular slice width in tile only when number of tile column is greater than 1; likewise, signal rectangular slice height only when the number of tile row is greater than 1(JVET-Q0244 aspect 3).

  - Remove the second condition that guard the presence of tile_idx_delta[ i ] (i.e., i < num_slices_in_pic_minus1) is not really needed as it is always true anyway (JVET-Q0244 aspect 5).

  - Replace the signaling of the number of slices in picture with the signaling of the delta between the number of subpictures in picture and the number of slices in picture. Instead of num_slices_in_pic_minus1, signal num_slices_in_pic_minus_num_subpics (JVET-Q0332).

  - Proposed that when in raster rectangular slice mode (tile_idx_delta_present_flag equals 0), only specify the slice_height_in_tiles_minus1 syntax element at the start of each tile row. If it is asserted that this would prevent possibility to signal slice height that is illegal (JVET-Q0480)

  - Editorial changes for semantics of tile_idx_delta[ i ] and num_tiles_in_slice_minus1 (JVET-Q0289 aspect 3).

# On rectangular slices within a tile

- Remove signalling of slices within a tile. It is asserted initial concept of “tile-fraction” slices targets bitstream extraction and merging operations for omnidirectional content for which subpictures where adopted in VVC (JVET-Q0377 aspect 2). If we do this, we do not need to discuss proposed items below.

- For signalling of slices within a tile, instead of signalling all the slice height, except for the last slice in that tile, signal explicit slice height, which may be less than the number of slices within that tile minus 1, and the height of the rest of slices in that tile is derived (JVET-Q0203, JVET-Q0228 aspect 2, and JVET-Q0373 aspect 3)

- Signal the number of rectangular slices in a tile only when the number of CTU rows within the tile is greater than 1 (JVET-Q0244 aspect 4 and JVET-Q0373 aspect 2).

- Propose to infer the syntax element of slice_height_in_ctu_minus1 as value of 0 when the corresponding tile’s height is the same as the number of slice in the tile in the unit of CTU (JVET-Q0373 aspect 1). This would not be needed if we take action on item 2 above.

# On other aspects of slices

- JVET-Q0119 aspect 6: When rect_slice_flag is equal to 1, the length of slice_address is specified to be Max( Ceil( Log2( NumSlicesInSubpic[ SubPicIdx ] ) ), 1 ) bits, as opposed to be Ceil( Log2( NumSlicesInSubpic[ SubPicIdx ] ) ) bits. This seems to be a bug fix proposal.

- The following may need to be discussed:

  - JVET-Q0201: Asserted that there are some problems with slice_data signalling and proposes fixes for them

  - JVET-Q0223: Proposes to bring back dependent slice.

  - JVET-Q0224 & JVET-Q0225: Allow CTU-based raster-scan slices, in addition to tile-based raster-scan slice.

# List of reviewed contributions

JVET-Q0164 AHG9: On indication of rectangular slice height in video subpictures [P. Wu (ZTE)]

This contribution asserts that rectangular slice is not flexible such that it cannot be defined without involving tile.

Propose to allow possibility to define rectangular slice in PPS based on CTU height. This will allow defining rectangular slice within subpicture without tiles.

Comments:

Currently we actually can have the case of multiple subpictures with each slice is one subpicture without tiles. Simply have the signalling of subpictures in SPS, set the value of no_pic_partition_flag equal to 0, set the number of tile column and tile row to be equal to 1 and then set the value of single_slice_per_subpic_flag equal to 1.

Then the question is if we don’t want to involve tiles, do we need to have multiple slices in subpicture?

The difference between what being proposed and what the design in the spec is that we can have less partition within subpicture since no tile is involved. If the proposed is needed in many use-cases / major use-case, may be it is OK. If not, the current spec can do it although it is less efficient.

JVET-Q0202 [AHG12]: On signalling of subpicture and rectangular slice [Hendry, S. Paluri (LGE)]

This contribution proposes to change the signalling of rectangular slice by signalling the information of the bottom-right tile, instead of signalling slice width and height. Bit-saving analysis shows that compared to the current signalling method, for the given example, the proposed method saves 14%.

Similar signalling is also proposed for subpicture and for the given example, the proposed method saves 60%.

JVET-Q0203 [AHG12]: On signalling of multiple rectangular slices in a tile [Hendry, S. Paluri (LGE)]

This contribution proposes that for signalling of slices within a tile, instead of signalling all the slice height, except for the last slice in that tile, signal explicit slice height, which may be less than the number of slices within that tile minus 1, and the height of the rest of slices in that tile is derived.

This is asserted the same technique that is adopted last meeting for unifying uniform and non-uniform tile signalling.

This is similar to aspect #2 in JVET-Q0228. Same idea, some differences in text implementation.

JVET-Q0204 [AHG12]: On single tile per slice flag [Hendry, S. Paluri (LGE)]

This contribution proposes to have single_tile_per_slice_flag. When this flag is equal to 1, no signalling for number of slice in picture, slice width, and slice height. These 3 are inferred.

Condition the presence of single_tile_per_slice_flag on the value of single_slice_per_subpic_flag (i.e., not present when single_slice_per_subpic_flag is equal to 1).

This is similar to JVET-Q0218 aspect #2. Just minor difference in text implementation.

Comments:

The difference between single_tile_per_slice_flag and the existing flag single_slice_per_subpic_flag is that single_slice_per_subpic_flag allows a tile to be split into one or more slices. This may be good for 3060 video use-cases. On the other hand, single_tile_per_slice_flag specifically say each tile is one slice. This seems to be more for parallel processing use-cases.

JVET-Q0218 AHG12/ AHG9: On Slice Signalling [S. Deshpande (Sharp)]

Aspect 1: propose to condition the presence of tile_idx_delta_present_flag on the value of num_slices_in_pic_minus1. It is asserted that tile_idx_delta_present_flag is not necessary to be present when the value of num_slices_in_pic_minus1 is equal to 0

Aspect 2: propose to have single_tile_per_slice_flag. Same idea as JVET-Q0204. Slight difference in text

Aspect 3: propose to add inference value for single_slice_per_subpic_flag when not present to be equal to 0. This is asserted to be needed since the flag is used in the conditioning of syntax elements in PPS and also in the derivation of CTB raster scanning, tile scanning, and subpicture scanning processes.
Comment:The flag is needed only when slices are rectangular and when rect_slice_flag is equal to 1. Thus, it seems that when single_slice_per_subpic_flag is not present, it is not used anyway.

JVET-Q0228 AHG12: Signalling of rectangular slices [B.-K. Lee (Xris), D. Jun (Kyungnam University)]

Aspect 1: propose to replace tile_idx_delta[ i ] with tile_idx_delta_minus1[ i] and tile_idx_delta_sign[ i ]. tile_idx_delta_sign[ i ] is not present for the first slice.

Aspect 2: Same idea as JVET-Q0203. Some differences in text implementation.

JVET-Q0230 AHG12: Signalling of multiple slices in a tile [B.-K. Lee (Xris), D Jun (Kyungnam University)]

This contribution proposes 2 new flags for signalling of rectangular slices. It is asserted to save bits for signalling.

- multiple_slice_in_tile_present_flag indicates whether at least one tile is composed of more than slice or not

- multiple_slice_in_tile_flag[i] indicates whether a tile is composed of more than one slice or not.

Example of use-cases that may benefit from this two flags in term of bit-saving are given from 3060 video cases. It shows the savings are between 16% ~ 18% for the given examples.

Comment:

Is the proposed two flags mostly for 3600 video? if yes, for the given example actually the flag single_slice_per_subpic_flag would be used.

JVET-Q0244 [AHG12]: Misc improvements to tile and rectangular slice signalling [S. Paluri, Hendry (LGE)]

Aspect 1: Signal a flag to specify whether the explicitly signalled tile row is the same as the explicitly signalled tile column. If true, tile row information can be derived from tile column information.

Aspect 2: Signal a flag for each rectangular slice in the picture to indicate if the slice height in tiles is the same as the slice width in tiles. If this flag is false, then the slice height is explicitly signalled.

Aspect 3: Signal rectangular slice width in tile only when number of tile column is greater than 1; likewise, signal rectangular slice height only when the number of tile row is greater than 1.

Aspect 4: Signal the number of rectangular slices in a tile only when the number of CTU rows within the tile is greater than 1.

Aspect 5: It is asserted that the second condition for the presence of tile_idx_delta[ i ] (i.e., i < num_slices_in_pic_minus1) is not really needed as it is always true anyway. Therefore, it is proposed to remove it.

Aspect 6: It is asserted that the derivation of CtbToTileColBd[ ] and CtbToTileRowBd[ ] have out of bound / out of range problems. Detail fixes to the asserted problems are given

JVET-Q0289 AHG9/AHG12: Comments on miscellaneous HLS text [L. Chen, C.-W. Hsu, C.-C. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

Aspect 1: signal single_tile_per_slice_flag. Similar to JVET-Q0203 and JVET-Q0218 aspect #2. This contribution also suggest that loop_filter_across_tiles_enabled_flag is not present when the value of single_tile_per_slice_flag is equal to 1.

Aspect 3: editorial suggestions for semantics of tile_idx_delta[ i ] and num_tiles_in_slice_minus1

This contribution contains one aspect regarding mixed NAL unit types.

JVET-Q0332 AhG12: On the numbers of slices and subpictures [Y.-J. Chang, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

This contribution proposes to replace the signaling of the number of slices in picture with the signaling of the delta between the number of subpictures in picture and the number of slices in picture. Instead of num_slices_in_pic_minus1, signal num_slices_in_pic_minus_num_subpics

Comment:

This would cause parsing dependency from PPS to SPS.

JVET-Q0359 AHG12: Constraints on tile signaling [X. Ma, H. Yang (Huawei)]

Aspect 1: on tile_column_width_minus1[ i ], tile_row_height_minus1[ i ], suggest the following:

- Constrain that the width or the height of each tile shall not larger than the width of the picture and the height of the picture in unit of CTB, respectively.

- Constrain that that the sum of the all tile width and the sum of all tile height should less than or equal to the width or height of the picture in unit of CTB, respectively

Comment: It seems that the above constraints are not necessary since it would never happen. It is already prevented by the way syntax, semantic, and also derivations related to num_exp_tile_columns_minus1 and num_exp_tile_rows_minus1

Aspect 2: Add the following: For i in the range of 0 to num_slices_in_pic_minus1-1, inclusive , the sum of tile_idx_delta[ i ] shall less than NumTilesInPic.

JVET-Q0373 AHG12: On signalling of slice [J. H. Do, Y.-U. Yoon, D. H. Park, J.-G. Kim (KAU)]

Aspect 1: Propose to infer the syntax element of slice_height_in_ctu_minus1 as value of 0 when the corresponding tile’s height is the same as the number of slice in the tile in the unit of CTU.

Aspect 2: Proposed to infer the syntax element of num_slices_in_pic_minus1 as value of 0 when the corresponding tile’s height is equal to 1 in the unit of CTU. This is similar to JVET-Q0244 aspect #4.

Aspect 3: Proposed to replace the syntax elements of num_slices_in_tile_minus1 as num_exp_slices_in_pic_minus1 which represents the number of last explicitly signaled slice height. This is similar to JVET-Q0203 and JVET-Q0228 aspect 2.

Comment: If we do aspect 3, aspect 1 is not needed.

JVET-Q0377 AHG12: On "tile-fraction" slices and signaling of slices per subpicture [N. Ouedraogo, G. Laroche, P. Onno (Canon)]

Aspect 1: Propose to remove signalling of num_slice_in_pic_minus1 and introduce num_slice_in_subpic_minus1[ i ].

Comment: this seems to introduce parsing dependency to SPS. pps_num_subpic_minus1 is not defined.

Aspect 2: Propose to remove signalling of slices within a tile. It is asserted initial concept of “tile-fraction” slices targets bitstream extraction and merging operations for omnidirectional content for which subpictures where adopted in VVC.

JVET-Q0480 AHG9: A More Robust Syntax for Raster Rectangular Slices [W. Wan, T. Hellman, B. Heng (Broadcom)]

This contribution proposed that when in raster rectangular slice mode (tile_idx_delta_present_flag equals 0), only specify the slice_height_in_tiles_minus1 syntax element at the start of each tile row. If it is asserted that this would prevent possibility to signal slice height that is illegal.

JVET-Q0119 AHG12: Cleanups on signalling of subpictures, tiles, and rectangular slices [Y.-K. Wang (Bytedance)]

Item 6 of this contribution belongs to this category.

It is asserted that when rect_slice_flag is equal to 1, the length of slice_address is specified to be Ceil( Log2( NumSlicesInSubpic[ SubPicIdx ] ) ) bits, which would be 0 bits when NumSlicesInSubpic[ SubPicIdx ] is equal to 1. This is problematic, as the length of any present syntax elements cannot be 0 bits.

It is therefore proposed that, when rect_slice_flag is equal to 1, the length of slice_address is specified to be Max( Ceil( Log2( NumSlicesInSubpic[ SubPicIdx ] ) ), 1 ) bits, as opposed to be Ceil( Log2( NumSlicesInSubpic[ SubPicIdx ] ) ) bits.

JVET-Q0201 [AHG12]: On slice data signalling [Hendry, S. Paluri (LGE)]

It is asserted the current slice data signalling has the following issues:

- The bin indicating the end of slice is checked for every CTU, which is considered unnecessary.

- The presence of the bins specifying the end of tile and end of subset are not conditioned correctly.

- For the SAO signalling, information regarding the position of the CTU with respect to its picture partition is not correctly derived.

The proposed fixes:

- End of the slice bit (i.e., end_of_slice_one_bit) is signalled at the end of the loop of the CTU loop.

- Define NextCtbAddrInRs (i.e., the index of the next CTU from the current CTU in decoder order in picture raster scan order). When the current CTU is the last CTU in the slice, the NextCtbAddrInRs is set to be the same as CtbAddrInRs

- The bin indicating the end of tile (i.e., end_of_tile_one_bit) followed by a byte_alignment( ) is signalled when at least one of the following conditions is true:

  - The tile column border of the current CTU is not the same as the tile column border of the next CTU.

  - The tile row border of the current CTU is not the same as the tile row border of the next CTU.

- The bin indicating the end of subset (i.e., end_of_subset_one_bit) followed by a byte_alignment( ) is signalled when both of the following conditions are true:

  - The CTU row of the current CTU is not the same as the CTU row of the next CTU.

  - WPP is enabled (i.e., entropy_coding_sync_enabled_flag is equal to 1).

- The variable FirstCtbRowInSlice is renamed to FirstCtbRowInPicPartition and the value of FirstCtbRowInPicPartition is set as follows:

  - At the beginning of slice data, the value of FirstCtbRowInPicPartition is set to be equal to 1.

  - Every time the CTU row of the current CTU and the next CTU is different, set the FirstCtbRowInPicPartition to be equal to 0

  - When the end of a tile is detected, set the FirstCtbRowInPicPartition to be equal to 1

JVET-Q0223 AHG9/AHG12: On dependent slice and slice header [L. Chen, C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

This contribution proposes to enable the dependency between slices in decoding order. There are two aspects of the dependency.

- One is the slice dependency, similar to that in HEVC. Some coding tools may be used across the dependent slice boundaries for the coding efficiency, while a design with a smaller slice size of independent or dependent slice may help in applications with low latency and IP packet size matching.

- The other is the slice header only dependency. A shorter version of the slice header (SH) may be used as the result of the SH size reduction.

JVET-Q0224 AHG9/AHG12: On raster scan slice within a picture [L. Chen, C.-C. Chen, C.-W. Hsu, Y.-L. Hsiao, Y.-W. Huang, S.-M. Lei (MediaTek)]

This contribution proposed to allow and support the raster scan slice in units of CTU within a picture. It is asserted that this would enable HEVC-like raster scan slice and the coding efficiency benefits from a larger tile size partition in a picture.

CTU-based raster-scan slice can be present only whether the whole picture is one tile.

The signalling in PPS is as follows: if no_pic_partition_flag is equal to 1, a new flag raster_scan_slice_in_pic_enabled_flag is signal. If this flag is equal to 1, another flag loop_filter_across_slices_enabled_flag is present.

The signalling in slice header is as follows: if raster_scan_slice_in_pic_enabled_flag is equal to 1 or (rect_slice_flag is equal to 0 and there is only 1 tile in pic), signal slices_in_pic_present_flag. If this flag is equal to 1, signal slice_address and num_ctus_in_slice_minus1.

This proposal does not remove any existing rectangular slice or tile-based raster-scan slice.

JVET-Q0225 AHG9/AHG12: On raster scan slice within a tile [L. Chen, C.-C. Chen, C.-W. Hsu, Y.-L. Hsiao, Y.-W. Huang, S.-M. Lei (MediaTek)]

This contribution proposes to extend the slice-in-tile case to support the raster scan slice within a tile, i.e., the slice is specified in units of CTUs in raster scan within a tile and the slice shape may or may not be rectangular.

The proposed signalling is as follows: if rect_slice_flag is equal to 0, Number of tiles is greater than 1, and num_tiles_in_slice_minus1 is equal to 0, signal slices_in_tile_present_flag. If slices_in_tile_present_flag is equal to 1, signal num_ctus_in_slice_minus1.