| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0591-v1 |
| --- | --- |

| Title: | AHG12: A summary of proposals on subpicture layout signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang San Diego, CA, USA | Tel: Email: | yekui.wang@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution intends to provide a summary of all proposals on subpicture layout signalling submitted to this JVET meeting. It is suggested that this summary is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner.

# List of proposals on subpicture layout signalling

This category of proposals is listed in the agenda item 6.20.1.2, including 6 contributions. These contributions and summaryies of the related proposals are listed below.

JVET-Q0202 [AHG12]: On signalling of subpicture and rectangular slice [Hendry, S. Paluri (LGE)]

The subpicture layout signalling part of this contribution belongs to this category.

This contribution proposes to modify the signalling for subpicture layout by simply signalling the bottom-right CTU, and deriving other information needed for a subpicture (i.e., the top-left CTU position, the width, and the height).

The comparison shows that the proposed signalling reduces bit count 60% for subpicture signalling.

JVET-Q0210 [AHG9]: Miscellaneous HLS clean-ups [Hendry, S. Paluri (LGE)]

Items 2 and 5 of this contribution belong to this category.

2. When subpicture signalling is present but there is only one subpicture, omit the signalling of syntax elements subpic_ctu_top_left_x[ 0 ], subpic_ctu_top_left_y[ 0 ], subpic_width_minus1[ 0 ], subpic_height_minus1[ 0 ], subpic_treated_as_pic_flag[ i ], and loop_filter_across_subpic_enabled_flag[ i ].

5. Remove the following constraint: When subpics_present_flag is equal to 0, single_slice_per_subpic_flag shall be equal to 0. And slightly change Eqn. 29 accordingly.

JVET-Q0215 AHG9/AHG12: Comments on signalling subpicture layout [L. Chen, C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

This contribution proposes the following:

1) Infer the size information when there is only one subpicture in the picture. When sps_num_subpics_minus1 is equal to 0, the subpicture size is the picture size.

2) Infer the top-left position for the first subpicture and infer the last subpicture position when there is more than one subpicture in the picture.

JVET-Q0222 AHG12: On Subpicture Signalling [S. Deshpande (Sharp)]

Items 1-2 of this contribution belong to this category.

Proposal 1: Skip signalling the position of top left CTU of 0-th subpicture, infer it instead.

Proposal 2: Skip signalling the size of the last subpicture, infer it instead.

JVET-Q0413 AHG9/AHG12: On subpicture partitioning signaling [J. Chen, R.-L. Liao, Y. Ye, J. Luo (Alibaba)]

Modification 1: skip subpicture partitioning information signaling and subpic_treated_as_pic_flag and loop_filter_across_subpic_enabled_flag signaling when sps_num_subpics_minus1 is equal to 0.

Modification 2: skip the width and height signaling of the last subpicture.

JVET-Q0475 [AHG9/AHG12] On subpicture signaling [Y. He, A. Hamza (InterDigital)]

Items 1 and 2 of this contribution belong to this category.

1. Remove subpicture layout and ID mapping from SPS.

2. Indicate subpicture boundaries in PPS using slices and conditioned by rect_slice_flag.

# List of design questions

- Modify the signalling for subpicture layout by simply signalling the bottom-right CTU, and deriving other information needed for a subpicture (i.e., the top-left CTU position, the width, and the height). The reported bit saving is 60% for subpicture signalling. (JVET-Q0202)

- When subpicture signalling is present but there is only one subpicture, omit the signalling of subpicture layout and the syntax elements subpic_treated_as_pic_flag[ 0 ], and loop_filter_across_subpic_enabled_flag[ 0 ]. (JVET-Q0210, JVET-Q0215, JVET-Q0413)

- Remove the following constraint: When subpics_present_flag is equal to 0, single_slice_per_subpic_flag shall be equal to 0. (JVET-Q0210)

- Skip the signalling and infer the top-left position for the first subpicture. (JVET-Q0215, JVET-Q0222)

- Skip the signalling and infer the the last subpicture position. (JVET-Q0215, JVET-Q0222, JVET-Q0413)

- Remove subpicure signalling from SPS, and signal subpicture layout in PPS based on the laytout of rectangular slices instead. (JVET-P0475)