| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0595-v1 |
| --- | --- |

| Title: | AHG9: A summary of proposals on combination of RPR and reference wraparound |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang San Diego, CA, USA | Tel: Email: | yekui.wang@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution intends to provide a summary of all proposals on combination of RPR and reference wraparound submitted to this JVET meeting. It is suggested that this summary is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner.

# List of proposals on combination of RPR and reference wraparound

This category of proposals is listed in the agenda item 6.19.1.2, including 7 contributions. These contributions and summaryies of the related proposals are listed below.

JVET-Q0134 AHG8: Disabling reference wraparound for reference picture resampling [B. Heng, P. Chen, T. Hellman, W. Wan, M. Zhou (Broadcom)]

This contribution contends that reference wraparound should be disabled when RPR is enabled, in that the reference picture wraparound does not work when a current picture and its reference have different resolutions, by conditioning sps_ref_wraparound_enabled_flag on "if( !ref_pic_resampling_enabled_flag )", and infer the value to be equal to 0 when not present.

JVET-Q0184 AHG9: On signalling of wrap-around motion compensation [C.-Y. Chiu, C.-C. Chen, C.-W. Hsu, L. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

Proposal: Allow wraparound MC only when RPR is disabled and no ILRP is used.

Justification: Asserted that there are problems for wrap-around MC when the resolutions of reference picture and current picture are different.

JVET-Q0238 AHG8/AHG9: On reference picture wraparound [M. M. Hannuksela (Nokia)]

1. Move the reference wraparound signalling from SPS to PPS. Consequently, enable both reference wraparound and RPR for the same reference picture in the same layer as the current picture.

2. Keep the capability of VVC Draft 7 to apply the reference wrapround feature for spatial scalability.

JVET-Q0316 AHG9: On signaling of the wraparound offset [K. Zhang, L. Zhang, Y.-K. Wang, H. Liu, J. Xu, Z. Deng (Bytedance)]

Signal the wraparound offset in PPS instead of in SPS.

JVET-Q0287 AHG9: On wrap-around motion compensation [B. Choi, S. Wenger, S. Liu (Tencent)]

This contribution proposes two methods to address the issue that the wrap around motion compensation cannot correctly work when the reference picture width is different than the current picture width.

Option 1: Disable the combination of RPR and wraparound motion compensation by syntax, and disable wraparound motion compensation for pictures of layers using ILP by semantics.

Option 2: Disable the wrap around processing during the interpolation process for motion compensation, when the reference picture width is different than the current picture width.

JVET-Q0335 AhG9: On the wraparound offsets [Y.-J. Chang, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

Items 1 and 2 of this contribution belong to this category.

The proposed modifications on top of the current VVC text include:

- Because the reference picture resampling (RPR) design would renew the picture width and height in PPS, it is proposed that the wraparound offsets shall be signaled in PPS instead of SPS to align the design of RPR.

  - The value of ref_wraparound_enabled_flag shall be the same for all PPSs that are referred to by coded pictures in a CLVS. This constraint aligns the current VVC design which keeps the wraparound enabled flag unchanged in CLVS.

- Motion compensation is fixed by replacing the wraparound offset of the current picture with the wraparound offset of the reference picture.

JVET-Q0416 AHG8/AHG9: On horizontal wrap-around motion compensation [J. Chen, Y. Ye, R.-L Liao, J. Luo (Alibaba)]

Item 2 of this contribution belongs to this category.

2. The second aspect of this contribution is to solve the second problem and three methods are proposed:

Method 1 proposes to remove constraint for sps_ref_wraparound_enabled_flag and sps_ref_wraparound_offset_minus1, which is dependent on picture width, but only enable wrap-around MC for the pictures whose width is large enough.

Method 2 proposes to remove constraint for sps_ref_wraparound_enabled_flag and sps_ref_wraparound_offset_minus1, which is dependent on picture width, but impose constraint for pic_width_in_luma_samples.

Method 3 proposes to exclusively use wrap-around MC and picture resolution change in the presence of each other. When wrap-around MC is enabled at the sequence level, all the pictures within the sequence shall have the same width.

# List of design questions

- Allow the combined use of RPR and reference wraparound?

  - No. (JVET-Q0134, JVET-Q0184, JVET-Q0287, JVET-Q0416)

    - Disallow ref_pic_resampling_enabled_flag and sps_ref_wraparound_enabled_flag to be both equal to 1.

      - By syntax (JVET-Q0134, JVET-Q0184, JVET-Q0287)

        - Also disallow sps_ref_wraparound_enabled_flag equal to 1 when inter_layer_ref_pics_present_flag is equal to 1, by syntax (JVET-Q0184)

        - Also disable wraparound motion compensation for pictures of layers using ILP by semantics. (JVET-Q0287)

      - By constraint. (JVET-Q0416)

      - By decoding process change: disable the wrap around processing during the interpolation process for motion compensation, when the reference picture width is different than the current picture width. (JVET-Q0287)

  - Yes. (JVET-Q0238, JVET-Q0316, JVET-Q0335, JVET-Q0416)

    - By moving the reference wraparound signalling from SPS to PPS. This would also keep the capability of VVC Draft 7 to apply the reference wrapround feature for spatial scalability. (JVET-Q0238, JVET-Q0316)

      - The value of ref_wraparound_enabled_flag shall be the same for all PPSs that are referred to by coded pictures in a CLVS. (JVET-Q0335)

      - Motion compensation is fixed by replacing the wraparound offset of the current picture with the wraparound offset of the reference picture. (JVET-Q0335)

    - By removing the constraint for sps_ref_wraparound_enabled_flag and sps_ref_wraparound_offset_minus1, which is dependent on picture width, but only enable wrap-around MC for the pictures whose width is large enough. (JVET-Q0416)

    - By removing remove the constraint for sps_ref_wraparound_enabled_flag and sps_ref_wraparound_offset_minus1, which is dependent on picture width, but impose constraint for pic_width_in_luma_samples. (JVET-Q0416)