| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0603-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-Q0323 (Non-CE3: Encoder optimization for chroma BDPCM) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Man-Shu Chiang No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 man-shu.chiang@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

The document reports the results of crosschecking JVET-Q0323 proposed by ITRI International. JVET-Q0323 proposes two methods of encoder optimization for chroma BDPCM. Both methods were implemented on top of CE3-2.2 and CE3-2.4a source codes. The verification tasks under AI configuration were done, and the BD-rate results match those in JVET-Q0323.

Verification

ITRI International provided the source code of their proposed methods described in JVET-Q0323 [1]. Four verifications were carried out:

- Test 1: method 1 on top of the CE3-2.2 source code

- Test 2: method 2 on top of the CE3-2.2 source code

- Test 3: method 1 on top of the CE3-2.4a source code

- Test 4: method 2 on top of the CE3-2.4a source code

The tests were conducted under CE3 test condition [2]. Table 1 and Table 2 show the results of the proposed method 1 and method 2 using VTM7.0 as the anchor and enabling the proposed methods based on CE3-2.2, respectively. Table 3 and Table 4 show the results of the proposed method 1 and method 2 using VTM7.0 as the anchor and enabling the proposed methods based on CE3-2.4a, respectively.

The BD-rates of the four tests from our experimental results match those in JVET-Q0323.

Table 1.  Proposed method 1 in JVET-Q0323 based on CE3-2.2 under CE3 test condition

|  |  | All Intra |  |
| --- | --- | --- | --- |
|  | ratio |  |  |
|  | VTM7 | Test | bit-rate savings |
| Class A1 | 2.2 | 2.3 | -0.76% |
| Class A2 | 1.6 | 1.6 | -1.27% |
| Class B | 2.2 | 2.2 | -2.27% |
| Class C | 1.9 | 2.0 | -2.08% |
| Class D | 1.9 | 2.0 | -4.15% |
| Class E | 2.8 | 2.9 | -2.73% |
| Class F | 5.3 | 5.6 | -4.58% |
| TGM | 11.8 | 12.1 | -2.08% |
| Overall | 2.1 | 2.2 | -1.89% |
| Enc Time[%] |  | 103% |  |
| Dec Time[%] |  | 88% |  |

Table 2.  Proposed method 2 in JVET-Q0323 based on CE3-2.2 under CE3 test condition

|  |  | All Intra |  |
| --- | --- | --- | --- |
|  | ratio |  |  |
|  | VTM7 | Test | bit-rate savings |
| Class A1 | 2.2 | 2.3 | -1.13% |
| Class A2 | 1.6 | 1.6 | -1.31% |
| Class B | 2.2 | 2.2 | -2.43% |
| Class C | 1.9 | 2.0 | -2.26% |
| Class D | 1.9 | 2.0 | -4.29% |
| Class E | 2.8 | 2.9 | -4.79% |
| Class F | 5.3 | 5.7 | -4.72% |
| TGM | 11.8 | 12.2 | -2.77% |
| Overall | 2.1 | 2.2 | -2.38% |
| Enc Time[%] |  | 106% |  |
| Dec Time[%] |  | 86% |  |

Table 3.  Proposed method 1 in JVET-Q0323 based on CE3-2.4a under CE3 test condition

|  |  | All Intra |  |
| --- | --- | --- | --- |
|  | ratio |  |  |
|  | VTM7 | Test | bit-rate savings |
| Class A1 | 2.2 | 2.4 | -5.98% |
| Class A2 | 1.6 | 1.8 | -10.08% |
| Class B | 2.2 | 2.3 | -5.87% |
| Class C | 1.9 | 2.1 | -6.57% |
| Class D | 1.9 | 2.1 | -8.34% |
| Class E | 2.8 | 3.0 | -6.97% |
| Class F | 5.3 | 5.7 | -6.66% |
| TGM | 11.8 | 12.0 | -1.76% |
| Overall | 2.1 | 2.3 | -6.93% |
| Enc Time[%] |  | 98% |  |
| Dec Time[%] |  | 90% |  |

Table 4.  Proposed method 2 in JVET-Q0323 based on CE3-2.4a under CE3 test condition

|  |  | All Intra |  |
| --- | --- | --- | --- |
|  | ratio |  |  |
|  | VTM7 | Test | bit-rate savings |
| Class A1 | 2.2 | 2.4 | -6.23% |
| Class A2 | 1.6 | 1.8 | -10.23% |
| Class B | 2.2 | 2.3 | -5.95% |
| Class C | 1.9 | 2.1 | -6.70% |
| Class D | 1.9 | 2.1 | -8.42% |
| Class E | 2.8 | 3.0 | -8.78% |
| Class F | 5.3 | 5.7 | -6.79% |
| TGM | 11.8 | 12.1 | -2.34% |
| Overall | 2.1 | 2.3 | -7.35% |
| Enc Time[%] |  | 101% |  |
| Dec Time[%] |  | 87% |  |

References

[1] C.-C. Kuo, S.-P. Wang, C.-C. Lin, C.-L. Lin, “Non-CE3: Encoder optimization for chroma BDPCM,” Document of Joint Video Experts Team, JVET-Q0323.

[2] T. Nguyen, T. C. Ma, and A. Nalci, “Description of Core Experiment 3 (CE3): Lossless Coding,” Document of Joint Video Experts Team, JVET-P2023.

Patent rights declaration(s)

MediaTek Inc. may not have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).