| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0630 |
| --- | --- |

| Title: | AHG9: Parsing dependency for subpicture level information SEI (Ticket 775) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karsten Suehring Robert Skupin Yago Sanchez Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes to resolve a parsing dependency between SPS and subpicture level information SEI. The problem was initially reported in the bug tracker as issue #775.

# Introduction

Currently the subpicture level information SEI seems to be the only SEI message for which parsing depends on the active SPS: the syntax element sps_num_subpics_minus1 is used as a loop maximum:

| subpic_level_info( payloadSize ) { | Descriptor |
| --- | --- |
| sli_seq_parameter_set_id | u(4) |
| num_ref_levels_minus1 | u(3) |
| explicit_fraction_present_flag | u(1) |
| for( i = 0; i <= num_ref_levels_minus1; i++ ) { |  |
| ref_level_idc[ i ] | u(8) |
| if( explicit_fraction_present_flag ) |  |
| for( j = 0; j <= sps_num_subpics_minus1; j++ ) |  |
| ref_level_fraction_minus1[ i ][ j ] | u(8) |
| } |  |
| } |  |

As stated in the related bug tracker issue, we think, that this was unintentional and happened while editing the document during the last meeting.

# Proposed solution

It is proposed to repeat the syntax element in the SEI message instead of referring to the SPS. It will be semantically constrained to use the same value as in the active SPS.

| subpic_level_info( payloadSize ) { | Descriptor |
| --- | --- |
| sli_seq_parameter_set_id | u(4) |
| num_ref_levels_minus1 | u(3) |
| explicit_fraction_present_flag | u(1) |
| if( explicit_fraction_present_flag ) |  |
| sli_num_subpics_minus1 | u(8) |
| for( i = 0; i <= num_ref_levels_minus1; i++ ) { |  |
| ref_level_idc[ i ] | u(8) |
| if( explicit_fraction_present_flag ) |  |
| for( j = 0; j <= spssli_num_subpics_minus1; j++ ) |  |
| ref_level_fraction_minus1[ i ][ j ] | u(8) |
| } |  |
| } |  |

sli_num_subpics_minus1 plus 1 specifies the number of subpictures. When present, the value of sli_num_subpics_minus1 shall be equal to the value of sps_num_subpics_minus1 in the SPS referred to by the associated picture.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).