| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0645-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-Q0506: Interaction between ACT and cross-component coding tools |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Weijia Zhu 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-360-525-0535 weijia.zhu@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This document reports crosscheck results of JVET-Q0506 (Interaction between ACT and cross-component coding tools). The results match the ones provided by the proponent.

# Introduction

In JVET-Q0506, it is proposed to disable JCCR mode at CU-level when ACT mode is enabled on a CU.

# Simulation Results

The simulations were performed by using the code provided by the proponent. Two methods are tested. The first one is to disable JCCR mode at CU-level based on ACT flag. The second one is to disable JCCR mode at SPS-level. Simulation results are shown below. Please note that the complexity is not reliable due to the heterogeneous clusters.

Table 1. Coding performance of disabling JCCR mode at CU-level compared to VTM7.1 under AI condition when dual tree is off

|  | G/Y | B/U | R/V |
| --- | --- | --- | --- |
| RGB, text & graphics with motion, 1080p & 720p | 0.24% | 0.21% | 0.15% |
| RGB, mixed content, 1440p & 1080p | 0.04% | 0.11% | 0.04% |
| RGB, Animation, 720p | 0.07% | 0.18% | 0.05% |
| RGB, camera captured, 1080p | 0.11% | 0.11% | 0.07% |
| YUV, text & graphics with motion, 1080p & 720p | 0.01% | 0.00% | 0.00% |
| YUV, mixed content, 1440p & 1080p | 0.01% | 0.03% | 0.01% |
| YUV, Animation, 720p | 0.00% | 0.01% | 0.00% |
| YUV, camera captured, 1080p | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 97% |  |  |

Table 2. Coding performance of disabling JCCR mode at CU-level compared to VTM7.1 under RA condition when dual tree is off

|  | G/Y | B/U | R/V |
| --- | --- | --- | --- |
| RGB, text & graphics with motion, 1080p & 720p | 0.36% | 0.27% | 0.14% |
| RGB, mixed content, 1440p & 1080p | 0.04% | 0.17% | 0.04% |
| RGB, Animation, 720p | -0.02% | 0.12% | -0.03% |
| RGB, camera captured, 1080p | 0.36% | 0.12% | -0.01% |
| YUV, text & graphics with motion, 1080p & 720p | 0.02% | 0.01% | 0.03% |
| YUV, mixed content, 1440p & 1080p | 0.07% | 0.04% | 0.10% |
| YUV, Animation, 720p | 0.03% | 0.04% | -0.04% |
| YUV, camera captured, 1080p | 0.02% | 0.03% | 0.00% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 100% |  |  |

Table 3. Coding performance of disabling JCCR mode at SPS-level compared to VTM7.1 under AI condition when dual tree is off

|  | G/Y | B/U | R/V |
| --- | --- | --- | --- |
| RGB, text & graphics with motion, 1080p & 720p | 0.6% | 0.5% | 0.4% |
| RGB, mixed content, 1440p & 1080p | 0.4% | 0.0% | -0.1% |
| RGB, Animation, 720p | 0.8% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 2.5% | 0.4% | -1.5% |
| YUV, text & graphics with motion, 1080p & 720p | 0.7% | 0.0% | 0.3% |
| YUV, mixed content, 1440p & 1080p | 1.3% | 1.2% | 1.1% |
| YUV, Animation, 720p | 2.1% | 7.1% | 4.4% |
| YUV, camera captured, 1080p | 2.8% | 0.7% | -1.2% |
| Enc Time[%] | 95% |  |  |
| Dec Time[%] | 103% |  |  |

Table 4. Coding performance of disabling JCCR mode at SPS-level compared to VTM7.1 under RA condition when dual tree is off

|  | G/Y | B/U | R/V |
| --- | --- | --- | --- |
| RGB, text & graphics with motion, 1080p & 720p | 0.8% | 0.8% | 0.7% |
| RGB, mixed content, 1440p & 1080p | 0.4% | -0.1% | -0.2% |
| RGB, Animation, 720p | 0.0% | -0.2% | -0.1% |
| RGB, camera captured, 1080p | 2.0% | 1.2% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.5% | 0.1% | 0.3% |
| YUV, mixed content, 1440p & 1080p | 1.1% | 0.7% | 0.9% |
| YUV, Animation, 720p | 2.1% | 6.8% | 5.5% |
| YUV, camera captured, 1080p | 0.9% | 0.4% | -0.4% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 104% |  |  |