| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0646 |
| --- | --- |

| Title: | Cross-check of JVET-Q0144 (Non-CE3: Contexts in Chroma transform skip residual coding) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tsung-Chuan Ma | Tel: Email: | tsung-chuanma@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This is a crosscheck for JVET-Q0144. It is proposed to introduce individual contexts for luma and chroma in all syntaxes of transform skip residual (TSRC) to improve coding gains of Lossless coding. Proposed aspect is confirmed by cross-checkers, source code is identical to description. Simulation results partially match proponent’s results.

# Introduction

It is proposed to introduce chroma contexts in TSRC to improve coding gains in Lossless mode. Derivation process for ctxInc for chroma TS contexts is completely the same as those for luma TS contexts.

Table 2: Number of contexts in RRC and proposed TSRC

|  | RRC |  | TSRC |  |
| --- | --- | --- | --- | --- |
|  | Luma | Chroma | Luma | Chroma |
| coded_sub_block_flag | 2 | 2 | 3 | 3 |
| sig_coeff_flag | 36 | 24 | 3 | 3 |
| coeff_sign_flag |  |  | 6 | 6 |
| abs_level_gt1_flag | 21 | 11 | 4 | 4 |
| parity_flag | 21 | 11 | 1 | 1 |
| abs_level_gt3_flag | 21 | 11 | 1 | 1 |
| abs_level_gt5_flag |  |  | 1 | 1 |
| abs_level_gt7_flag |  |  | 1 | 1 |
| abs_level_gt9_flag |  |  | 1 | 1 |

Proposed aspect is confirmed by cross-checkers. Source code does match description and spec is correct.

# Results

Results are proposed over the VTM7.0 anchor. Runtimes are not reliable.

Table 1. CTC result

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.04% | 0.03% | 99% | 98% |
| Class A2 | -0.01% | 0.01% | -0.04% | 98% | 99% |
| Class B | 0.00% | -0.03% | -0.02% | 99% | 97% |
| Class C | 0.01% | 0.00% | 0.05% | 96% | 94% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Overall | 0.00% | -0.01% | 0.00% | 98% | 97% |
| Class D | 0.00% | 0.07% | 0.03% | 103% | 102% |
| Class F | 0.00% | 0.02% | -0.01% | 97% | 90% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.06% | 0.08% | 98% | 99% |
| Class A2 | -0.03% | 0.03% | 0.06% | 100% | 100% |
| Class B | -0.01% | 0.03% | -0.10% | 100% | 98% |
| Class C | 0.02% | 0.08% | 0.00% | 102% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.02% | 0.00% | 100% | 99% |
| Class D | -0.03% | -0.22% | -0.16% | 102% | 99% |
| Class F | 0.01% | 0.02% | -0.06% | 101% | 97% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.20% | -0.16% | 99% | 98% |
| Class C | 0.00% | -0.21% | -0.04% | 102% | 98% |
| Class E | 0.04% | 0.09% | -0.11% | 97% | 98% |
| Overall | 0.00% | -0.13% | -0.11% | 100% | 98% |
| Class D | -0.03% | -0.15% | -0.51% | 102% | 98% |
| Class F | 0.00% | -0.13% | 0.77% | 101% | 99% |

Table 2. Low Qp result (Need to Updated)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | -0.01% | 0.00% |  | -0.03% |  | 95% |  | 99% |  |
| Class A2 | 0.00% | -0.01% |  | -0.01% |  | 98% |  | 99% |  |
| Class B | -0.01% | 0.01% |  | 0.00% |  | 101% |  | 101% |  |
| Class C | -0.01% | -0.02% |  | -0.02% |  | 102% |  | 108% |  |
| Class E | -0.07% | 0.01% |  | 0.05% |  | 105% |  | 103% |  |
| Overall | -0.02% | 0.00% |  | 0.00% |  | 100% |  | 102% |  |
| Class D | -0.02% | 0.02% |  | -0.02% |  | 104% |  | 103% |  |
| Class F | -0.09% | -0.14% |  | -0.12% |  | 102% |  | 102% |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 | #VALUE! |  | #VALUE! |  | #VALUE! |  | #NUM! |  | #NUM! |
| Class A2 | #VALUE! |  | #VALUE! |  | #VALUE! |  | #NUM! |  | #NUM! |
| Class B | 0.00% |  | -0.02% |  | -0.05% |  | 101% |  | 101% |
| Class C | 0.00% |  | -0.09% |  | -0.11% |  | 104% |  | 101% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | #VALUE! |  | #VALUE! |  | #VALUE! |  | #NUM! |  | #NUM! |
| Class D | 0.01% |  | -0.01% |  | -0.16% |  | 106% |  | 107% |
| Class F | -0.05% |  | -0.07% |  | 0.02% |  | 99% |  | 99% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | 0.01% | -0.03% |  | -0.03% |  | 99% |  | 97% |  |
| Class C | 0.00% | -0.03% |  | -0.05% |  | 102% |  | 100% |  |
| Class E | -0.03% | 0.03% |  | 0.05% |  | 93% |  | 96% |  |
| Overall | -0.01% | -0.02% |  | -0.02% |  | 99% |  | 97% |  |
| Class D | 0.00% | 0.01% |  | -0.06% |  | 101% |  | 104% |  |
| Class F | -0.05% | -0.10% |  | -0.18% |  | 105% |  | 104% |  |

Table 3. Lossless result

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM7.0 | proposal |  | VTM7.0 | proposal |  | VTM7.0 | proposal |  |
| Class A1 | 2.2 | 2.2 | -0.28% | 2.2 | 2.3 | -0.79% |  |  |  |
| Class A2 | 1.6 | 1.6 | -0.05% | 1.7 | 1.7 | -0.24% |  |  |  |
| Class B | 2.2 | 2.2 | -0.19% | 2.3 | 2.4 | -0.59% | 2.3 | 2.4 | -0.60% |
| Class C | 1.9 | 1.9 | -0.22% | 2.4 | 2.5 | -0.91% | 2.4 | 2.4 | -0.96% |
| Class D | 1.9 | 1.9 | -0.23% | 2.8 | 2.8 | -0.85% | 2.7 | 2.8 | -0.92% |
| Class E | 2.8 | 2.8 | -0.41% |  |  |  | 3.1 | 3.2 | -1.44% |
| Class F | 5.3 | 5.3 | -0.17% | 33.7 | 33.9 | -0.43% | 50.7 | 50.9 | -0.45% |
| TGM | 11.8 | 11.9 | -0.36% | 107.1 | 107.6 | -0.55% | 124.9 | 125.4 | -0.52% |
| Overall | 2.1 | 2.1 | -0.23% | 2.2 | 2.2 | -0.65% | 2.6 | 2.6 | -0.93% |
| Enc Time[%] | 101% |  |  | 100% |  |  | 106% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 102% |  |  |

# References

[1] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

[2] Takeshi Tsukuba , Masaru Ikeda, Yoichi Yagasaki, Teruhiko Suzuki “ CE8-2.1: Transform Skip for Chroma with limiting maximum number of context-coded bin in TS residual coding”, JVET-P0058