| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, 6–17 January 2020 | Document: JVET-Q0650 |
| --- | --- |

| Title: | Cross-check of JVET-Q0292 (Cleanup of intra reference sample filter selection) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | Tsung-Chuan Ma | Tel: Email: | tsung-chuanma@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to simplify the intra reference sample filter selection. Two method are tested and cross-checked. The source code matches description. Simulation results all match proponent’s results.

# Introduction

This contribution contains two aspects related to intra reference sample filter selection.

In Method 1, the reference sample filter is selected as the same way as VTM-7.0 except for the angular mode. Intra reference sample filters for the angular mode are selected as follows.

- If diff is greater than size-dependent threshold - refFilterFlag is set to false. - interpolationFlag is set to Gaussian. - Otherwise, - refFilterFlag is set to false. - interpolationFlag is set to DCT-IF.

Since intra reference sample filters are directly determined according to the size-dependent threshold, intra mode information is unnecessary for Method 1.

Method 2 proposes to remove the reference sample filter flag decision process. This means in all intra prediction the reference sample filter process is completely removed in intra coding process.

Two aspects are confirmed by cross-checkers. Source code does match description and spec is correct.

# Results

Results 	are proposed over the VTM7.0 anchor. Runtimes are not reliable.

Table 1. Simulation results of JVET-Q0292 test1

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT | DecT |  |
| Class A1 | 0.00% |  | -0.11% |  | 0.08% | 97% | 95% |  |
| Class A2 | 0.00% |  | -0.01% |  | -0.06% | 100% | 100% |  |
| Class B | 0.00% |  | 0.01% |  | -0.05% | 101% | 102% |  |
| Class C | 0.00% |  | 0.05% |  | 0.03% | 97% | 92% |  |
| Class E | 0.01% |  | -0.05% |  | -0.01% | 99% | 99% |  |
| Overall | 0.00% |  | -0.01% |  | -0.01% | 99% | 98% |  |
| Class D | -0.02% |  | 0.03% |  | 0.03% | 100% | 104% |  |
| Class F | 0.00% |  | 0.05% |  | 0.12% | 99% | 90% |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |
| Class A1 | 0.01% | -0.03% |  | 0.05% |  | 98% |  | 98% |
| Class A2 | 0.00% | 0.09% |  | 0.00% |  | 100% |  | 99% |
| Class B | 0.01% | 0.01% |  | -0.10% |  | 100% |  | 99% |
| Class C | 0.00% | -0.04% |  | -0.01% |  | 100% |  | 96% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.00% | 0.01% |  | -0.03% |  | 100% |  | 98% |
| Class D | 0.01% | -0.27% |  | -0.17% |  | 102% |  | 97% |
| Class F | 0.04% | -0.14% |  | -0.01% |  | 100% |  | 97% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | 0.00% |  | -0.03% |  | -0.10% | 100% | 102% |  |
| Class C | -0.02% |  | -0.15% |  | -0.09% | 100% | 99% |  |
| Class E | -0.12% |  | 0.06% |  | -0.31% | 97% | 95% |  |
| Overall | -0.04% |  | -0.05% |  | -0.15% | 99% | 99% |  |
| Class D | -0.01% |  | 0.24% |  | -0.74% | 98% | 94% |  |
| Class F | -0.01% |  | -0.38% |  | -0.16% | 99% | 98% |  |

Table 1. Simulation results of JVET-Q0292 test2

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 | 0.05% |  | -0.14% |  | 0.09% |  | 103% |  | 99% |
| Class A2 | 0.01% |  | -0.06% |  | -0.11% |  | 101% |  | 97% |
| Class B | 0.04% |  | -0.04% |  | -0.12% |  | 105% |  | 101% |
| Class C | 0.02% |  | 0.11% |  | 0.07% |  | 98% |  | 93% |
| Class E | -0.08% |  | -0.21% |  | -0.08% |  | 103% |  | 99% |
| Overall | 0.01% |  | -0.06% |  | -0.03% |  | 102% |  | 98% |
| Class D | 0.02% |  | -0.01% |  | -0.02% |  | 101% |  | 100% |
| Class F | 0.01% |  | 0.01% |  | 0.11% |  | 99% |  | 94% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.02% | -0.10% |  | -0.03% |  | 100% |  | 99% |  |
| Class A2 | 0.00% | 0.04% |  | 0.05% |  | 102% |  | 100% |  |
| Class B | 0.01% | 0.06% |  | 0.07% |  | 101% |  | 98% |  |
| Class C | 0.03% | 0.01% |  | -0.02% |  | 101% |  | 97% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.02% | 0.01% |  | 0.02% |  | 101% |  | 98% |  |
| Class D | 0.01% | -0.18% |  | 0.15% |  | 101% |  | 99% |  |
| Class F | 0.02% | 0.02% |  | 0.06% |  | 101% |  | 97% |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | 0.03% | -0.32% |  | -0.19% |  | 100% |  | 98% |  |
| Class C | 0.03% | -0.18% |  | -0.19% |  | 100% |  | 99% |  |
| Class E | 0.03% | -0.73% |  | -0.10% |  | 95% |  | 96% |  |
| Overall | 0.03% | -0.37% |  | -0.17% |  | 99% |  | 98% |  |
| Class D | 0.04% | 0.12% |  | -0.11% |  | 97% |  | 94% |  |
| Class F | 0.03% | -0.03% |  | 0.54% |  | 99% |  | 100% |  |