| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0669 |
| --- | --- |

| Title: | Crosscheck of JVET-Q0507 (CE4-related: Quality scalable GEO) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Zhipin Deng Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | +1(858)999-7093 zhipin.deng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results of JVET-Q0507, where a quality scalable GEO is proposed by using a SPS flag to specify whether a reordered 64 modes or the first 16 of the reordered 64 modes are used. The simulation results reportedly match those provided by the proponents.

# Verification

In the JVET-Q0507 proposal [1], three tests are arranged as follows:

- Test 1: based on CE4-1 software, the 64 modes are reordered and only the first 16 modes of GEO mode table are valid

- Test 2: based on CE4-1 software, the 64 modes are reordered and all 64 modes of GEO mode table are valid

- Test 3: based on CE4-1 software, the 64 modes are reordered and only the first 32 modes of GEO mode table are valid

We verified that

- The source code implements match with what was described in the proposal.

- Simulation results match with those provided in the proposal.

CE4 test condition [2] is performed with the provided source code based on CE4 comman base software [3]. The coding results of test 1 to test 3 compared to VTM-7.0 are summarized in Table 1 to Table 3, respectively.

Table 1: Performance of test 1 over VTM-7.0

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.09% | 0.00% | 102% | 101% |
| Class A2 | -0.06% | 0.04% | 0.05% | 102% | 100% |
| Class B | -0.06% | 0.03% | -0.04% | 105% | 102% |
| Class C | -0.21% | -0.34% | -0.28% | 102% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.09% | -0.08% | 103% | 101% |
| Class D | -0.16% | -0.32% | -0.25% | 102% | 103% |
| Class F | -0.22% | -0.21% | -0.19% | 102% | 102% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.16% | -0.28% | -0.17% | 102% | 99% |
| Class C | -0.25% | -0.42% | -0.09% | 99% | 96% |
| Class E | -0.52% | 0.22% | -0.38% | 99% | 101% |
| Overall | -0.28% | -0.20% | -0.20% | 100% | 98% |
| Class D | -0.33% | 0.12% | -0.53% | 98% | 96% |
| Class F | -0.34% | -0.49% | 0.01% | 107% | 101% |

Table 2: Performance of test 2 over VTM-7.0

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.20% | -0.18% | 104% | 101% |
| Class A2 | -0.19% | -0.14% | -0.02% | 103% | 101% |
| Class B | -0.14% | -0.11% | -0.09% | 106% | 101% |
| Class C | -0.61% | -0.97% | -0.86% | 104% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.28% | -0.36% | -0.30% | 104% | 100% |
| Class D | -0.41% | -0.64% | -0.52% | 105% | 103% |
| Class F | -0.33% | -0.34% | -0.35% | 105% | 102% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.32% | -0.44% | -0.26% | 107% | 102% |
| Class C | -0.81% | -1.15% | -0.96% | 103% | 96% |
| Class E | -1.18% | -0.21% | -0.77% | 100% | 99% |
| Overall | -0.70% | -0.62% | -0.62% | 104% | 100% |
| Class D | -0.68% | -0.56% | -0.76% | 102% | 96% |
| Class F | -0.53% | -0.75% | 0.05% | 101% | 96% |

Table 3: Performance of test 3 over VTM-7.0

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.19% | -0.07% | 103% | 101% |
| Class A2 | -0.13% | -0.12% | 0.02% | 103% | 101% |
| Class B | -0.12% | -0.08% | -0.11% | 101% | 98% |
| Class C | -0.41% | -0.60% | -0.53% | 104% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.19% | -0.25% | -0.19% | 103% | 101% |
| Class D | -0.31% | -0.46% | -0.52% | 103% | 98% |
| Class F | -0.29% | -0.32% | -0.27% | 102% | 98% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.29% | -0.34% | -0.17% | 103% | 100% |
| Class C | -0.59% | -0.80% | -0.48% | 99% | 94% |
| Class E | -0.84% | -0.24% | -0.49% | 100% | 100% |
| Overall | -0.53% | -0.47% | -0.35% | 101% | 98% |
| Class D | -0.54% | -0.46% | -0.64% | 100% | 96% |
| Class F | -0.41% | -1.17% | -0.22% | 103% | 99% |

The BD rate differences match with the results from the proposal [1]. The run times are basically in accordance with that from the proposal. Note that timing results are not well comparable due to differences in the simulation environment.

# References

J. Li, C. S., Lim, “CE4-related: Quality scalable GEO”, JVET-Q0507, January 2020, Brussels, BE.

C.-C. Chen, R.-L. Liao, X. Xiu, H. Yang, “Description of Core Experiment 4 (CE4): Inter prediction with geometric partitioning,” Joint Video Expert Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-P2024, 16th Meeting: Geneva, CH, 1–11 October 2019.

H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang, M. Blaser, J. Sauer, “Simplified GEO without multiplication and minimum blending mask storage (harmonization of JVET-P0108, JVET-P0264 and JVET-P0304),” Joint Video Expert Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-P0884, 16th Meeting: Geneva, CH, 1–11 October 2019.