| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0674 |
| --- | --- |

| Title: | Crosscheck of JVET-Q0468 (Constraint on minimum CU size) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Kai Zhang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1(858)-380-8522 zhangkai.video@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results of JVET-Q0468. The simulation results reportedly match those provided by the proponent.

# Verification

In JVET- Q0468, it is proposed to allow minimum CU size setting in VTM software. It is further proposed to set the range of minimum CU size value in the range of {4, 8, 16} in luma samples, according to the investigation of the performance impact of applying minimum coding unit sizes on VTM-7.0.

Three tests are performed with the provided source code based on VTM-7.0 with mimimum CU size equal to 8, 16, and 32, summarized in Table 1~ Table 3.

Table 1: Results of test 1 on VTM-7.0

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.57% | -0.22% | 0.00% | 63% | 101% | 0.51% | 0.22% | 0.65% | 85% | 108% |
| Class A2 | 1.14% | 0.61% | 0.55% | 56% | 98% | 1.47% | 1.89% | 1.92% | 81% | 107% |
| Class B | 2.05% | 1.35% | 1.34% | 52% | 98% | 1.82% | 2.18% | 2.35% | 80% | 107% |
| Class C | 5.03% | 2.94% | 2.88% | 49% | 87% | 4.72% | 4.27% | 4.81% | 77% | 105% |
| Class E | 4.14% | 2.28% | 2.41% | 53% | 97% |  |  |  |  |  |
| Overall | 2.66% | 1.47% | 1.51% | 54% | 96% | 2.26% | 2.29% | 2.58% | 80% | 107% |
| Class D | 5.67% | 3.52% | 3.90% | 47% | 92% | 5.64% | 6.33% | 6.68% | 73% | 105% |
| Class F |  |  |  |  |  |  |  |  |  |  |

Table 2: Results of test 2 on VTM-7.0

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 3.40% | 3.23% | 3.48% | 26% | 95% | 3.76% | 4.47% | 5.88% | 60% | 108% |
| Class A2 | 5.50% | 9.84% | 9.65% | 18% | 90% | 8.92% | 14.21% | 13.95% | 58% | 102% |
| Class B | 9.34% | 10.65% | 10.98% | 17% | 87% | 10.94% | 14.58% | 15.05% | 59% | 109% |
| Class C | 16.76% | 17.49% | 18.73% | 14% | 70% | 21.98% | 24.28% | 26.37% | 49% | 103% |
| Class E | 15.33% | 12.46% | 12.61% | 19% | 83% |  |  |  |  |  |
| Overall | 10.36% | 11.10% | 11.50% | 18% | 84% | 12.04% | 15.07% | 16.01% | 56% | 106% |
| Class D | 16.82% | 18.59% | 19.59% | 14% | 85% | 23.93% | 31.58% | 32.70% | 50% | 101% |
| Class F |  |  |  |  |  |  |  |  |  |  |

Table 3: Results of test 3 on VTM-7.0

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 11.72% | 13.57% | 14.71% | 12% | 89% | 15.76% | 19.86% | 24.34% | 33% | 102% |
| Class A2 | 14.47% | 26.31% | 25.43% | 8% | 81% | 27.51% | 40.40% | 37.60% | 36% | 99% |
| Class B | 20.90% | 27.79% | 28.77% | 7% | 77% | 31.18% | 41.58% | 43.23% | 32% | 102% |
| Class C | 32.99% | 39.07% | 42.10% | 6% | 64% | 62.09% | 68.04% | 75.67% | 24% | 92% |
| Class E | 37.47% | 37.02% | 36.90% | 9% | 78% |  |  |  |  |  |
| Overall | 23.75% | 29.22% | 30.19% | 8% | 76% | 35.60% | 44.05% | 46.98% | 30% | 98% |
| Class D | 35.42% | 41.44% | 44.45% | 7% | 76% | 74.15% | 89.37% | 91.37% | 34% | 90% |
| Class F |  |  |  |  |  |  |  |  |  |  |