| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0692 |
| --- | --- |

692

| Title: | Crosscheck of JVET-Q0127: Non-CE4: On merge list generation for geometric partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Gao | Tel: Email: | han.gao@huawei.com |
| Source: | Huawei |  |  |

_____________________________

# Abstract

This contribution reports the crosschecking of JVET-Q0127. One method is proposed in JVET-Q0127. It’s reported that the WD and software are matching with the descripted methods. Simulation results of this report are matched with the results provided by the proponent of JVET-Q0127

# Introduction

In JVET-Q0127, the following method is proposed

One is a spatial MV similarity and another is a temporal MV similarity.

As the spatial MV similarity, when the block is predicted with geometric partitioning, possibility of disconnected region for merge block may increase due to the unequaled regions. To consider such the situation, diversity of spatial marge candidates can reduce the disconnect region in the merge list. To realize it as merge list generation process, we propose to reorder neighboring spatial merge processes.

In detail, when the geometric partitioning is used, B2 is reorder in front of B0 and A0 without any other conditions. It is noted that the current order of candidates for a marge list are as follows; B1, A1, B0, A0, and B2, following by some special candidates like temporal, history, pairwise, and zero. In the working draft 7, spatial candidates are illustrated as follows.

Figure 1 Location of spatial candidates in the working draft 7.

As the temporal MV similarity, when temporal motion vector prediction is enabled in a current bitstream, disconnected region can borrow motion vectors from a co-located block in a temporally different picture. To realize is as a merge list generation process, we propose to reorder the temporal merge order in a merge list generation.

In detail, the order of processes for merge list is modified as follows; B1, A1, B2, temporal, B0, and A0, following by remaining special merge processes like history, pairwise, and zero. It is noted that pruning process is maintained as regular marge list generation.

It’s verified by the crosschecker that both the WD and software are matching the descripted methods. Simulations results for the proposed is matched with the results provided by the proponent of JVET-Q0127 based on the simulation results in the following Table

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.19% | -0.13% | 103% | 100% |
| Class A2 | -0.21% | -0.17% | -0.05% | 103% | 100% |
| Class B | -0.13% | -0.11% | -0.12% | 103% | 100% |
| Class C | -0.61% | -0.93% | -0.86% | 103% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.27% | -0.36% | -0.31% | 103% | 100% |
| Class D | -0.43% | -0.63% | -0.49% | 103% | 100% |
| Class F (optional) | -0.28% | -0.30% | -0.27% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.36% | -0.74% | -0.61% | 103% | 98% |
| Class C | -0.72% | -1.17% | -0.74% | 104% | 99% |
| Class E | -1.13% | -0.20% | -0.72% | 101% | 98% |
| Overall | -0.67% | -0.75% | -0.68% | 103% | 98% |
| Class D | -0.68% | -0.29% | -0.54% | 104% | 99% |
| Class F (optional) | -0.50% | -1.07% | -0.15% | 102% | 99% |

# Patent rights declaration(s)

Huawei does not have any current or pending patent rights relating to the technology described in this contribution.