| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0698-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-Q0293 (Removal of chroma Nx2 blocks in PDPC) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ling Li | Tel: Email: | aurali@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This proposal JVET-Q0293 proposes to remove chroma Nx2 blocks in PDPC. The BD-rate performance of the proposed method generally matches with crosscheck results except some values in random access in 4th digit after decimal.

# Introduction

Since 2x2, 2x4, 4x2, and 2xN chroma blocks were removed in chroma intra prediction in VTM-7.0, the position-dependent prediction sample filtering process (PDPC) is applied in all other chroma blocks except for those chroma blocks. While PDPC is applied to luma block which is greater than or equal to 4x4 in luma intra prediction. This means that the PDPC condition is different for Nx2 blocks according to luma and chroma intra prediction. Therefore, this contribution proposes to remove the remaining Nx2 chroma intra blocks for PDPC.

# Simulation results

|  | Over VTM-7 AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.06% | 0.04% | 100% | 101% |
| Class A2 | 0.00% | 0.03% | 0.02% | 101% | 101% |
| Class B | 0.00% | 0.01% | -0.06% | 100% | 102% |
| Class C | 0.00% | 0.03% | -0.02% | 100% | 102% |
| Class E | 0.00% | -0.08% | 0.07% | 100% | 101% |
| Overall | 0.00% | -0.01% | 0.00% | 100% | 102% |
| Class D | 0.00% | 0.00% | 0.02% | 101% | 105% |
| Class F | 0.01% | -0.03% | -0.04% | 100% | 103% |

|  | Over VTM-7 RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.16% | -0.01% | 100% | 99% |
| Class A2 | -0.01% | -0.10% | -0.01% | 100% | 99% |
| Class B | 0.01% | 0.02% | 0.07% | 100% | 101% |
| Class C | 0.04% | -0.19% | -0.01% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.10% | 0.02% | 100% | 100% |
| Class D | -0.02% | -0.25% | -0.17% | 100% | 102% |
| Class F | 0.01% | 0.10% | 0.09% | 100% | 102% |

|  | Over VTM-7 LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.45% | -0.32% | 100% | 99% |
| Class C | 0.01% | -0.41% | -0.35% | 101% | 103% |
| Class E | -0.05% | 0.42% | -0.12% | 100% | 101% |
| Overall | -0.01% | -0.22% | -0.28% | 100% | 101% |
| Class D | -0.03% | -0.12% | -0.48% | 100% | 102% |
| Class F | -0.12% | -0.28% | -0.21% | 101% | 102% |

# Conclusion

The BD-rate performance of the proposed three methods generally matches with crosscheck results except some values in random access in 4th digit after decimal.

# Patent rights declaration(s)

Tencent does not have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).