| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0706 |
| --- | --- |

| Title: | Crosscheck of JVET-Q0056 (CE3-related: Level prediction for transform skip samples) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yusuke Kato Kiyofumi Abe Tadamasa Toma | Tel: Email: | +81-80-3083-1171 kato.yusuke003@jp.panasonic.com abe.kiyo@jp.panasonic.com toma.tadamasa@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

- Abstract

This contribution is a cross-check report of JVET-Q0056. It is confirmed that the cross-check results (partial for RA of LowQP) are identical with the proponent’s results. The cross-checker carefully checked the provided source code and confirmed that the implementation matches the input document.

Introduction

Brief technical description of option1 of JVET-Q0056

Average based sample value prediction for transform skip coefficients which are bypass coded is proposed. The proposed sample prediction process uses the same reference samples as the existing absLevel prediction process for the non-bypass samples. The proposed bypass sample prediction is enabled when the TU is coded in intra mode and when both the reference samples have the same sign.

Simulation results

Table 1, 2 and 3 show the lossless, CTC and LowQP of proposed method. Run time of LowQP is not accurate.

Table 1: Simulation results of crosscheck of Q0056

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM7 | Q0056 |  | VTM7 | Q0056 |  | VTM7 | Q0056 |  |
| Class A1 | 2.2 | 2.3 | -1.85% | 2.2 | 2.3 | -1.39% |  |  |  |
| Class A2 | 1.6 | 1.6 | -2.51% | 1.7 | 1.7 | -0.43% |  |  |  |
| Class B | 2.2 | 2.3 | -3.62% | 2.3 | 2.4 | -1.37% | 2.3 | 2.4 | -1.25% |
| Class C | 1.9 | 2.0 | -3.09% | 2.4 | 2.5 | -0.85% | 2.4 | 2.4 | -0.68% |
| Class D | 1.9 | 2.0 | -3.79% | 2.8 | 2.8 | -0.75% | 2.7 | 2.8 | -0.48% |
| Class E | 2.8 | 2.9 | -3.88% |  |  |  | 3.1 | 3.1 | -0.49% |
| Class F | 5.3 | 5.5 | -2.68% | 33.7 | 33.9 | -0.68% | 50.7 | 50.9 | -0.52% |
| TGM | 11.8 | 11.9 | -0.04% | 107.1 | 106.9 | 0.08% | 124.9 | 124.7 | 0.10% |
| Overall | 2.1 | 2.2 | -3.07% | 2.2 | 2.2 | -1.05% | 2.6 | 2.6 | -0.87% |
| Enc Time[%] | 100% |  |  | 103% |  |  | 102% |  |  |
| Dec Time[%] | 92% |  |  | 100% |  |  | 100% |  |  |

Table 2: Simulation results of crosscheck of Q0056 in CTC

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | 0.01% | 102% | 101% |
| Class A2 | 0.00% | 0.00% | -0.02% | 102% | 101% |
| Class B | 0.00% | 0.03% | -0.03% | 102% | 101% |
| Class C | 0.01% | 0.04% | 0.08% | 101% | 101% |
| Class E | 0.00% | -0.01% | 0.00% | 101% | 100% |
| Overall | 0.00% | 0.01% | 0.01% | 101% | 101% |
| Class D | -0.01% | 0.04% | -0.03% | 100% | 100% |
| Class F | 0.01% | 0.04% | 0.07% | 101% | 101% |
| TGM | 0.06% | 0.06% | 0.05% | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | -0.01% | 101% | 101% |
| Class A2 | 0.00% | 0.09% | 0.01% | 100% | 100% |
| Class B | 0.02% | -0.03% | 0.08% | 100% | 100% |
| Class C | 0.01% | -0.11% | -0.14% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.03% | -0.01% | 100% | 100% |
| Class D | -0.01% | -0.26% | -0.10% | 100% | 100% |
| Class F | 0.03% | 0.11% | -0.05% | 100% | 100% |
| TGM | 0.05% | -0.02% | 0.04% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.23% | 0.05% | 99% | 99% |
| Class C | 0.02% | -0.10% | -0.20% | 104% | 99% |
| Class E | -0.03% | 1.01% | -0.46% | 100% | 98% |
| Overall | 0.00% | 0.12% | -0.16% | 101% | 98% |
| Class D | -0.06% | 0.45% | 0.03% | 104% | 102% |
| Class F | 0.03% | -0.37% | 0.24% | 100% | 99% |
| TGM | -0.07% | -0.20% | -0.12% | 101% | 100% |

Table 3: Simulation results of crosscheck of Q0056 in LowQP

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0(100Frm,4K,LowQP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.03% | 0.02% | 99% | 100% |
| Class A2 | 0.00% | -0.01% | -0.01% | 100% | 100% |
| Class B | -0.01% | 0.00% | 0.01% | 99% | 99% |
| Class C | -0.01% | -0.01% | -0.01% | 99% | 99% |
| Class E | -0.02% | -0.03% | -0.05% | 100% | 101% |
| Overall | 0.00% | 0.00% | -0.01% | 99% | 99% |
| Class D | 0.00% | -0.03% | -0.03% | 101% | 101% |
| Class F | -0.17% | 0.05% | 0.03% | 100% | 100% |
| TGM | 0.38% | 0.30% | 0.30% | 101% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-7.0(100Frm,4K,LowQP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | -0.01% | -0.01% | 97% | 95% |
| Class C | 0.00% | -0.02% | -0.06% | 102% | 102% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.01% | -0.03% | -0.09% | 98% | 97% |
| Class F | -0.85% | 0.10% | 0.02% | 98% | 97% |
| TGM | 0.33% | 0.30% | 0.31% | 103% | 105% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0(100Frm,4K,LowQP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.01% | 0.02% | 104% | 102% |
| Class C | 0.00% | 0.00% | -0.01% | 97% | 99% |
| Class E | -0.02% | 0.02% | 0.06% | 104% | 106% |
| Overall | 0.00% | 0.00% | 0.02% | 101% | 102% |
| Class D | -0.01% | 0.01% | -0.04% | 96% | 99% |
| Class F | 0.04% | -0.04% | -0.12% | 101% | 100% |
| TGM | 0.00% | 0.04% | 0.07% | 102% | 102% |

Conclusion

It is confirmed that the cross-check results are identical with the proponent’s results.

Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).