| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0724-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-Q0627: On Dependent Quantization: Combination of JVET-Q0243 and JVET-Q0227 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Heiko Schwarz | Email: | heiko.schwarz@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

The cross checker reports that the source code provided by InterDigital was carefully analyzed and found to be consistent with the description in the input document JVET-Q0627.  The cross checker could exactly reproduce the results reported in JVET-Q0627.  The measured encoder and decoder run time ratios are similar to those reported in JVET-Q0627.

# Source Code Analysis

The source code was carefully analyzed. It is confirmed that the implementation matches the description.

The proposal represents a straightforward combination of the proposals JVET-Q0243 and JVET-Q0227:

- JVET-Q0243 proposes to support dependent quantization with 8 states in addition to dependent quantization with 4 states. The state transition table used is chosen based on a corresponding picture header / picture parameter set syntax element. All other aspects of the decoding process are the same for both variants of dependent quantization.

- JVET-Q0227 proposes an QP-dependent QP offset for dependent quantization. Instead of a fixed offset of 1 (as in VTM-7), it is proposed to derive the QP offset as follows:

- If qP > (30 + QpBdOffset), qPOffset = 2

- Otherwise, if qP > (23 + QpBdOffset), qPOffset = 1

- Otherwise, qPOffset = 0

# Simulation results

Simulations were conducted for all configurations and test sequences specified in the Common Test Conditions.  The simulation results for JVET-Q0627 are compared to VTM-7.0. The complete simulation results are provided in the accompanying Excel documents. A summary of the results is shown in the following tables.  The bit rates and PSNR values exactly match the results reported in JVET-Q0627. Encoding/Decoding runtimes are reported to be of same order as the ones reported by the proponent.

Table 1:  Average simulation results for the proposed combination of dependent quantization with 8 states and the QP offset modification.

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.50% | 0.23% | 0.53% | 109% | 100% |
| Class A2 | -0.91% | 0.47% | 0.31% | 111% | 98% |
| Class B | -0.48% | 0.50% | 0.40% | 112% | 96% |
| Class C | -0.23% | 0.41% | 0.51% | 112% | 94% |
| Class E | -0.29% | 0.46% | 0.40% | 108% | 97% |
| Overall | -0.47% | 0.42% | 0.43% | 110% | 97% |
| Class D | -0.26% | 0.68% | 0.53% | 113% | 93% |
| Class F | -0.15% | 0.31% | 0.40% | 110% | 98% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.25% | 0.29% | 0.12% | 103% | 97% |
| Class A2 | -0.63% | -0.09% | -0.35% | 103% | 92% |
| Class B | -0.45% | -0.11% | -0.14% | 106% | 96% |
| Class C | -0.26% | -0.16% | -0.14% | 106% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0.39% | -0.04% | -0.13% | 105% | 95% |
| Class D | -0.28% | -0.24% | -0.05% | 106% | 102% |
| Class F | -0.23% | -0.10% | -0.11% | 106% | 97% |
|  |  |  | Low delay B |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.62% | -0.05% | 0.12% | 104% | 94% |
| Class E | -0.28% | 0.71% | 0.85% | 100% | 96% |
| Overall |  |  |  |  |  |
| Class D | -0.45% | 0.87% | 0.40% | 101% | 96% |
| Class F | -0.62% | -0.46% | 0.34% | 103% | 96% |