| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0736-v1 |
| --- | --- |

| Title: | Cross-check result of JVET-Q0251: CE5-related: Unified cross component adaptive loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Z. Zhang | Tel: Email: | +46761499428 zhi.a.zhang@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This document reports the results of crosschecking JVET-Q0251 provided by Qualcomm. JVET-Q0251 proposes to unify the signaling of the CC-ALF filter coefficients for the two chroma components. The two components Cb and Cr can share the same filters in an APS.

The source code in JVET-Q0251 bases on VTM7.0. The update of the code had been reviewed and confirmed consistent with the proposal description. The encoder implementation is different compared to the CE5 common base, however, the source code in JVET-Q0251 has implement the encoder feature of common base to provide a fair comparation. An all intra simulation results shows that when the feature of JVET-Q0251 is turned off, the simulation results have minor BDR (0.00%/0.00%/0.03% for Y/U/V of all intra configuration) impact compared to CE5 common base.

The draft text change had been reviewed and confirmed consistent with the proposal description.

The simulation results match the results reported in JVET-Q0251.

# Verification

Qualcomm had provided the source code and results of their proposed methods in JVET-Q0251. The verification simulation results match the results in JVET-Q0251.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE5 Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.10% | 0.16% | 97% | 98% |
| Class A2 | 0.06% | -0.12% | -0.92% | 96% | 97% |
| Class B | 0.02% | -0.36% | 0.17% | 96% | 99% |
| Class C | -0.06% | 0.22% | 0.02% | 92% | 100% |
| Class E | -0.09% | 0.64% | -0.58% | 94% | 101% |
| Overall | -0.01% | 0.05% | -0.17% | 95% | 99% |
| Class D | -0.01% | -0.15% | -0.65% | 94% | 101% |
| Class F | -0.02% | 0.01% | 0.09% | 98% | 100% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE5 Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.16% | 0.30% | 100% | 99% |
| Class A2 | 0.08% | -0.30% | -0.70% | 102% | 100% |
| Class B | 0.02% | -0.05% | 0.05% | 100% | 100% |
| Class C | -0.03% | 0.00% | 0.00% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.04% | -0.06% | 101% | 100% |
| Class D | -0.03% | -0.29% | -0.30% | 101% | 101% |
| Class F | -0.02% | 0.09% | -0.10% | 107% | 99% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE5 Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.25% | 0.47% | 98% | 97% |
| Class C | -0.03% | -0.02% | -0.38% | 98% | 100% |
| Class E | -0.08% | 0.23% | -0.92% | 96% | 101% |
| Overall | -0.04% | -0.05% | -0.16% | 98% | 99% |
| Class D | -0.01% | -0.49% | -1.07% | 96% | 102% |
| Class F | 0.07% | -0.09% | 0.79% | 100% | 96% |