| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0739 |
| --- | --- |

| Title: | Crosscheck of JVET-Q0442 (Transform Selection for MIP in Implicit MTS) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Michael Schäfer, Jonathan Pfaff Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-642 michael.schaefer@hhi.fraunhofer.de jonathan.pfaff@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results for JVET-Q0442 (Transform Selection for MIP in Implicit MTS). In this documents, it is proposed to use the combination of DCT2 and DST7 instead of the DCT2 for implicit MTS if the intra prediction mode is an MIP mode and the width and height of the transform block are less or equal than 16. The cross checker verified that the software matches the description and the proposed changes in the draft text. The simulation results reportedly match those provided by the proponents.

# Crosscheck results

The implementation in the provided software matches the description. The BD rates as reported in the proposal were reproduced. The runtime behavior is similar to the values reported in the proposal.

# Experimental results

## Results test

All intra:

|  | Over VTM-7.0 with --MTS=0 --MTSImplicit=1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | 0.01% | 0.05% | 99% | 101% |
| Class A2 | -0.05% | 0.03% | -0.01% | 100% | 104% |
| Class B | 0.00% | -0.06% | -0.05% | 100% | 98% |
| Class C | -0.01% | 0.08% | -0.15% | 101% | 99% |
| Class E | -0.06% | 0.03% | 0.03% | 100% | 99% |
| Overall | -0.03% | 0.01% | -0.04% | 100% | 100% |
| Class D | -0.01% | 0.00% | -0.03% | 99% | 103% |
| Class F | -0.04% | 0.11% | 0.03% | 101% | 99% |

Random access:

|  | Over VTM-7.0 with --MTS=0 --MTSImplicit=1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.06% | 0.09% | 101% | 99% |
| Class A2 | 0.01% | -0.03% | 0.00% | 100% | 103% |
| Class B | -0.04% | 0.04% | 0.06% | 100% | 100% |
| Class C | -0.03% | 0.01% | -0.18% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.00% | -0.01% | 100% | 100% |
| Class D | -0.05% | -0.07% | -0.19% | 100% | 101% |
| Class F | -0.01% | -0.18% | -0.06% | 100% | 102% |