| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0743-v1 |
| --- | --- |

| Title: | AHG9: A summary of proposals on HRD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yago Sanchez | Tel: Email: | yago.sanchez@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution intends to provide a summary of all proposals on HRD

It is suggested that this summary is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner.

# List of proposals on HRD

This category of proposals is listed in the agenda item 6.19.11, including 10 contributions. These contributions and summaryies of the related proposals are listed below.

JVET-Q0048 AHG9: On order of HRD related SEI messages [V. Drugeon (Panasonic)]

Parsing of the picture timing and the decoding unit information SEI messages is dependent on information signalled within the buffering period SEI message. In order to simplify parsing of these SEI messages, the following constraints are proposed:

When both BP and PT SEI messages are present within an AU, the BP SEI messages shall precede the PT SEI messages in decoding order.

When both BP and DUI SEI messages are present within an AU, the BP SEI messages shall precede the DUI SEI messages in decoding order.

JVET-Q0216 AHG9: On Picture Timing Information Signalling [S. Deshpande (Sharp)]

A syntax modification is proposed to picture timing SEI message. It is proposed that when cpb_removal_delay_delta_enabled_flag[i] is equal to 1 for i in the range of TemporalId to bp_max_sub_layers_minus1-1, inclusive the syntax element cpb_removal_delay_delta_idx[i] be only signalled if num_cpb_removal_delay_deltas_minus1 is greater than 0. Otherwise the value of cpb_removal_delay_delta_idx[i] is inferred.

JVET-Q0219 AHG9: On Alternative Timing Information Signalling [S. Deshpande (Sharp)]

Modifications are proposed to the alternative timing information signaling for HRD of VVC.

Following is proposed:

- Proposal 1: It is proposed to signal a separate set of alternative buffering delay parameters for VCL HRD and for NAL HRD. Additionally the HRD operation is correspondingly modified.

- Proposal 2:  It is proposed that a separate set of alternative buffering delay parameters be signalled for different temporal sub-layer representations. Additionally the HRD operation is correspondingly modified.

JVET-Q0221 AHG9: On Decoding Unit Parameters Signalling [S. Deshpande (Sharp)]

Modifications are proposed related to decoding unit parameters signalling including information in PT and DU information message. Following is proposed:

If DU level HRD parameters are present, a flag is signalled in Buffering period SEI message to control if DU level picture DPB output delay information is signalled only in picture timing SEI message or in DU information SEI message.

JVET-Q0283 AHG9: On subbitstream extraction [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

Included in summary JVET-Q0694.

JVET-Q0393 AHG9: On DU based HRD with temporal scalability [Y. Sanchez, R. Skupin, K. Sühring, T. Schierl (HHI)]

This contribution asserts that the values of delay increment in the loop over temporal layers for DU timing signalling in PT and DUI SEI messages can be derived with a scaling factor for the tick devisor using the highest temporal layer delay increment. It is proposed to remove the loop and add a syntax element for the scaling factor to the HRD syntax in the SPS.

JVET-Q0394 AHG9: On OLS extraction and scalable nesting SEI message [Y. Sanchez, R. Skupin, K. Sühring, T. Schierl (HHI)]

This proposal is two-fold.

First, it is proposed to address the following aspects related to scalable nesting:

- Proposal for non-nested BP/PT/DUI SEI messages being applicable to OLS corresponding to the bitstream (instead of 0-th OLS).

- Normative text for extracting HRD related SEI messages encapsulated in scalable nesting SEI messages

Second, changes to the PT SEI message are proposed that allow sharing of non-scalable nested PT SEI messages among OLSs or when using DU timing.

JVET-Q0397 AHG12: On subpicture extraction [R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)]

Included in summary JVET-Q0694.

JVET-Q0404 AHG12: On CBR subpicture extraction [R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)]

Included in summary JVET-Q0694.

JVET-Q0407 AHG9: On alternative HRD timing and temporal sub-layers [Y. Sanchez, R. Skupin, K. Sühring, T. Schierl (HHI)]

This contribution proposes to extend the signalling for alternative removal times in picture timing SEI messages to support temporal scalability when dropping of AUs occrus (leading pictures or up to DRAP) by providing one for each temporal sub-layer.

# Summary and list of design questions

- Mandate a bitstream order for BP/PT/DUI SEI message that obeys the parsing dependency (Q0048)?

- Not signal and infer cpb_removal_delay_delta_idx[ i ] to be 0 when there is only one cpb_removal_delay_delta[ i ] in BP SEI message? (Q0216)?

- Signal the alternative times in the PT SEI message for each sub-layer when sub-layer information is present instead of a single value (Q0219, Q0407)?

- Signal alternative times for VCL HRD and NAL HRD and not only one value (Q0219)?

- Change BP/PT/DUI SEI messages to apply to the bitstreamToDecode, which is an OLS (Q0394)?

- The extraction process exchanges the non-nested BP/PT/DUI SEI messages with the corresponding nested BP/PT/DUI SEI messages?

- Add a flag to general_constraint_flag() to indicate the PT SEI message applies to any OLS, which happens when no DU information is included and same framerate in all OLSs?

  - if yes, modify PT SEI message to support the same functionality for DU-based extraction (e.g. for subpicture extraction).?

du_common_cpb_removal_delay_flag is changed to be used when each DU corresponds to a single VCL NAL unit. The number of DUs within an AU is derived to be equal to ClockTick * ( elemental_duration_in_tc_minus1[ i ] + 1 ) divided by  ClockSubTick * (du_cpb_removal_delay_increment_minus1[ i ][ j ]  + 1)

- If DU level HRD parameters are present, a flag is signalled in Buffering period SEI message to control if DU level picture DPB output delay information is signalled only in picture timing SEI message or in DU information SEI message. (Q0221)?

  - If desired to signal it in both, in addition to the previous flag, signal the presence of the information also in the DUI SEI message with a flag in the PT SEI message?

- Remove the loop over temporal sub-layers for DU information in PT/DUI SEI message and derive a different ClockSubTick by indicating a scaling factor for the different sub-layers in the HRD parameters in parameter set (Q0393)?