| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0751 |
| --- | --- |

| Title: | AHG9: On mixed NAL unit types |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Sachin Deshpande Miska M. Hannuksela Hendry Seethal Paluri | Tel: Email: | +46 721 88 21 69 samuelssonj@sharplabs.com |
| Source: | Sharp Labs of America, Inc., Nokia Technologies, Tampere, Finland LG Electronics USA |  |  |

_____________________________

# Abstract

This contribution relates to support for mixed NAL unit types in VVC. It is proposed to modify the text related to mixed_nalu_types_in_pic_flag to allow support for mixing bitstreams containing CRA pictures. This joint contribution is a combination of aspects from JVET-Q0206, JVET-Q0239, and JVET-Q0261.

# Background

It is asserted that it would be beneficial to be able to mix NAL units from bitstream that use CRA pictures. A detailed motivation is provided in JVET-Q0239.

# Proposal

In order to provide support for CRA NAL units in Access Units with mixed NAL unit types, the following changes are proposed:

1. Update the constraints in subclause 7.4.2.2 about mixed_nalu_types_in_pic_flag as follows:

For VCL NAL units of any particular picture, the following applies:

- If mixed_nalu_types_in_pic_flag is equal to 0, the value of nal_unit_type shall be the same for all coded slice NAL units of a picture. A picture or a PU is referred to as having the same NAL unit type as the coded slice NAL units of the picture or PU.

- Otherwise (mixed_nalu_types_in_pic_flag equal to 1), a picture or a PU is a trailing picture regardless of the nal_unit_type values of the VCL NAL units contained in the picture and one of the following shall be true:

  - One or more of the VCL NAL units all have a particular value of nal_unit_type in the range of IDR_W_RADL to CRA_NUT, inclusive, and the other VCL NAL units all have a particular value of nal_unit_type in the range of TRAIL_NUT to RSV_VCL_6, inclusive, or equal to GRA_NUT.

  - One or more of the VCL NAL units all have a particular value of nal_unit_type equal to RADL_NUT or RASL_NUT and the other VCL NAL units all have a particular value of nal_unit_type equal to TRAIL_NUT, STSA_NUT, RSV_VCL_4, RSV_VCL_5, RSV_VCL_6, or GRA_NUT.

  - All VCL NAL units have a particular value of nal_unit_type in the range of TRAIL_NUT to RSV_IRAP_12, inclusive.

2. Change the semantics of mixed_nalu_types_in_pic_flag in clause 7.4.3.4 as indicated by the yellow highlights. By changing "do" to "may", fewer PPS updates would be needed, and PPSs for the merged bitstream can be provided by the content author and no client-side rewriting of the PPSs is required.

mixed_nalu_types_in_pic_flag equal to 1 specifies that each picture referring to the PPS has more than one VCL NAL unit and that the VCL NAL units may do not have the same value of nal_unit_type and that the picture is a trailing not an IRAP picture. mixed_nalu_types_in_pic_flag equal to 0 specifies that each picture referring to the PPS has one or more VCL NAL units and the VCL NAL units of each picture refering to the PPS have the same value of nal_unit_type.

3. Modify the constraints in clause 7.4.3.4 by adding the yellow-highlighted part.

For each slice with a nal_unit_type value nalUnitTypeA in the range of IDR_W_RADL to CRA_NUT, inclusive, in a picture picA that also contains one or more slices with another value of nal_unit_type (i.e., the value of mixed_nalu_types_in_pic_flag for the picture picA is equal to 1), the following applies:

–	The slice shall belong to a subpicture subpicA for which the value of the corresponding subpic_treated_as_pic_flag[ i ] is equal to 1.

–	The slice shall not belong to a subpicture of picA containing VCL NAL units with nal_unit_type not equal to nalUnitTypeA.

–	If nalUnitTypeA is equal to CRA:

For all the PUs in the CLVS following the current picture in decoding order and in output order, neither RefPicList[ 0 ] nor RefPicList[ 1 ] of a slice in subpicA shall include any picture preceding picA in decoding order in an active entry.

–	Otherwise:

For all the following PUs in the CLVS following the current picture in decoding order, neither RefPicList[ 0 ] nor RefPicList[ 1 ] of a slice in subpicA shall include any picture preceding picA in decoding order in an active entry.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).