| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0759-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-Q0295 (On residual coding for MTS) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Moonmo Koo 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +1-858-635-5226 moonmo.koo@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This is cross-check report for JVET-Q0295 (On residual coding for MTS). The cross-check result matched with what is provided by proponent in JVET-Q0295.

Introduction

In the current VVC WD, an MTS index is not signalled and inferred as 0 if the last significant position is outside of the upper-left 16×16 region. However, even if the last significant position is inside of the upper-left 16x16 region, the MTS index could be 0, because coefficient scanning outside of the upper-left 16x16 region where non-zero coefficients exist can happen for a luma TB larger than 16x16. For this case, a bitstream conformance constraint is described in the current spec text which enforces the MTS index to be zero.

JVET-Q0295 proposes a scan pattern such that the last position check is sufficient to ensure no significant coefficients are present outside of the 16×16 region. Specifically, the sub-blocks of TBs larger in width or height of 16 samples are scanned in a tiled manner, such that the scan progresses from one 16×16 region to the next 16×16 region. Furthermore, in order to avoid increasing storage requirements, the scan patterns are changed globally, so chroma TBs use the same region-wise scanning even though MTS is not applicable to chroma TBs.

Source code analysis

It was confirmed that code implementation is aligned with what JVET-Q0295 describes.

Experimental results

The software package was obtained from the proponent. Code is compiled and tested following JVET common test condition [1], and compared with the result of VTM 7.0 as an anchor. The available BD-rate results and run-time ones match with what is provided by proponent.

Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010, March 2019.