| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0769 |
| --- | --- |

| Title: | BoG Report on CE1 informal viewing session |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Mathias Wien Lehrstuhl für Bildverarbeitung RWTH Aachen University Germany | Tel: Email: | +49-241-8027867 wien@lfb.rwth-aachen.de |
| Source: | BoG |  |  |

_____________________________

# Abstract

The BoG met on January 9th 2020 from 19:00h to 20:45h in the BOZAR viewing room. The meeting was announced on the JVET email reflector in the afternoon of the same day. There were 12 viewing participants, including four proponents representing all assessed sub-CEs. A total of 15 A-B comparisons of proposals against their anchors were performed, with a randomized and undisclosed assignment of the anchor and the proposal to A or B. The presented sequences were selected by recommendation of the proponents. Each set was shown multiple times according to the request of the participants and participants were encouraged to watch each set from different viewing positions. Non-proponent comments and expressions of preference were collected first before allowing proponents to comment. Only after completion of this assessment, the A-B assignment of anchor and proposal was disclosed. For CE1-1 a slight preference for the proposed changes of CE1-1.1 and CE1-1.4 as found, without a clear preference for one of the two variants. For CE1-2 and CE1-3, no clear preference for the proposals or the anchor was found. In some cases, artifacts not related to the CE proposals, such as pumping and artifacts on CTU boundaries were observed.

# Visual Assessment

## Test setup

The raw YUV sequences were presented on an LG OLED55B9PLA display using a Linux PC (Ubuntu 16.04) with a pre-installed video player capable of playing UHD 60 Hz video at full frame rate and stable output. The viewing room was dark with a low indirect light source placed behind the display.

The viewing compared the reconstruction quality of the proposals under test to the reconstruction quality of the VTM 7 anchor.

For each test cell, let video A and video B denote either the anchor or the proposal for the same sequence and same QP for a test point of the core experiment under evaluation. The assignment of anchor and proposal to A or B was randomized and not revealed to the test subjects before completing the collection of comments. A test cell was constructed as follows:

- “A” (on a mid-gray background, 0.5 s presentation)

- Video A

- “B” (on a mid-gray background, 0.5 s presentation)

- Video B

- “A” (on a mid-gray background, 0.5 s presentation)

- Video A

- “B” (on a mid-gray background, 0.5 s presentation)

- Video B

Each test cell was presented to the participants multiple times according to the request of the participants.

## Recorded Assessment Notes

The sub-CE results are recorded here in the order of the conducted visual assessments. The proponents were requested to recommend the sequences to assess, with the best example to be shown first. Before starting the assessment, the proponents were asked to briefly summarize tool under test and indicate the expected changes.

### CE1-2

Proponent expresses that only subtle differences are to be expected, mainly due to different encoder decisions. Changes to be expected in the corner of blocks.

| Test point | Comments |
| --- | --- |
| S01 BasketballDrill QP32 | No preference for A or B |
| S01 BasketballDrill QP37 | Some experts express preference for A or for B, at same numbers General comments: - CTU boundaries visible, especially on the diagonal structures in the content - Persistent artifacts of crossing objects on the floor area visible |
| S04 Cactus QP37 | Slight preference for B (the proposal) |
| S05 KristenAndSara QP37 | Slight preference for A (the anchor) |
| S02 Campfire QP37 | Some experts express preference for A or for B, at same numbers General comment: Pumping visible |

Conclusion: no clear preference.

### CE1-3

Variant 1-3.2 considered the more exact approach. Effect expected to be more visible on LDB sequences, also more difference between the two variants.

| Test point | Comments |
| --- | --- |
| S01 BasketballDrill QP37 CE1-3.2 | Slight preference for A (the proposal) |
| S04 Cactus QP37 CE1-3.2 | No preference for A or B |
| S05 KristenAndSara QP37 CE1-3.2 | Slight preference for A (the anchor) |
| S01 BasketballDrill QP32 CE1-3.1 | Slight preference for A (the anchor) |

Conclusion: no clear preference.

### CE1-1

Proposals might filter (i.e. remove) less content, potentially remove some flickering artifacts.

| Test point | Comments |
| --- | --- |
| S08 EBUKidsSoccer QP37 | - CE1-1-1: Preference for B (proposal) - CE1-1-4: Preference for B (proposal) - CE1-1-1 indicated to be slightly better than 1-1-4 General comment: Pumping effect observer, especially in the trees |
| S04 Cactus QP37 | - Both variants preference for A (anchor in one, proposal in the other one) - CE1-1-1 and CE1-1-4 very similar in effect |
| S06 AlohaWave QP37 | No clear preference (some favor proposals, but also anchor). Potentially very slight benefit for 1.1.4 (only one expert) |

Conclusion: Proposals both show a slight benefit, no clear preference between the two.