| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0773-v1 |
| --- | --- |

| Title: | AHG9: A summary of HLS contributions on mixed NAL unit types within a coded picture |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hendry | Email | dr.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution is for information summarizing contributions on mixed NAL unit type within a coded picture (i.e., contributions in category 6.19.7).

# Summary

- In addition to coded picture with mixed IRAP and non-IRAP NAL unit type, do we want to allow mixed of other NAL unit types as well? (JVET-Q0206, JVET-Q0239, JVET-Q0284, and JVET-Q0396).

  - Allow more mix, still use the mixed_types_in_pic_flag as indication.

    - JVET-Q0206 aspect 2 & JVET-Q0284 aspect 3: allow picture to have mixed NAL unit types of leading picture type (i.e., RASL / RADL) and non-IRAP, non-leading picture NAL unit types.

    - JVET-Q0239: remove the constraint about what types can be mixed.

  - JVET-Q0163: allow picture to have mixed NAL unit types. When mixed_types_in_pic_flag is equal to 1, signal another flag mixed_irap_types_in_pic_flag to specify if the mixture involve IRAP and non-IRAP.

  - JVET-Q0284 & JVET-Q0396: change mixed_nalu_types_in_pic_flag to mixed_nalu_types_in_pic_idc

- JVET-Q0284: 0 means no mix; 1 means IRAP type with non-IRAP type; 2 means RASL/RADL/GDR_NUT with other non-IRAP types; 3 is reserved.

- JVET-Q0396: 0 means no mix; 1 means non-IRAP type with non-IRAP type; 2 means IRAP type with non-IRAP type; 3 means IDR_W_RADL with CRA_NUT.

- What kind of picture is the picture that has mixed NAL unit types?

  - JVET-Q0239 and JVET-Q0284: Picture containing mixed NAL unit types is a trailing picture.

  - JVET-Q0396: Picture type depends on mixture:

- If containing TRAIL slices,

- Otherwise (no TRAIL), picture is treated as leading picture with differentiation of RADL and RASL

- Otherwise (no TRAIL, no RAS/DL), picture is treated as CRA.

- New syntax element?

  - JVET-Q0284: move the flag from PPS to PH and add a flag in SPS to specify that mixed NAL unit picture may be present in the CLVS.

- The following changes are proposed:

  - JVET-Q0206: add a constraint such that the value of sps_idr_rpl_present_flag shall be equal to 1 when there is at least one PPS referring to the SPS with the value of mixed_nalu_types_in_pic_flag is equal to 1.

  - JVET-Q0208: have a constraint such that pictures that follow the current picture, which is a mixed of IRAP and non-IRAP NAL unit picture, in the decoding order do not use pictures that precede the current picture in decoding order as TMVP collocated reference picture.

  - JVET-Q0239: a picture referring to PPS with mixed_nalu_types_in_pic_flag equal to 1 may or may not contains VCL NAL unit of different NAL types.

  - JVET-Q0289: similar in spirit with JVET-Q0239 that allow a picture referring to PPS with mixed_nalu_types_in_pic_flag equal to 1 may or may not contains VCL NAL unit of different NAL types. When mixed_nalu_types_in_pic_flag is equal to 1, signal a flag in PH to specify if the picture associated with the PH contains slices with different NAL unit types.

  - JVET-Q0261: modify current constraint to make the current NAL unit mixed work particularly when a picture contains CRA and trail NAL units.

  - JVET-Q0284: add text to clearly specify picture type according to type of their VCL NAL unit.JVET-Q0284: constraint that if mixed NAL unit picture use inter-layer prediction, the inter-layer ref pic must also be mixed NAL unit picture.

- How do we want to handle mixing of STSA slices?

  - JVET-Q0284: disallow mixture of STSA and IRAP, or remove TemporalId constraint for STSA_NUT.

  - JVET-Q0396: have a flag called leading_stsa_pictures_flag in general_constraints_info( ).

leading_stsa_pictures_flag equal to 1 specifies that inter prediction reference for RADL and RASL pictures are constraint as specified below. leading_stsa_pictures_flag equal to 0 does not impose such a constraint.

When leading_stsa_pictures_flag equal to 1, the following applies:

- When the current picture is a RASL or RADL picture, there shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that has TemporalId equal to that of the current picture.

- When the current picture is a picture that follows, in decoding order, a RASL or RADL picture and preceds the associated IRAP picture that has TemporalId equal to that of the current picture, there shall be no picture that has TemporalId equal to that of the current picture included as an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes the RASL or RADL picture in decoding order.

# List of reviewed contributions

JVET-Q0163 AHG9: On mixed NAL unit types in a video picture [P. Wu (ZTE)]

This contribution proposed to add another syntax element called mixed_irap_nalu_types_in_pic_flag, in addition to the existing mixed_nalu_types_in_pic_flag, to indicate mixed irap nalu types and only irap nalu types in a video picture. It is asserted that the added indication will support wider range of application, particularly involving subpictures partition applications. It is also asserted that the added syntax element can be used to serve a flexible random access point indications with rather independently coded subpictures in a video picture.

JVET-Q0206 AHG9: On picture with mixed NAL unit types [Hendry (LGE)]

This contribution asserted that the current specification text regarding picture with mixed NAL unit types has the following problems:

- When a CLVS contains one or more PPSs with mixed_nalu_types_in_pic_flag is equal to 1, it is still possible that the value of sps_idr_rpl_present_flag is not equal to 1. Such situation would make the signalling of RPL in slice header of picture with mixed IDR and non-IRAP NAL unit types does not work.

- Currently for picture with mixed NAL unit types, the picture shall contain only one particular type of IRAP NAL unit type and one particular type of non-IRAP NAL unit type. If it is possible that a picture contains mixed of CRA and non-IRAP NAL unit types, it is inevitable that a picture with leading picture and non-IRAP NAL unit types is allowed, which is not yet.

This contribution proposes the following:

- Specify a constraint such that the value of sps_idr_rpl_present_flag shall be equal to 1 when there is at least one PPS referring to the SPS with the value of mixed_nalu_types_in_pic_flag is equal to 1.

- Allow picture to have mixed NAL unit types of leading picture type (i.e., RASL / RADL) and non-IRAP, non-leading picture NAL unit types.

JVET-Q0208 [AHG9]: On TMVP collocated reference picture [Hendry (LGE)]

This contribution proposed that a constraint should be placed to ensure that the pictures that follow the current picture, which is a mixed of IRAP and non-IRAP NAL unit picture, in the decoding order do not use pictures that precede the current picture in decoding order as TMVP collocated reference picture.

The proposed text:

It is a requirement of bitstream conformance that when the value of mixed_nalu_types_in_pic_flag is equal to 1 and the current picture contains one of more NAL unit of IRAP type, the collocated picture used by any of picture that follows the current picture in decoding order shall not be picture that precedes the current picture in decoding order.

NOTE – When the current picture contains one or more subpicture that may be merged with subpicture from other bitstream and the resulted picture may contains mixed NAL unit types with one or more NAL units is IRAP type, encoder has to ensure that collocated picture selected for any pictures that follows the current picture in decoder order is not picture that precedes the current picture in decoding order.

Furthermore, this contribution suggest the JVET group to consider signalling a flag in picture header such that when the flag is equal to 1, it specifies that pictures that follow the picture associated with the picture header shall not use pictures that precede the picture associated with the picture header in decoding order as TMVP collocated reference picture.

JVET-Q0239 AHG9: On mixed NAL unit types in a coded picture [M. M. Hannuksela (Nokia)]

This contribution asserted that the capability of mixing of RASL and TRAIL subpictures into the same coded picture would be beneficial for rate-distortion performance and could also reduce the quality pumping that might be experienced with IDR pictures. The changes for enabling mixing RASL and TRAIL subpictures into the same coded picture are asserted to be small.

The following is proposed:

- A picture referencing a PPS with mixed_nalu_types_in_pic_flag equal to 1 is a trailing picture in the decoding process.

- An IRAP subpicture in a picture with mixed_nalu_types_in_pic_flag equal to 1 is allowed to be followed, in decoding order, by RASL subpicture(s) that include pictures preceding, in decoding order, the IRAP subpicture in their reference picture lists.

- A picture referencing a PPS with mixed_nalu_types_in_pic_flag equal to 1 may or may not contain VCL NAL unit of different nal_unit_type values.

JVET-Q0261 AHG9: On mixed NAL unit types [J. Samuelsson, S. Deshpande, A. Segall (Sharp)]

This contribution proposed to modify the semantics of mixed_nalu_types_in_pic_flag so that CRA NAL units are handled differently than other NAL units and allow leading pictures.

The proposed modification involves modification of the following constraint:

For each slice with a nal_unit_type value nalUnitTypeA in the range of IDR_W_RADL to CRA_NUT, inclusive, in a picture picA that also contains one or more slices with another value of nal_unit_type (i.e., the value of mixed_nalu_types_in_pic_flag for the picture picA is equal to 1), the following applies:

–	The slice shall belong to a subpicture subpicA for which the value of the corresponding subpic_treated_as_pic_flag[ i ] is equal to 1.

–	The slice shall not belong to a subpicture of picA containing VCL NAL units with nal_unit_type not equal to nalUnitTypeA.

–	If nalUnitTypeA is equal to CRA:

For all the following PUs in the CLVS following the current picture in decoding order and in output order, neither RefPicList[ 0 ] nor RefPicList[ 1 ] of a slice in subpicA shall include any picture preceding picA in decoding order in an active entry.

–	Otherwise:

For all the PUs in the CLVS following the current picture in decoding order, neither RefPicList[ 0 ] nor RefPicList[ 1 ] of a slice in subpicA shall include any picture preceding picA in decoding order in an active entry.

JVET-Q0284 AHG9: On mixed NAL unit type [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

This contribution asserted that the current spec has the following issues and proposed a solution for each of the asserted issue as follow:

- Issue 1: The picture type is ambiguous when the picture is composed of mixed NAL unit types.

  - Proposal 1-1: Individually define the picture type depending on the nal_unit_type, to maintain the full functionality.

  - Proposal 1-2:Propose that the picture with mixed_nalu_types_in_pic_flag equal to 1 is set equal to a trailing picture.

- Issue 2: When NAL unit types are mixed in the picture, TemporalId contraints may have conflict.

  - Proposal 2-1: Propose to disallow the mixture of STSA NAL units and IRAP NAL units in the same PU.

  - Proposal 2-2: Propose to remove the TemporalId constraint on STSA_NUT.

- Issue 3: If mixed_nalu_types_in_pic_flag is signalled in PPS, at least two PPS NAL units shall be referred to by slice NAL units in a CLVS.

  - Proposal 3: Propose to move the syntax element mixed_nalu_types_in_pic_flag to PH.

- Issue 4: The current design cannot support the coexistence of RADL/RASL NAL units with Trail NAL units in a PU.

  - Proposal 4: Propose to replace the flag mixed_nalu_types_in_pic_flag with an indicator mixed_nalu_types_in_pic_idc, to indicate more mixture types.

- Issue 5: When a picture in a layer references another picture in a different layer in the same AU, the values of mixed_nalu_types_in_pic_flag shall be aligned across those layers.

  - Proposal 5: Propose to align the value of mixed_nalu_types_in_pic_flag between a dependent layer and its reference layer.

JVET-Q0289 AHG9/AHG12: Comments on miscellaneous HLS text [L. Chen, C.-W. Hsu, C.-C. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

Item 2 of this contribution belongs to this category.

Proposal 2: When the value of mixed_nalu_types_in_pic_flag is equal to 1 in PPS ppsA, signal a flag in PH referring to ppsA (i.e., mixed_nalu_types_present_flag) to specify whether the picture associated with the PH contains slices with different NAL unit types.

This contribution, in spirit, allows a picture referring to PPS with mixed_nalu_types_in_pic_flag equal to 1 may or may not contains VCL NAL unit of different NAL types.

JVET-Q0396 AHG9: On mixing NAL unit types in a coded picture [R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)]

This contribution asserted that the current draft specification only considers mixing IRAP types and TRAIL_NUT up to RSV_VCL_6 or GDR_NUT which is not sufficient for envisioned applications and proposed a set of rules for NAL unit types that are allowed to be mixed within a coded picture.

It is asserted tht compared to the current draft specification, the proposal in this contribution allows TRAIL_NUT and STSA_NUT NAL unit types to be mixed with NAL unit types such as RADL_NUT and RASL_NUT.

This contribution also proposed to allow mixing CRA_NUT and IDR_W_RADL NAL units, as well as RADL_NUT and RASL_NUT NAL units.

This contribution also proposed to have a flag called leading_stsa_pictures_flag in general_constraint_info( )

leading_stsa_pictures_flag equal to 1 specifies that inter prediction reference for RADL and RASL pictures are constraint as specified below. leading_stsa_pictures_flag equal to 0 does not impose such a constraint.

When leading_stsa_pictures_flag equal to 1, the following applies:

- When the current picture is a RASL or RADL picture, there shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that has TemporalId equal to that of the current picture.

- When the current picture is a picture that follows, in decoding order, a RASL or RADL picture and preceds the associated IRAP picture that has TemporalId equal to that of the current picture, there shall be no picture that has TemporalId equal to that of the current picture included as an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes the RASL or RADL picture in decoding order.

JVET-Q0751 AHG9: On mixed NAL unit [J. Samuelsson, S. Deshpande(Sharp), M. M. Hannuksela (Nokia), Hendry, S. Paluri (LGE)]

This contribution relates to support for mixed NAL unit types in VVC. It is proposed to modify the text related to mixed_nalu_types_in_pic_flag to allow support for mixing bitstreams containing CRA pictures. This joint contribution is a combination of aspects from JVET-Q0206, JVET-Q0239, and JVET-Q0261.

The aspects that are included in this joint contribution are:

- Allow picture to have mixed NAL unit types of leading picture type (i.e., RASL / RADL) and non-IRAP, non-leading picture NAL unit types.

- Picture that contains mixed NAL unit types is treated as TRAIL picture.

- A picture referring to PPS with mixed_nalu_types_in_pic_flag equal to 1 may or may not contains VCL NAL unit of different NAL types.

- Modify current constraint to make the current NAL unit mixed work particularly when a picture contains CRA and trail NAL units.

This contribution asserted that the current draft specification only considers mixing IRAP types and TRAIL_NUT up to RSV_VCL_6 or GDR_NUT which is not sufficient for envisioned applications and proposed a set of rules for NAL unit types that are allowed to be mixed within a coded picture.