| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0774 |
| --- | --- |

| Title: | Crosscheck of JVET-Q0274 (MIP with fixed length mode coding and memory reduction) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Michael Schäfer, Jonathan Pfaff Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-642 michael.schaefer@hhi.fraunhofer.de jonathan.pfaff@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results for JVET-Q0274 (MIP with fixed length coding and memory reduction). In this document, it is proposed to change the number of MIP-modes from 12 to 8 for MIP-sizeId=2. Thus, for MIPsizeId=2, 4 instead of 6 MIP matrices need to be stored in the proposed technology. Also, in the proposed technology, the MIP-mode can always be coded using a fixed length code instead of a truncated binary code.

# Crosscheck results

The implementation in the provided software matches the description. The BD rates as reported in the proposal were reproduced. The runtimes deviate due to cluster inaccuracies on the cross checker’s cluster.

# Experimental results

## Results test

All intra:

|  | Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,03% | -0,23% | 0,06% | 112% | 103% |
| Class A2 | 0,03% | -0,02% | -0,06% | 110% | 102% |
| Class B | 0,02% | 0,00% | -0,09% | 112% | 103% |
| Class C | 0,00% | 0,05% | 0,01% | 110% | 109% |
| Class E | 0,00% | -0,06% | 0,06% | 111% | 108% |
| Overall | 0,02% | -0,04% | -0,01% | 111% | 105% |
| Class D | -0,01% | 0,04% | -0,06% | 115% | 102% |
| Class F | -0,03% | -0,03% | 0,07% | 112% | 101% |

Random access:

|  | Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,02% | 0,04% | 0,00% | 105% | 100% |
| Class A2 | 0,00% | -0,02% | 0,05% | 106% | 99% |
| Class B | 0,01% | 0,07% | 0,16% | 109% | 97% |
| Class C | 0,03% | -0,01% | -0,01% | 104% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0,02% | 0,02% | 0,06% | 106% | 98% |
| Class D | 0,03% | -0,07% | -0,11% | 107% | 98% |
| Class F | -0,01% | 0,02% | 0,10% | 117% | 92% |