| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0775 |
| --- | --- |

| Title: | AHG9: Picture Header in Slice Header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Sachin Deshpande (Sharp) Rickard Sjöberg Martin Pettersson Mitra Damghanian Davood Saffar (Ericsson) Ling Li, Byeongdoo Choi, Xiaozhong Xu, Xiang Li, Stephan Wenger, Shan Liu (Tencent) | Tel: Email: | +46 721 88 21 69 samuelssonj@sharplabs.com |
| Source: | Sharp Labs of America, Inc. Ericsson Tencent |  |  |

_____________________________

# Abstract

This contribution contains a proposal for signalling the picture header in the slice header. The contribution is based on JVET-Q0255, JVET-Q0419, JVET-Q0426. The introduction and overhead analysis provided in the contribution has been copied from JVET-Q0255.

# Introduction

In development of the VVC standard it has been identified that support for in-picture error concealment is not a high priority design goal and it has been decided to gather parameters that applies to a picture in a picture header instead of repeating it in every slice header. Each picture contains exactly one picture header and it will generally not be possible to parse, decode or interpret a coded slice unless the associated picture header is available. Consequently, there will be limited benefit to splitting up pictures in slices, especially since parallel encoding (such as tiles and wavefronts) are supported in a single slice. One exception is Virtual Reality applications where slices can be used to extract and decode only parts of the video (e.g. using subpictures). However, for most general video applications it can be assumed that the most efficient encoding will be achieved by encoding pictures in a single slice and it can be assumed that this will be a very common configuration. It would therefore be highly desirable to support this configuration as efficiently as possible.

## Low bitrate video applications

Even though modern video codecs generally enable support for higher resolutions, higher bitdepths and higher bitrates, it should be noted that new codecs are also used to provide a more acceptable and enjoyable video experience at the low end of the bitrate range. One example is the “HLS Authoring Specification for Apple Devices” [1] where a bitrate ladder starting at 145 kbps is recommended for both H.264/AVC and HECV/H.265. Another example is the low-quality profile recently introduced by Vimeo to increase reach and reduce buffer times. The profile uses AVC/H.264 at 300 kbps [2].

Since VVC provides clearly improved compression performance compared to HEVC and massively improved compression performance over H.264/AVC it reasonable to assume that VVC can be used at bitrates as low as 150 kbps (or even lower).

When encoding a 30 fps video at 150 kbps, each picture uses on average 625 bytes. Consequently, a single byte of extra overhead increases the bitrate by 0.16%.

# Overhead of using multiple NAL units

In all encapsulation formats, file formats and transport protocols there is a certain amount of signalling overhead associated with each NAL unit.

- In Annex B byte streams there is a startcode overhead of typically 3 or 4 bytes.

- In the file format for Carriage of NAL unit structured video in the ISO Base Media File Format, a sample length field of 4 bytes is typically used [3].

- The RTP payload format offers the ability to aggregate multiple NAL units in the same RTP packet wherein each NAL unit is preceded by a 2 byte length field. However, the aggregation of NAL units can only be used if all aggregated NAL units completely fit into a single RTP packet. If the aggregation does not fit completely in a single RTP packet, the NAL units have to be sent in separate RTP packets which is typically associated with an overhead of at least 40 bytes (20 bytes IP-header, 8 bytes UDP-header, and 12 bytes RTP header) [4].

In addition to this overhead, there is the overhead of the NAL unit header, which in VVC is 2 bytes.

# Discussion at the 17th JVET meeting

The topic was discussed at the 17th JVET meeting and the following notes were captured:

A suggested minimum approach:

- There was discussion of limiting the combined slice header to a special case for when there is only one slice per picture without subpicture support.

  - There could be an SPS flag indicating that the whole CLVS always has one slice per picture. This might or might not be necessary

- At a minimum, a flag would need to be added to the header of every slice (regardless of whether is being used or not) that would indicate whether the PH data are in the SH or not.

  - There should be a constraint that this flag must be 0 for the entire CLVS or 1 for the entire CLVS.

  - The flag would be required to be 0 unless there is only one slice per picture and no subpicutures.

- The POC LSBs would be in the PH, regardless of wether that is in a separate NAL unit or not.

- The presence of the four flags that control whether something is in the PH or SH could be conditioned on the that added flag. These flags could be grouped together.

Revisit after side activity to draft proposed text.

The following can be observed:

- Adding an SPS flag is not necessary and has not been included in the proposed draft text

- Moving POC LSB to PH is orthogonal to this proposal and has not been included in the proposed draft text except for removing slice_pic_order_cnt_lsb from the slice header

- Subsequent to the above discussion it was decided to move the four flags that control whether something is in PH or SH to the PPS so this aspect is no longer relevant and has not been included in the proposed draft text

# Proposal

The complete text for the proposal is attached with this contribution. It consists of three basic elements included below:

- Modification to PU and AU boundary detection.

- Updates to picture header and slice header syntax, including the new flag picture_header_in_slice_header_flag.

- Semantics for the picture_header_in_slice_header_flag and related bitstream constraints.

## Modification to PU and AU boundary detection.

This clause specifies the order of NAL units and coded pictures and their association to PUs and AUs for CVSs that conform to one or more of the profiles specified in Annex A and that are decoded using the decoding process specified in clauses 2 through 9.

A PU consists of zero or one PH NAL unit, one coded picture, which comprises of one or more VCL NAL units, and zero or more non-VCL NAL units. The association of VCL NAL units to coded pictures is described in clause 7.4.2.4.4.

An AU consists of zero or one AU delimiter NAL unit and one or more PUs in increasing order of nuh_layer_id.

The first AU in the bitstream starts with the first NAL unit of the bitstream. There shall be at most one AU delimiter NAL unit in an AU.

It is a requirement of bitstream conformance that when a picture consists of more than one VCL NAL unit, a PH NAL unit shall be present in the PU.

If a PH NAL unit is present, the first VCL NAL unit of a picture is the first VCL NAL unit that follows the PH NAL unit in decoding order of the picture. Otherwise (no PH NAL unit is present), the first VCL NAL unit of a picture is the only VCL NAL unit of the picture.

A VCL NAL unit is the first VCL NAL unit of an AU (and consequently the picture containing the first VCL NAL unit is the first picture of the AU) when the VCL NAL unit is the first VCL NAL unit of a picture and one or more the following conditions are true:

–	The value of nuh_layer_id of the VCL NAL unit is less than the nuh_layer_id of the previous picture in decoding order.

–	The value of slice_pic_order_cnt_lsb of the VCL NAL unit differs from the PicOrderCntVal of the previous picture in decoding order.

–	PicOrderCntVal derived for the VCL NAL unit differs from the PicOrderCntVal of the previous picture in decoding order.

## Updates to picture header and slice header including the new flag picture_header_in_slice_header_flag.

#### Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| picture_header_structure( ) |  |
| } |  |

#### Picture header RBSPstructure syntax

| picture_header_rbspstructure( ) { | Descriptor |
| --- | --- |
| non_reference_picture_flag | u(1) |
| gdr_pic_flag | u(1) |
| no_output_of_prior_pics_flag | u(1) |
| if( gdr_pic_flag ) |  |
| recovery_poc_cnt | ue(v) |
| ph_pic_parameter_set_id | ue(v) |
| … |  |
| } |  |

#### General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| picture_header_in_slice_header_flag | u(1) |
| if(picture_header_in_slice_header_flag) |  |
| picture_header_structure( ) |  |
| slice_pic_order_cnt_lsb | u(v) |
| if( subpics_present_flag ) |  |
| slice_subpic_id | u(v) |
| if( rect_slice_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && NumTilesInPic > 1 ) |  |
| num_tiles_in_slice_minus1 | ue(v) |
| slice_type | ue(v) |
| if( !pic_rpl_present_flag &&( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && !pps_ref_pic_list_sps_idc[ i ] && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| slice_rpl_sps_flag[ i ] | u(1) |
| … |  |
| } |  |

## Semantics for the picture_header_in_slice_header_flag and related bitstream constraints.

picture_header_in_slice_header_flag equal to 1 specifies that the picture header syntax structure is present in the slice header. picture_header_in_slice_header_flag equal to 0 specifies that the picture header syntax structure is not present in the slice header.

It is a requirement of bitstream conformance that the value of picture_header_in_slice_header_flag is the same in all slices of a CLVS.

When picture_header_in_slice_header_flag is equal to 1, it is a requirement of bitstream conformance that no NAL unit with NAL unit type equal to PH_NUT is present in the CLVS.

When picture_header_in_slice_header_flag is equal to 0, it is a requirement of bitstream conformance that a NAL unit with NAL unit type equal to PH_NUT is present in the PU, preceding the first VCL NAL unit of the PU.

# References

[1] HLS Authoring Specification for Apple Devices - 
https://developer.apple.com/documentation/http_live_streaming/hls_authoring_specification_for_apple_devices

[2] Vimeo Engineering Blog - https://medium.com/vimeo-engineering-blog/a-story-of-lowering-quality-for-improving-accessibility-121dcd3871cc

[3] ISO/IEC 14496-15 Carriage of NAL unit structured video in the ISO Base Media File Format

[4] IETF RFC 7798 RTP Payload Format for High Efficiency Video Coding (HEVC)

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).