| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0780-v2 |
| --- | --- |

| Title: | BoG Report on CE5 Related Contributions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrew Segall Ching-Yeh Chen | Tel: Email: | chingyeh.chen@mediatek.com asegall@sharplabs.com |
| Source: | Chairs |  |  |

_____________________________

# Abstract

This contribution provides DRAFT notes of the BoG on CE5 related contributions.  The BoG met during the following time:

January 10th from 8:00-10:30AM

January 10th from 5:00-6:45PM

January 10th from 8:00-10:00PM

The mandate of the BoG was to review all CE5 related contributions.

Recommendations were determined during the BoG meetings.  However, it was largely agreed that most of the recommendations should be revisited by the BoG subject to JVET decisions on CE5.  Thus, the BoG has not yet closed and would plan to reconvene as needed.

The BoG met again during the following times:

January 11th from 7:00-9:30PM

January 12th from 6:30-7:50PM

The mandate of the BoG was to re-review all CE5 related contributions given the decision use CE5-2.1 as a base for the CC-ALF design.

The BoG provided the following recommendations:

- CC-ALF design

  - Incorporate DC neutrality constraint from JVET-Q0165 into the CC-ALF design

  - Incorporate aspect 2 (Disabling CC-ALF when luma ALF is disabled) from JVET-Q0190-v1.docx into the CC-ALF design.

  - (Bugfix) Align boundary handling of CC-ALF specification text with current ALF boundary handing.

  - Incorporate the second and third aspects of JVET-Q0382 into the CC-ALF design

    - Second aspect: Do not signal CC-ALF syntax if chroma arrays are unavailable

    - Third aspect: Remove signaling for the number of filters in the slice and picture headers

  - Incorporate JVET-Q0782 into the CC-ALF design.

  - Syntax elements related to CC-ALF should not be signalled if ChromaArrayType is equal to 0.

- Track to discuss if CC-ALF should be controlled by flag in SPS.  (Related proposals include JVET-Q0253 and JVET-Q0520.)

- Review JVET-Q0494 in the context of ALF

- Formal study of JVET-Q0125 in a CE

The BoG also created integrated text that included the recommendations.  The text is available in JVET-Q0795.

# Notes

Notes from the BoG discussion are captured below.

## CE5 related – Cross-component adaptive loop filtering (24)

Contributions in this category were discussed Friday 10 Jan. 800–1030,  17:00-18:45 and 20:00-22:00 in BoG (chaired by Andrew Segall).  Some of the documents were revisited, which is also captured in the notes.

JVET-Q0074 CE5-related: Performance of CE5 anchor with 8-bit dynamic range for CC-ALF coefficients [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)]

No need to present.

JVET-Q0124 CE5-related: JC-CCALF with Power of 2 Weight Values [C.-W. Kuo (Panasonic)]

This document proposes to use power of 2 values for Joint Chroma Cross Component Adaptive Loop Filter (JC-CCALF) weight candidates to avoid multiplication or division in the Cr refinement value derivation. In method 1, the weight candidates are restricted to ±1. In method 2, the weight candidates are extended to 16 weights: ±1/8, ±1/4, ±1/2, ±1, ±2, ±4, ±8, and ±16. Compared with the JC-CCALF in CE5-3.2, negligible coding difference and runtime change are observed.

Simulation results with joint YUV performance (AHG13 8:1:1 metric) are summarized as follows as compared to the VTM anchor.

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV | Y | U | V | YUV |
| CCALF (CE5 anchor) | 0.17% | -8.31% | -7.06% | -1.40% | 0.14% | -10.14% | -9.82% | -1.88% | 0.12% | -13.00% | -9.48% | -2.15% |
| JC-CCALF (CE5-3.2) | 0.10% | -5.84% | -4.87% | -0.99% | 0.16% | -8.63% | -8.27% | -1.56% | 0.12% | -12.71% | -9.47% | -2.12% |
| weights = ±1 | 0.10% | -6.15% | -5.02% | -1.04% | 0.15% | -9.05% | -8.73% | -1.66% | 0.12% | -12.86% | -9.89% | -2.18% |
| weights = ±1/8 to ±16 | 0.10% | -6.06% | -5.09% | -1.04% | 0.14% | -8.91% | -8.56% | -1.64% | 0.12% | -12.88% | -9.83% | -2.17% |

The proponent asserted that the proposal is a simplification of the method studied in CE5-3.2.  Additionally, it was observed that there is a small improvement in the performance relative to the proposal studied in CE5-3.2.

One participant commented that the proposal looks interesting and simplification appears clear.

It was later discussed during the presentation of JVET-Q0125, that JVET-Q0125 might be a more preferable way forward.

No action.

JVET-Q0653 Cross-check of JVET-Q0124 (CE5-related: JC-CCALF with Power of 2 Weight Values) [Y.-W. Chen (Kwai Inc.)] [late]

Cross-check was not presented.

JVET-Q0125 CE5-related: JC-CCALF with Alternative Filter Weight [C.-W. Kuo (Panasonic)]

This contribution proposes to use individual JC-CCALF filter weights for each alternative filter. Specifically, instead of using various weight values as in JC-CCALF, only sign changes are applied for 4 alternative filters, e.g., +--+. In the decoder, the current CTU selects the filter weight according to the parsed alternative filter index. Compared to the JC-CCALF in CE5-3.2, coding gain improvement and negligible runtime change are observed.

Simulation results with joint YUV performance (AHG13 8:1:1 metric) are summarized as follows:

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV | Y | U | V | YUV |
| CCALF (CE5 anchor) | 0.17% | -8.31% | -7.06% | -1.40% | 0.14% | -10.14% | -9.82% | -1.88% | 0.12% | -13.00% | -9.48% | -2.15% |
| JC-CCALF (CE5-3.2) | 0.10% | -5.84% | -4.87% | -0.99% | 0.16% | -8.63% | -8.27% | -1.56% | 0.12% | -12.71% | -9.47% | -2.12% |
| alternative filter weight | 0.10% | -6.65% | -5.56% | -1.14% | 0.15% | -9.39% | -9.16% | -1.74% | 0.11% | -13.03% | -10.11% | -2.22% |

The proponent asserted that the proposal provides coding efficiency improvement of the method studied in CE5-3.2.  Using the YUV metric, the gain appears to be 0.1% relative to the method 1 in JVET-Q0125. (The JVET-Q0125 method proposes to share a single +/-1 for all filters.)

It was commented that the filter weights are signaled in the slice header, as is done for JVET-CE5-3.2.  It was commented that this could be included in the APS.

It was commented by multiple participants that the method in JVET-Q0125 would be more interesting than JVET-Q0124 as a way forward.

It was commented that the method requires additional searching at the encoder for the weights of the different filters.

Reconsider proposal if CE5-3.2 is adopted.

On January 11th, the document was further discussed.  Notes for the discussion follow:

The proponent reported that the proposal would have a coding performance of 0.3%, 0.18%, -0.07 for AI, RA and LDB relative to C.B.

One participant asked if subjective viewing had been performed.  No subjective viewing was reported.

It was noted that CE5-3.2 was not adopted.

The proponents also reported on a study of the YUV metric used in AHG13.  This metric has also been used in the study of CC-ALF.  The proponents reported that they had computing a fitting operation using the assumption that the YUV metric should be the same before and after a bit-shifting operation.  The R-D data for JVET-P0080 and JVET-O0636 was used in the training.  In this exercise, it was reported that a weighting that achieved that would be 15:1:1, 16:1:1 and 35:1:1 for AI, RA and LDB, respectively, for JVET-P0080.  Similarly, a weighting that achived that would be 17:1:1, 17:1:1 and 35:1:1 for AI, RA, and LDB, respectively, for JVET-O0636.

It was commented that while the AHG13 YUV metric had been used in the summary of the CC-ALF proposals, it was well understood that the metric often over-estimated the gains of a tool.  And, that using a weighting of the PSNR values might be a better predictor.  (Please see notes from the discussion of JVET-Q0025 in the Track.)

It was commented that if the gains were shifted from luma to chroma, the gains may be in the range of 50-70% of the AHG13 YUV metric.

Proponents were encouraged to provide results using the combination of their method with the DC neutrality approach in JVET-Q0165.

On January 12th, the document was further discussed.  Notes for the discussion follow:

Results were shown comparing JVET-Q0125 versus the CE5 Common Base as well as comparing JVET-Q0125+JVET-Q0165 versus the CE5 Common Base.  The proponent explained that six R-D points were still running for the JVET-Q0125+JVET-Q0165 scenario.  The results from JVET-Q0125 were used for these points.

It was commented that chroma loses of up to 6% were observed for some sequences with the proposed method compared to the CE5 Common Base.

The proponent commented that in the cases where there was a 6% chroma loss, there was also a 0.04% luma gain.

Recommendation: Formal study of the method in a CE.

JVET-Q0585 CE5-related: cross-check report of JVET-Q0125 on JC-CCALF with Alternative Filter Weight [E. François (InterDigital)] [late]

The cross-checker reported that the method looks interesting and provides both a coding efficiency improvement and simplification compared the method studied in CE5-3.2.

JVET-Q0165 CE5-related: On the CC-ALF filtering process [Z. Zhang, J. Ström, K. Andersson (Ericsson)]

The proponent asserts that a common feature to all tests in CE5 is that the filters in CC-ALF are not limited to be DC neutral, which means that it is not guaranteed that chroma would be unchanged if the luma component is flat. The main aspect of this contribution is to address that issue. A common feature of CE5-1.1 and CE5-2.2 is to remove the multiplications of CC-ALF filtering process. This contribution presents results of a DC neutral design on top of CE5 anchor, CE5-2.1 and CE5-2.2.

Compared to the CE5-CC-ALF anchor, the modified version of CE5-2.1 has a Y/U/V BD-rate impact of
 -0.02%/-0.02%/-0.19% (AI), 0.00%/-0.28%/-0.31% (RA) and 0.03%/-0.51%/-0.63% (LDB).

Compared to CE5-2.1, the modified version of CE5-2.1 has a Y/U/V BD-rate impact of 
0.00%/-0.55%/-0.54% (AI), -0.02%/-0.36%/-0.37% (RA) and 0.03%/-0.27%/-0.50% (LDB).

Compared to the CE5-CC-ALF anchor, the modified version of CE5-2.2 has a Y/U/V BD-rate impact of
-0.03%	/0.15%/-0.06% (AI), -0.01%/-0.13%/-0.13% (RA) and 0.00%/-0.55%/-0.63% (LDB).

Compared to CE5-2.2, the modified version of CE5-2.2 has a Y/U/V BD-rate impact of
0.00%	/-0.58%/-0.65% (AI), -0.01%/-0.47%/-0.57% (RA) and -0.01%/-0.47%/-0.53% (LDB).

It was commented that there are two changes in the proposal.  The first is to reduce the number of signaled coefficients from 8 to 7.  And, the second change is to limit the residual correction to be a zero-sum filter (e.g., DC neutral).

It was commented that the method requires the calculation of the difference between samples, which increases the bit-depth of the multiplication.

It was asked if there would be a change in performance if the bit-depth of the weight C_sum was limited to the bit-depth of the signaled coefficients.  The proponent asserted that the impact is expected to be small, but the results were unknown.

It was commented that the approach reduces the number of multiplies (or shifts) by one, but it requires an additional seven adds/subtracts to calculate the residual values.  As a result, the proposed method may have higher complexity than the methods studied in CE5. It was further commented that it may be possible to reuse the residual calculation in ALF so that the additional 7 adds/subtracts would not be required.

It was commented that a similar method is proposed in JVET-Q0190 and JVET-Q0304  (See related discussion in the notes of those proposals.)

On January 11th, the document was further discussed.  Notes for the discussion follow:

The YUV metric for the method was reported to be -0.11%, -0.09% and -0.05% for AI, RA and LDB, respectively, when compared to CE5-2.1

The complexity of the metric would remove one shift and require an additional 7 subtractions. In addition, the bit-depth of the multiplication would be increased.

It was asserted by the proponent that these subtractions could potentially be reused from the ALF calculation.

Multiple participants expressed support for the approach.

One participant raised concerns if the gains of the approach justified the complexity.

One participant expressed that the benefit of the proposal was for functionality.

The proponent reported that the software for JVET-Q0165 would be provided in a revision of the contribution.

Recommendation: Incorporate DC neutrality constraint from JVET-Q0165 into the CC-ALF design

JVET-Q0544 Crosscheck of JVET-Q0165 Test 1 (CE5-related: On the CC-ALF filtering process) [O. Chubach (MediaTek)] [late]

No issues were reported by the cross-checker.

JVET-Q0607 Crosscheck of JVET-Q0165 (CE5-related: On the CC-ALF filtering process) [N. Hu (Qualcomm)] [late]

No issues were reported by the cross-checker.  Furthermore, it was reported that the encoder was observed to be very straightforward.

JVET-Q0167 Non-CE5: Multiplication simplification for ALF and CC-ALF [J. Ström, Z. Zhang, K. Andersson (Ericsson)]

Document to be re-categorized as an ALF related document.

JVET-Q0608 Crosscheck of JVET-Q0167 (Non-CE5: Multiplication simplification for ALF and CC-ALF) [N. Hu (Qualcomm)] [late]

Document to be re-categorized as an ALF related document.

JVET-Q0190 CE5-related: On CC-ALF modifications related to coefficients and signalling [O. Chubach, C.-Y. Chen, C.-Y. Lai, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)]

This contribution considers two aspects related to CC-ALF. Aspect 1 limits the sum of the coefficients of CC-ALF filters to be equal to 0. This would guarantee that CC-ALF has a zero response at DC frequency, which results in CC-ALF behaving as a high pass, band pass, or other zero-DC filter. Aspect 2 proposes to update the syntax of CC-ALF such that it is aligned to the ALF process: disabling CC-ALF when luma ALF is disabled.

The combined results of Aspect 1 and Aspect 2 are as follows (over the CC-ALF common base, Y/Cb/Cr):
In AI: -0.01% / -0.06% / -0.11%
In RA: 0.00% / -0.10% / -0.06%
In LB: -0.01% / -0.22% / -0.05%

The results of the two aspects were reported separately.  For the first aspect, the YUV improvement for AI, RA and LDB was 0.00%, 0.00% and 0.00% respectively.

For the second aspect, the YUV improvement for AI, RA and LDB was reported to be -0.02%, -0.02%, and -0.04%, respectively.

For the combined result, the YUV improvement for AI, RA and LDB was reported to be -0.02%, -0.02%, and -0.04%, respectively.

It was commented that the presented slides do not match the uploaded slides, which resulted in some confusion among participants.  The proponent was requested to upload a revision of the slides.

It was commented that the implementation includes a conformance constraint that the inferred coefficient is required to be in the range of the signaled coefficients.

It was asked for Aspect 2 if enabling CC-ALF should be conditioned on if chroma ALF was enabled, as opposed to the proposal to condition enabling CC-ALF on if luma ALF is enabled.

One participant suggested that there had not been study on if an encoder might want to use CC-ALF when luma ALF was disabled.

One participant commented that the Aspect 2 seems well motivated.

Another participant commented that there is a loss for Aspect 2 for Class F for LDB.

It was commented that one aspect of JVET-Q0304 is related to the first aspect of this proposal.  (See related notes in the discussion of JVET-Q0304.)

It was commented that JVET-Q0253 and JVET-Q0382 may be related to the second aspect of this proposal.

In further discussion it was suggested to recommend that the conditional signaling of CC-ALF on luma ALF be incorporated to a future design.

Recommendation: Incorporate Aspect 2 (Disabling CC-ALF when luma ALF is disabled) from JVET-Q0190-v1.docx into the CC-ALF design.

JVET-Q0648 Cross-check of JVET-Q0190 (CE5-related: On CC-ALF modifications related to coefficients and signaling) [T.-C. Ma (Kwai Inc.)] [late]

The cross-checker expressed support for the proposal and asserted that it was consistent with how chroma ALF is enabled.

JVET-Q0251 CE5-related: Unified cross component adaptive loop filter [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)]

In JVET-P1008, a cross component adaptive loop filter (CC-ALF) was tested that refines chroma components by using luma samples. To calculate an offset for a chroma sample, the filter is applied to luma samples (after applying luma sample adaptive offset) where the center of the filter is the co-located luma sample of the current chroma sample. In an APS, a Cb (Cr resp.) CC-ALF filter set with up to 4 filters could be signalled and a filter from this Cb (Cr resp.) filter set could be applied to a Cb (Cr resp.) coding tree block. On the other hand, in an APS, a chroma adaptive loop filter (ALF) filter set with up to 8 filters could be signaled, and a filter from this chroma filter set could be applied to a chroma coding tree block. In this contribution, CC-ALF filters are proposed to be combined for the two chroma components, where Cb and Cr components share the same filters in an APS.  It is asserted that this design would be more similar to the approach used in a regular chroma ALF.

Under common test conditions, compared with CE5 anchor, the average BD rate of the proposed method was reported to be:

AI   : -0.01%(Y), 0.05%(U), -0.17%(V)

RA  : 0.02%(Y), -0.04%(U), -0.06%(V)

LDB: 0.xx%(Y), 0.xx%(U), 0.xx%(V)

LDP: 0.04%(Y), -0.12%(U), -0.40%(V)

In summary, the document proposes to allow the signaling of eight filters in the APS and then selecting any of these eight filters for the Cb and Cr component.  It is asserted that this unifies the design with the current chroma ALF design, as well as providing some additional flexibility.

One participant suggested that this may be a good direction.

It was commented that the approach would not be applicable if the joint chroma CC-ALF solution would be adopted.

On January 11th, the document was further discussed.  Notes for the discussion follow:

It was commented that the results use a generalized multiplier approach.

The proponents were encouraged to generate results using the multiplier free method.

The proponents were also encouraged to show a benefit of the method for an encoder implementation.

No action.

JVET-Q0736 Cross-check result of JVET-Q0251: CE5-related: Unified cross component adaptive loop filter [Z. Zhang (Ericsson)] [late]

The cross-checker reported that the encoder was different than the common base.  The cross-checker reported that they investigated this in more detail and concluded that the impact of the difference was very minor.

JVET-Q0253 CE5-related: High level syntax modifications for CCALF [A. M. Kotra, S. Esenlik, B. Wang, H. Gao, E. Alshina (Huawei)]

The contribution proposes to decouple the high level syntax control of Adaptive loop filter (ALF) and Cross component adaptive loop filter (CCALF) completely by introducing a new sequence level syntax element called “sps_ccalf_enabled_flag” and the corresponding new entries for CCALF in the picture header and slice header.  This decoupling of high level syntax control of ALF and CCALF reportedly provides encoders with better flexibility while encoding sequences with different signal characteristics.

It was commented that results were not provided.

One participant questioned if a SPS level flag is needed. A flag in the picture header may be sufficient.

One participant commented that CC-ALF should be treated as a sub-set of the ALF process, and so enabling CC-ALF should be dependent on if ALF is enabled.

One participant commented that the benefit would be to save 2-bits in the picture header.

It was commented that one aspect of JVET-Q0382 is the same as this proposal.

It was commented by one participant that SPS level control may not be needed.

Note: Please see the later discussion related to the SPS level control in JVET-Q0520.

On January 11th, the document was further discussed.  Notes for the discussion follow:

The proposal was incorporated into JVET-Q0782.  Please see the notes for JVET-Q0782.

JVET-Q0296 Non-CE5: Adaptive precision for CCALF coefficients [L.-H. Xu, J. Yao, J.-Q. Zhu, K. Kazui (Fujitsu)]

This contribution proposes a method to increase the precision of CCALF coefficients without computational complexity or coefficients length increment. It is proposed to use an adaptive bit-shift instead of a constant bit-shift to fix the floating filter coefficients when encoding. Simulation results show 0.01%(Y), -0.29%(U), -0.24%(V) BD-rates change for AI and 0.01%(Y), -0.40%(U), -0.41(V) BD-rates change for RA.

The proposal signals a bit-shift for each filter in the bit-stream.  The coefficient bit-depth is maintained at 6 bits.  The signaled bit-shift values were in the range of 5 to 12.

It was further reported that using the range of 7 to 11 had similar performance.

It was noted that the CE5 Common Base used a fixed bit-shift of 7.

A question was raised on how the encoder would determine the adaptive shift value.  It was explained by the proponent that the encoder determined the maximum value of the coefficients and then selected the value of the shift based on this maximum value.

One participant asked why the dynamic range was chosen.

The proponent commented that the approach could be applied to most methods of signaling the filter that used multiplication (as opposed to methods that use power of two).

It was commented that the approach may create an issue when creating a DC neutral filter.

It was commented that the approach may require more bits for the multiplication.

Revisit after decision regarding CCALF status.

On January 11th, the document was further discussed.  Notes for the discussion follow:

No action due to later decision to use a multiplication free design.

JVET-Q0301 CE5-related: CCALF filter for 4:2:2 and 4:4:4 color format [J. Choi, J. Nam, J. Heo, J. Lim, S. Kim (LGE)]

This contribution proposes to use a simplified cross-component ALF (CCALF) filter shape for 4:2:2 and 4:4:4 color format. Considering the different chroma sampling structure of 4:2:0, 4:2:2 and 4:4:4, the CCALF filter shape is reduced from a 3x4 diamond (non-square 8-tap) to a 3x3 diamond (square 5-tap) in 4:2:2 and 4:4:4 color format.  It is asserted that the  symmetrical luma samples is better suited for the chroma refinement for these color formats and that the number of CCALF operations could then be reduced by 38% without any syntax change of the CCALF common base.

Experimental results for the proposed method with 4:4:4 test sequences were reported to provide 0.08%, -1.33%, -1.95% BD-rate changes on Y, Cb, and Cr components in AI, 0.10%, -1.44%, -2.32% in RA, and -0.04%, -2.49%, -3.64% in LDB configuration, compared to the VTM7.0 anchor.

In addition, compared to the common base of CC-ALF, the proposal provided -0.01%, 0.15%, 0.16% BD-rate changes in AI, -0.02%, 0.17%, 0.22% in RA, and 0.02%, 0.36%, 0.26% in the LDB configuration

It was commented that the proposal did not change the syntax for signaling the coefficients.  This means that 8 filter coefficients would still be transmitted for the 4:2:2 and 4:4:4 case, but only 5 of the filter coefficients would be used.

It was commented that the results show that the loss is higher for camera captured content.

It was also commented that the results presented do not follow the CTC.

It was commented that JVET-Q0467 is also related to this contribution.

It was commented that using a different filter shape for different color formats may not be desireable.

It was suggested that the proponent discuss with additional hardware experts to determine if the complexity of the filter for 4:2:2 and 4:4:4 content is a concern.

Further study encouraged.

JVET-Q0660 Crosscheck of JVET-Q0301 "CE5-related: CCALF filter for 4:2:2 and 4:4:4 color format" [G. Li (Tencent)] [late]

Cross checker reported that all results matched.

JVET-Q0304 Non-CE5: Non-Linear Cross Component Adaptive Loop Filter [J. Yao, L.-H. Xu, J.-Q. Zhu, K. Kazui (Fujitsu)]

The contribution proposes a “Non-Linear Cross Component Adaptive Loop Filter (NL-CCALF)” approach that is asserted to improve the efficiency of CC-ALF more efficient by using a clipping operation to reduce the impact of neighbor sample values when they are too different with the current sample value being filtered.

Additionally, the contribution proposes to enforce a DC neutrality constraint on the CC-ALF filter.

The results reportedly show the average BDR gains for AI, RA and LDB are -0.08%, -0.11% and -0.16% respectively, when compared to the CE5 Anchor and using the AHG13 YUV metric.

The tool is enabled when signaling the APS.

It was asserted by the proponent that the clipping approach is the same as what is used in luma ALF.

It was commented that the approach is the same as what was proposed in JVET-Q0165.

It was commented that the gains for the clipping approach for luma ALF were larger than what is reported in this contribution.  It was further noted that the gain for luma ALF was on the order of 0.5%.

It was commented by multiple participants that it was unclear if the added complexity justified the gains. It was further commented that in software the approach may double the number of operations.

A participant commented that the run-time performance of the approach was reported to be faster than the anchor.  The proponent responded that the run-time information was not reliable.

Further study encouraged.

All of the proposals considering DC neutrality was then discussed further.  Notes for the discussion follow:

It was commented that the DC neutrality could be achieved in either a normative or non-normative approach.

It was then commented that the DC neutrality constraint may be more difficult to achieve non-normatively in the case that CC-ALF uses a multiplication free design.

More than one participant expressed that the benefit of the DC neutrality constraint is not clear.

It was commented that there may be an increase in complexity due to the increase in the number of additions.  However, this may not be a significant issue.

On January 11th, the document was further discussed.  Please see the notes for JVET-Q0165.

JVET-Q0310 Non-CE5: Suggested text for CC-ALF padding process with raster scan slices [Y. Wang, H. Liu, L. Zhang, K. Zhang, Y. Wang (Bytedance)]

This contribution presents suggested text changes for CC-ALF padding process with raster-scan slices. More specifically, it is proposed to align the luma sample padding process in CC-ALF to ALF for raster-scan slices. It is reported that no performance changes are observed in the common test conditions.

It was commented that the design intent of CC-ALF is to re-use the boundary handling of ALF.

It was commented that the same change was proposed in JVET-Q0073 and JVET-Q0085.

Recommendation: (Bugfix) Align boundary handling of CC-ALF specification text with current ALF boundary handing.

On January 12th, the recommendation was further discussed.  It was noted that JVET-Q0073 had been selected in Track to be included in the CC-ALF design.  It was then deemed to be an editorial decision whether to use the text from JVET-Q0073, JVET-Q0085 or JVET-Q0310 in order to align the boundary handling design with ALF.

JVET-Q0311 Non-CE5: On CC-ALF padding for ALF virtual boundaries [Y. Wang, H. Liu, L. Zhang, K. Zhang, Z. Deng, Y. Wang (Bytedance)]

In this contribution, two methods are proposed for CC-ALF padding for virtual boundaries. In the first method, filtering was disabled for chroma samples between ALF virtual boundary and CTU bottom boundary. In the second method, a different symmetric padding method was proposed to replace existing CC-ALF padding method for ALF virtual boundaries. Compared with CE5 common base, {Y, U, V} BD-rate changes of these two methods were reported to be:

Method #1: {0.00%, 0.22%, 0.19%}, {0.00%, 0.16%, 0.22%}, and {0.00%, 0.15%, 0.24%} for AI, RA, and LDB, respectively.

Method #2: {0.00%, 0.00%, 0.01%}, {0.00%, -0.01%, 0.00%}, and {0.01%, -0.07%, 0.11%} for AI, RA, and LDB, respectively.

It was commented that the boundary padding method for the CC-ALF common base is currently the same as the ALF boundary padding method for design consistency.

It was commented that the shape of the filter may affect this proposal.

It was commented that the proposal would require performing the padding process twice – one time for luma ALF and another for CC-ALF.

No action.

JVET-Q0537 Crosscheck of JVET-Q0311 (Non-CE5: On CC-ALF padding for ALF virtual boundaries) [T. Ikai (Sharp)] [late]

No issues were reported.

JVET-Q0382 CE5-related: On high level syntax of CC-ALF [F. Chen, L. Xu, L. Wang (Hikvision)]

This proposal reports the modifications on the high level syntax of cross component adaptive loop filter (CC-ALF). First, it introduces a new SPS level syntax for CC-ALF to decouple of high level syntax control of ALF and CC-ALF. Second, a bug that CC-ALF related syntax in picture header (PH) is still signalled even if two chroma arrays are unavailable is fixed. Third, the numbers of filters for CC-ALF in slice and picture header are removed.

It was reported that Aspect 2 and Aspect 3 are also proposed in JVET-Q0171, JVET-Q0250, JVET-Q0326, and JVET-Q0520.

Recommendation: Incorporate the second aspect (do not signal CC-ALF syntax if chroma arrays are unavailable) and third aspects (remove signaling for the number of filters in the slice and picture headers) into the CC-ALF design.

JVET-Q0467 CE5-related: Simplified CCALF [G. Li, X. Li, X. Xu, S. Liu (Tencent)]

A CC-ALF filter was proposed to reduce computational complexity and signaling cost. The proposed methods were implemented on top of the CE5 anchor software and simulations were conducted with common test conditions. It is reported that the proposed method 1 has 25% fewer multiplication and the following performance:

- Over VTM-7: BD rate changes are 0.16%, -7.83%, -6.75% for AI, 0.13%, -9.92%, -9.65% for RA, 0.09%，-13.09%，-9.67% for LB, 0.15%, -12.84%, -9.61% for LP.

- Over CE5 Anchor: BD rate changes are -0.01%, 0.54%, 0.37% for AI, -0.01%, 0.29%, 0.25% for RA, -0.03%， -0.08%， -0.11% for LB, 0.01%, 0.10%, -0.30% for LP.

The second method had 37.5% fewer multiplications and the following performance:

- Over VTM-7: BD rate changes are 0.14%, -7.37%, -6.32% for AI, 0.14%, -9.59%, -9.13% for RA, 0.12%，-12.31%，-8.88% for LB, 0.13%, -12.28%, -8.76% for LP.

- Over CE5 Anchor: BD rate changes are -0.03%, 1.07%, 0.87% for AI, 0.00%, 0.70%, 0.83% for RA, 0.00%， 0.88%， 0.82% for LB, -0.01%, 0.77%, 0.73% for LP.

It was asserted by the proponent that “method 1” may also provide benefit for 4:2:2 and 4:4:4 content. However, results are not yet available.  The proponent reported that they could be provided.

On January 11th, the document was further discussed.  Notes for the discussion follow:

It was reported that the results for “method 1” would be uploaded in a revision of the contribution.  Presentation of the results during the BoG showed a chroma gains ranging from 0.3% to 0.46% compared to the CE5-1.3.

The proponent was encouraged to provide results using the multiplier free design of CE5-2.1 and, optionally, the DC neutrality constraint.

Further study encouraged.

JVET-Q0624 Crosscheck of JVET-Q0467 (CE5-related: Simplified CCALF) [J. Choi (LGE)] [late]

No issues were reported.

JVET-Q0494 CE5-related: Joint clip operation for CCALF and chroma ALF [T.-C. Ma, X. Xiu, Y.-W. Chen, H.-J. Jhu, X. Wang (Kwai Inc.)]

According to Core Experiment (CE5) [1] common base software, when both chroma ALF and CC-ALF are enabled, clipping operations are performed on modified chroma samples separately in each of the two modules to prevent overflow or underflow. In this contribution, it is proposed to perform only one range-clipping operation after chroma samples processed by ALF and CC-ALF. The proposed change is implemented based on CE5 [1] software. Simulation results show that the proposed change results in a BD-rate change (YUV) of 0.00%, 0.00%, 0.00% for AI and 0.00%, 0.00%, 0.00% for RA, respectively.

It was commented that the benefit of the clipping may be architecture dependent.

It was commented that clipping the output of chroma ALF reduces the storage requirement if an architecture needs to buffer the result.

It was requested that the proponent compute the dynamic range of the output of chroma ALF.

On January 11th, the document was further discussed.  Notes for the discussion follow:

It was reported by the proponent that when the chroma bit-depth is 10 bits, the output of chroma ALF will be a maximum of 15-bits. One participant expressed a concern that this could result in the need to store an additional 5 bits per sample if the clipping was removed.

Recommendation: Review document in the context of ALF

JVET-Q0531 Crosscheck of JVET-Q0494 (CE5-related: Joint clip operation for CCALF and chroma ALF) [O. Chubach (MediaTek)] [late]

No issues were identified.

JVET-Q0673 Crosscheck of JVET-Q0494 (CE5-related: Joint clip operation for CCALF and chroma ALF) [J. Heo (LGE)] [late]

No issues were identified.

JVET-Q0171 AHG9: On CC-ALF modifications related to HLS [O. Chubach, C.-Y. Chen, C.-Y. Lai, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)]

See notes for JVET-Q0382.

JVET-Q0250 CE5-related: Removing number of filters for CC-ALF in slice and picture header [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)]

See notes for JVET-Q0382.

JVET-Q0326 CE5-related: On CC-ALF slice and picture header syntax [X. W. Meng (PKU), X. Zheng (DJI), S. S. Wang, S. W. Ma (PKU)]

See notes for JVET-Q0382.

JVET-Q0580 Crosscheck report of JVET-Q0326 [X. Xu (Tencent)] [late]

No issues were identified.

JVET-Q0520 AHG9: Cleanups on signaling for CC-ALF, BDPCM, ACT and Palette [Y. Wang, L. Zhang, K. Zhang, W. Zhu (Bytedance)]

Note: Items 1 and 2 of this contribution belong to this category.

In the current VVC and the suggested specification for cross-component adaptive loop filter (CC-ALF), certain coding tools, including CC-ALF, chroma BDPCM, ACT, and Palette modes are only allowed under certain conditions. More specifically, CC-ALF should be enabled for non-4:0:0 colour format, chroma BDPCM, ACT and Palette mode are only enabled for 4:4:4 colour format without separate plane coding. However, the corresponding syntax elements and semantics are not well defined to reflect these restrictions. In this contribution, two aspects are proposed to cleanup signalling for CC-ALF, and a third aspect is proposed to cleanup signalling for BDPCM, ACT, and Palette.

Aspect #1: Syntax elements related to CC-ALF are not signalled if ChromaArrayType is equal to 0.

Aspect #2: ALF and CC-ALF are controlled by two separate flags in SPS.

Aspect #3: BDPCM, ACT, and Palette mode for chroma are signalled under the condition of “ChromaArrayType is equal to 3”

For Aspect #1: See the discussion above.

Recommendation: Syntax elements related to CC-ALF should not be signalled if ChromaArrayType is equal to 0.

Recommendation: Track and/or Plenary discuss if CC-ALF should be controlled by flag in SPS.  (Related proposals include JVET-Q0253 and JVET-Q0520.)

On January 11th, the document was further discussed.  Notes for the discussion follow:

During discussion in Track, it was decided that CC-ALF should be controlled by a flag in the SPS.

The proponents reported that JVET-Q0520 was included in JVET-Q0782.  See notes for JVET-Q0782 for further discussion.

JVET-Q0559 CE5-related: CCALF coefficient derivation using combined neighbors [K. Panusopone, S. Hong, L. Wang, J. Lainema (Nokia)] [late]

The document proposes two methods for modifying the filtering process in CC-ALF.  The first method averages the two samples at the left of the kernel and averages the two samples at the right of the kernel.  This results in a filter with 6 unique coefficients.

The second method extends the filter shape to be 5x4.  The second method uses a similar approach as the first method.  Specifically, for each column that is not in the middle column, the two sample values are averaged before filtering.  This results in a filtering operation with 8 filter values.

Simulation results show the first method provides a BD-rate of approximately -0.02%, 0.24%, 0.05% for AI, -0.01%, 0.02%, 0.01% for RA, and 0.00%, 0.02%, -0.19% for LB, respectively, compared to CE5 common base. For the second method, simulation results are reported to provide a BD-rate of approximately 0.01%, -0.42%, -0.89% for AI, 0.00%, -0.67%, -0.93% for RA, and -0.01%, -1.31%, -1.53% for LB, respectively, compared to CE5 common base.

It was commented that a filter that has built in symmetries should be tested with different chroma types.

The presenter reported that they were running simulations with the HDR CTC and would share when they are finished.

It was commented that the method studied in CE5-1.1 used the same filter support.  However, the symmetries are not the same.

It was commented that it would be desirable to test the methods on 4:4:4 content.

One participant expressed support for the second method.

The proponents were encouraged to provide results using the multiplier free design in CE5-2.1 and possibly including the DC neutrality constraint from JVET-Q0165.

Further study encouraged.

JVET-Q0782 CE5-related: High level syntax modifications for CCALF (combination of JVET-Q0253 and JVET-Q0520) [A. M. Kotra, S. Esenlik, B. Wang, H. Gao, E. Alshina (Huawei), Y. Wang, L. Zhang, K. Zhang, W. Zhu (Bytedance)] [late]

Reviewed on January 11th.

This proposal combines the approaches proposed in JVET-Q0253 and JVET-Q0520, which had been previously presented.  Please see the note for those contributions for additional information.

The combined syntax was presented.

One participant questioned the need for the proposed pic_ccalf_enabled_present_flag.

One participant commented that the enabled flags may have been moved to the PPS due to another decision at the meeting.

More than one participant expressed that the pic_ccalf_enabled_present_flag is not needed.

Changes requested for the proposal:

- Remove pic_ccalf_enabled_present_flag

- Move signaling of pic_cross_compoent_alf_*_enabled_flag so that they are conditioned on pic_alf_enabled flag (as well as the sps_ccalf_enabled_present_flag)

- In the slice header, also move signaling of CC-ALF related items to be dependent on condition for signaling ALF

Revisit: Review updated draft text

On January 12th, the document was draft text was reviewed:

The document was noted to be v2 of the draft text but had not yet been uploaded.  The proponent was requested to upload the text to the document server.

The syntax included the relocation of the present_flag from the Picture Header to the PPS to align with the adoption of JVET-Q0200 at this meeting.

It was discussed that the no_ccalf_constraint_flag could not be equal to 1 if the no_alf_constraint_flag was 0 according to the current design philosophy.  However, it was decided that the signaling should not reflect this relationship to be consistent with other syntax elements in the General Constraint Information Syntax.

It was explained that the proposal includes an alf_present_in_ph_flag to be consistent with the adoption of JVET-Q0200 at this meeting.  Thus, if the decision related to JVET-Q0200 was modified, it was intention of the group was that the in JVET-Q0782 should be modified appropriately.

Recommendation: Incorporate JVET-Q0782 into the CC-ALF design.