| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0782-v1 |
| --- | --- |

| Title: | CE5-related: High level syntax modifications for CCALF (combination of JVET-Q0253 and JVET-Q0520) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra Semih Esenlik Biao Wang Han Gao Elena Alshina Yang Wang Li Zhang Kai Zhang Weijia Zhu | Tel: Email: | anand.meher.kotra@huawei.com lizhang.idm@bytedance.com |
| Source: | Huawei Technologies Dusseldorf GmbH, Bytedance |  |  |

_____________________________

# Abstract

This contribution proposes to decouple the high level syntax control of Adaptive loop filter (ALF) and Cross component adaptive loop filter (CCALF) completely by introducing a new sequence level syntax element called “sps_ccalf_enabled_flag” and the corresponding new entries for CCALF in the picture header and slice header.  This decoupling of high level syntax control of ALF and CCALF reportedly provides encoders with better flexibility while encoding sequences with different signal characteristics.

# Introduction

In the current design of CCALF [4],  ALF and CCALF can be separately turned on or off for each slice. The syntax in the slice header is as shown below.

| if( sps_alf_enabled_flag && !pic_alf_enabled_present_flag ) { |  |
| --- | --- |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) { |  |
| slice_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < slice_num_alf_aps_ids_luma; i++ ) |  |
| slice_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| slice_alf_chroma_idc | u(2) |
| if( slice_alf_chroma_idc ) |  |
| slice_alf_aps_id_chroma | u(3) |
| } |  |
| if( ChromaArrayType != 0 ) |  |
| slice_cross_component_alf_cb_enabled_flag | u(1) |
| if( slice_cross_component_alf_cb_enabled_flag ) { |  |
| slice_cross_component_alf_cb_aps_id | u(3) |
| slice_cross_component_cb_filters_signalled_minus1 | ue(v) |
| } |  |
| if( ChromaArrayType != 0 ) |  |
| slice_cross_component_alf_cr_enabled_flag | u(1) |
| if( slice_cross_component_alf_cr_enabled_flag ) { |  |
| slice_cross_component_alf_cr_aps_id | u(3) |
| slice_cross_component_cr_filters_signalled_minus1 | ue(v) |
| } |  |

As depicted in table above, the syntax element slice_alf_enabled_flag only controls the ALF related syntax elements and not the CCALF syntax related elements. However the flag sps_alf_enabled_flag still controls the presence of CCALF related syntax elements in the picture header and slice header. The current proposal targets to completely decouple the signalling of ALF and CCALF related syntax elements and thereby allow encoders the flexibility to completely switch of CCALF at the sequence level.

# Proposed method

The proposal completely decouples the signaling of CCALF and ALF syntax elements by introducing a sequence level flag called “sps_ccalf_enabled_flag”.  Furthermore the CCALF related syntax elements in the picture header and slice header are conditionally signalled based on the “sps_ccalf_enabled_flag”. This reportedly gives flexibility for encoders to simply switch off the CCALF at the sequence level and not signal any other CCALF related syntax elements in the picture header or slice header. The next section introduces the specification text changes

# Specification text changes

The specification text changes are provided on top of the CCALF specification text provided in JVET-Q0058[4], with newly added parts highlighted in green, and deleted text marked with red strikethrough.

7.3.2.3	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …… |  |
| sps_alf_enabled_flag | u(1) |
| if(sps_alf_enabled_flag && ChromaArrayType != 0 ) |  |
| sps_ccalf_enabled_flag | u(1) |
| ……. |  |

7.3.3.2	 General constraint information syntax

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| …. |  |
| no_alf_constraint_flag | u(1) |
| if (!no_alf_constraint_flag) |  |
| no_ccalf_constraint_flag | u(1) |

7.3.2.6	Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| …….. |  |
| if( sps_alf_enabled_flag ) { |  |
| pic_alf_enabled_present_flag | u(1) |
| if( pic_alf_enabled_present_flag ) { |  |
| pic_alf_enabled_flag | u(1) |
| if( pic_alf_enabled_flag ) { |  |
| pic_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < pic_num_alf_aps_ids_luma; i++ ) |  |
| pic_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| pic_alf_chroma_idc | u(2) |
| if( pic_alf_chroma_idc ) |  |
| pic_alf_aps_id_chroma | u(3) |
| } /* end of pic_alf_enabled_flag */ |  |
| } /* end of pic_alf_enabled_present_flag */ |  |
| } /* end of sps_alf_enabled_flag */ |  |
| if( sps_ccalf_enabled_flag ) { |  |
| pic_ccalf_enabled_present_flag | u(1) |
| if( pic_ccalf_enabled_present_flag ) { |  |
| if( ChromaArrayType != 0 ) |  |
| pic_cross_component_alf_cb_enabled_flag | u(1) |
| if( pic_cross_component_alf_cb_enabled_flag ) { |  |
| pic_cross_component_alf_cb_aps_id | u(3) |
| pic_cross_component_cb_filters_signalled_minus1 | ue(v) |
| } |  |
| if( ChromaArrayType != 0 ) |  |
| pic_cross_component_alf_cr_enabled_flag | u(1) |
| if( pic_cross_component_alf_cr_enabled_flag ) { |  |
| pic_cross_component_alf_cr_aps_id | u(3) |
| pic_cross_component_cr_filters_signalled_minus1 | ue(v) |
| } /* end of pic_cross_component_alf_cr_enabled_flag */ |  |
| } /* end of pic_ccalf_enabled_present_flag */ |  |
| } /* end of sps_ccalf_enabled_flag */ |  |
| …… |  |

7.3.7.1	General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| ……. |  |
| if( sps_alf_enabled_flag && !pic_alf_enabled_present_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) { |  |
| slice_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < slice_num_alf_aps_ids_luma; i++ ) |  |
| slice_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| slice_alf_chroma_idc | u(2) |
| if( slice_alf_chroma_idc ) |  |
| slice_alf_aps_id_chroma | u(3) |
| } |  |
| if( sps_ccalf_enabled_flag && ! pic_ccalf_enabled_present_flag) { |  |
| if( ChromaArrayType != 0 ) |  |
| slice_cross_component_alf_cb_enabled_flag | u(1) |
| if( slice_cross_component_alf_cb_enabled_flag ) { |  |
| slice_cross_component_alf_cb_aps_id | u(3) |
| slice_cross_component_cb_filters_signalled_minus1 | ue(v) |
| } |  |
| if( ChromaArrayType != 0 ) |  |
| slice_cross_component_alf_cr_enabled_flag | u(1) |
| if( slice_cross_component_alf_cr_enabled_flag ) { |  |
| slice_cross_component_alf_cr_aps_id | u(3) |
| slice_cross_component_cr_filters_signalled_minus1 | ue(v) |
| } /* end of slice_cross_component_alf_cr_enabled_flag */ |  |
| } /* end of sps_ccalf_enabled_flag && ! pic_ccalf_enabled_present_flag */ |  |
| ….. |  |

The semantics of the newly introduced syntax elements are as follows:

sps_ccalf_enabled_flag equal to 0 specifies that the cross component adaptive loop filter is disabled. sps_ccalf_enabled_flag equal to 1 specifies that the cross component adaptive loop filter is enabled.

no_ccalf_constraint_flag equal to 1 specifies that sps_ccalf_enabled_flag shall be equal to 0.  no_ccalf_constraint_flag equal to 0 does not impose such a constraint.

pic_ccalf_enabled_present_flag equal to 1 specifies that pic_cross_component_alf_cb_enabled_flag, pic_cross_component_alf_cb_aps_id, pic_cross_component_alf_cb_filter_count_minus1, pic_cross_component_alf_cr_enabled_flag, pic_cross_component_alf_cr_aps_id and pic_cross_component_alf_cr_filter_count_minus1 are present in the PH(picture header). pic_alf_enabled_present_flag equal to 0 specifies that pic_cross_component_alf_cb_enabled_flag, pic_cross_component_alf_cb_aps_id, pic_cross_component_alf_cb_filter_count_minus1, pic_cross_component_alf_cr_enabled_flag, pic_cross_component_alf_cr_aps_id and pic_cross_component_alf_cr_filter_count_minus1 are not present in the PH. When pic_ccalf_enabled_present_flag is not present, it is inferred to be equal to 0.

8.8 	In-loop filter process

8.8.1 General

……

4.   When sps_alf_enabled_flag is equal to 1, the following applies:

- When sps_ccalf_enabled_flag is equal to 1, the reconstructed picture sample array (before adaptive loop filter) SL’ is set equal to the reconstructed picture sample array SL

- The adaptive loop filter process as specified in clause 8.8.5.1 is invoked with the reconstructed picture sample array SL and, when ChromaArrayType is not equal to 0, the arrays SCb and SCr as inputs, and the modified reconstructed picture sample array S′L and, when ChromaArrayType is not equal to 0, the arrays S′Cb and S′Cr after adaptive loop filer as outputs.

- The array S′L and, when ChromaArrayType is not equal to 0, the arrays S′Cb and S′Cr are assigned to the array SL and, when ChromaArrayType is not equal to 0, the arrays SCb and SCr (which represent the decoded picture), respectively.

5.   When sps_ccalf_enabled_flag is equal to 1, the following applies:

- The cross component adaptive loop filter process as specified in clause x.x.x.x  is invoked with the reconstructed picture sample array SL’ and, when ChromaArrayType is not equal to 0, the arrays SCb and SCr as inputs, and the modified reconstructed picture sample array S′L and, when ChromaArrayType is not equal to 0, the arrays S′Cb and S′Cr after adaptive loop filer as outputs.

- The array S′L and, when ChromaArrayType is not equal to 0, the arrays S′Cb and S′Cr are assigned to the array SL and, when ChromaArrayType is not equal to 0, the arrays SCb and SCr (which represent the decoded picture), respectively.

# Conclusion

ALF and CCALF high level syntax elements signalling are decoupled by introducing a new sequence level syntax element called “sps_ccalf_enabled_flag”. Furthermore the CCALF related syntax elements are conditionally signalled in the picture header and slice header based on the value of “sps_ccalf_enabled_flag”.  This reportedly provides encoders with better flexibility while encoding sequences with different signal characteristics.

It is suggested to adopt the proposed syntax changes for the CCALF tool.

# References

- Versatile Video Coding (Draft 7), Document JVET-P2001-vE, B. Bross, J. Chen, S. Liu,Y.K. Wang

- “JVET common test conditions and software reference configurations”, JVET-N1010

- Description of Core Experiment 5 (CE5): Cross Component Adaptive Loop Filtering, JVET-P2025

- CE5 Common Base: Cross Component Adaptive Loop Filter, JVET-Q0058

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).