| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0784-v5 |
| --- | --- |

| Title: | Combination of JVET-Q0106, JVET-0686 and JVET-Q0133 for LFNST Signaling, Latency Reduction and Scaling Process |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hilmi E. Egilmez, Alican Nalci, Adarsh K. Ramasubramonian, Muhammed Coban, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr., San Diego, CA, 92121, USA Tomonori Hashimoto Takeshi Chujoh Eiichi Sasaki Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, Japan | Email: | hegilmez@qti.qualcomm.com analci@qti.qualcomm.com aramasub@qti.qualcomm.com mcoban@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com hashimoto.tomonori@sharp.co.jp chujoh.takeshi@sharp.co.jp sasaki.eiichi@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Qualcomm Inc., Sharp Corporation |  |  |

_____________________________

# Abstract

This document describes the specification text changes needed for the combination of JVET-Q0106, JVET-Q0686 and JVET-Q0133 proposals which are adopted during Main track A.

# Introduction

This document proposes to combine JVET-Q0106 and JVET-Q0686 in order to remove the undesired dependency of luma transform skip flag on chroma LFNST, and reduce the latency in LFNST design by disabling chroma LFNST for the single-tree partition mode. The combined spec text uses the LFNST signaling condition as described in JVET-Q0106 and disables LFNST for chroma component in the single-tree case.

Signaling and interaction with transform skip flags: The signaling design is directly based on JVET-Q0106.

| LfnstTransformNotSkipFlag = (treeType == DUAL_TREE_CHROMA ¦ ¦ transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0) && (treeType == DUAL_TREE_LUMA ¦ ¦ (transform_skip_flag[ x0 ][ y0 ][ 1 ] = = 0 && transform_skip_flag[ x0 ][ y0 ][ 2 ] = = 0)) |  |
| --- | --- |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && LfnstTransformNotSkipFlag = = 1 transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && ( treeType != DUAL_TREE_CHROMA ¦ ¦ ( !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |

Based on this design, luma LFNST is only allowed if all Y, Cb and Cr transform skip flags are set to zeroso that it does not allow mixed DCT-2 and transform skip modes in Cb and Cr components with luma LFNST.

Removing chroma LFNST for single tree: In JVET-Q0686, the removal of chroma LFNST for single-tree can be reflected by introducing the variable applyLfnstFlag as:

- If treeType is equal to SINGLE_TREE,

applyLfnstFlag = ( lfnst_idx[ xTbY ][ yTbY ] > 0 && cIdx = = 0 ) ? 1 : 0

- Otherwise,

applyLfnstFlag = ( lfnst_idx[ xTbY ][ yTbY ] > 0 ) ? 1 : 0

where applyLfnstFlag indicates whether LFNST is applied to a compononent (cIdx) or not. Section 2 presents the complete text using this variable.

Scaling process: The removal of chroma LFNST for the single-tree partition mode still requires a conditional check as proposed in JVET-Q0133, and it becomes based on the color component information to reflect JVET-Q0686. The condition is needed because the scaling should be different for luma component coded with LFNST and chroma components coded with a different transform. The same variable applyLfnstFlag is used in derivation of the scaling process.

# Specification text for the proposed combination

In overall, the following set of highlighted changes are proposed on the VVC draft text (version JVET-P2001-vE):

#### 7.3.9.5 Coding unit syntax

…

| LfnstDcOnly = 1 |  |
| --- | --- |
| LfnstZeroOutSigCoeffFlag = 1 |  |
| MtsZeroOutSigCoeffFlag = 1 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType, chType ) |  |

| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : ( ( IntraSubPartitionsSplitType = = ISP_VER_SPLIT ) ? cbWidth / NumIntraSubPartitions : cbWidth ) |  |
| --- | --- |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : ( ( IntraSubPartitionsSplitType = = ISP_HOR_SPLIT) ? cbHeight / NumIntraSubPartitions : cbHeight ) |  |
| LfnstTransformNotSkipFlag = (treeType == DUAL_TREE_CHROMA ¦ ¦ transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0) && (treeType == DUAL_TREE_LUMA ¦ ¦ (transform_skip_flag[ x0 ][ y0 ][ 1 ] = = 0 && transform_skip_flag[ x0 ][ y0 ][ 2 ] = = 0)) |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && LfnstTransformNotSkipFlag = = 1 transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && ( treeType != DUAL_TREE_CHROMA ¦ ¦ ( !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |

### 8.7.3			Scaling process for transform coefficients

…

The (nTbW)x(nTbH) array dz is set equal to the (nTbW)x(nTbH) array TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ].

The variable applyLfnstFlag is derived as follows:

- If treeType is equal to SINGLE_TREE,

applyLfnstFlag = ( lfnst_idx[ xTbY ][ yTbY ] > 0 && cIdx = = 0 ) ? 1 : 0

- Otherwise,

applyLfnstFlag = ( lfnst_idx[ xTbY ][ yTbY ] > 0 ) ? 1 : 0

For the derivation of the scaled transform coefficients d[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1, the following applies:

- The intermediate scaling factor m[ x ][ y ] is derived as follows:

- If one or more of the following conditions are true, m[ x ][ y ] is set equal to 16:

- sps_scaling_list_enabled_flag is equal to 0.

- pic_scaling_list_present_flag is equal to 0.

- transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1.

- scaling_matrix_for_lfnst_disabled_flag is equal to 1 and applyLfnstFlaglfnst_idx[ xTbY ][ yTbY ] is not equal to 0.

- Otherwise, the following applies:

…

### 8.7.4			Transformation process for scaled transform coefficients

#### 8.7.4.1		General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

Output of this process is the (nTbW)x(nTbH) array res[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variable applyLfnstFlag is derived as follows:

- If treeType is equal to SINGLE_TREE,

applyLfnstFlag = ( lfnst_idx[ xTbY ][ yTbY ] > 0 && cIdx = = 0 ) ? 1 : 0

- Otherwise,

applyLfnstFlag = ( lfnst_idx[ xTbY ][ yTbY ] > 0 ) ? 1 : 0

When applyLfnstFlag is not equal to 0 lfnst_idx[ xTbY ][ yTbY ] is not equal to 0 and both nTbW and nTbH are greater than or equal to 4, the following applies:

- The variables predModeIntra, nLfnstOutSize, log2LfnstSize, nLfnstSize, and nonZeroSize are derived as follows:

predModeIntra = ( cIdx = = 0 )  ?  IntraPredModeY[ xTbY ][ yTbY ]  :  IntraPredModeC[ xTbY ][ yTbY ]	(1149)

nLfnstOutSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  48  :  16	(1150)

log2LfnstSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  3  :  2	(1151)

nLfnstSize = 1  <<  log2LfnstSize		(1152)

nonZeroSize = ( ( nTbW = = 4  &&  nTbH  = = 4 )  | |  ( nTbW = = 8  &&  nTbH  = = 8 ) )  ?  8  :  16	(1153)

- When intra_mip_flag[ xTbY ][ yTbY ] is equal to 1 and cIdx is equal to 0, predModeIntra is set equal to INTRA_PLANAR.

- When predModeIntra is equal to either INTRA_LT_CCLM, INTRA_L_CCLM, or INTRA_T_CCLM, predModeIntra is derived as follows:

- If intra_mip_flag[ xTbY + nTbW * SubWidthC / 2 ][ yTbY + nTbH * SubHeightC / 2 ] is equal to 1, predModeIntra is set equal to INTRA_PLANAR.

- Otherwise, if CuPredMode[ 0 ][ xTbY + nTbW * SubWidthC / 2 ][ yTbY + nTbH * SubHeightC / 2 ] is equal to MODE_IBC or MODE_PLT, predModeIntra is set equal to INTRA_DC.

- Otherwise, predModeIntra is set equal to IntraPredModeY[ xTbY + nTbW * SubWidthC / 2 ][ yTbY + nTbH * SubHeightC / 2 ].

- The wide angle intra prediction mode mapping process as specified in clause 8.4.5.2.6 is invoked with predModeIntra, nTbW, nTbH and cIdx as inputs, and the modified predModeIntra as output.

- The values of the list u[ x ] with x = 0..nonZeroSize − 1 are derived as follows:

xC = DiagScanOrder[ 2 ][ 2 ][ x ][ 0 ]		(1154)

yC = DiagScanOrder[ 2 ][ 2 ][ x ][ 1 ]		(1155)

u[ x ] = d[ xC ][ yC ] 			(1156)

- The one-dimensional low frequency non-separable transformation process as specified in clause 8.7.4.2 is invoked with the input length of the scaled transform coefficients nonZeroSize, the transform output length nTrS set equal to nLfnstOutSize, the list of scaled non-zero transform coefficients u[ x ] with x = 0..nonZeroSize − 1, and the intra prediction mode for LFNST set selection predModeIntra as inputs, and the list v[ x ] with x = 0..nLfnstOutSize − 1 as output.

- The array d[ x ][ y ] with x = 0..nLfnstSize − 1, y = 0..nLfnstSize − 1 is derived as follows:

- If predModeIntra is less than or equal to 34, the following applies:

d[ x ][ y ] =  ( y < 4 )  ?  v[ x + ( y << log2LfnstSize ) ]  : 	(1157)
	( ( x < 4 )  ?  v[ 32 + x + ( ( y − 4 ) << 2 ) ]  :  d[ x ][ y ] )

- Otherwise, the following applies:

d[ x ][ y ] = ( x < 4 )  ?  v[ y + ( x << log2LfnstSize ) ]  : 	(1158)
	( ( y < 4 )  ?  v[ 32 + y + ( ( x − 4 ) << 2 ) ]  :  d[ x ][ y ] )

The variable implicitMtsEnabled is derived as follows:

- If sps_mts_enabled_flag is equal to 1 and one of the following conditions is true, implicitMtsEnabled is set equal to 1:

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- cu_sbt_flag is equal to 1 and Max( nTbW, nTbH ) is less than or equal to 32

- sps_explicit_mts_intra_enabled_flag is equal to 0 and CuPredMode[ 0 ][ xTbY ][ yTbY ] is equal to MODE_INTRA and lfnst_idx[ x0 ][ y0 ] is equal to 0 and intra_mip_flag[ x0 ][ y0 ] is equal to 0

- Otherwise, implicitMtsEnabled is set equal to 0.

The variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived as follows:

- If one or more of the following conditions is true, trTypeHor and trTypeVer are set equal to 0.

- cIdx is greater than 0

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and lfnst_idx is not equal to 0

- Otherwise, if implicitMtsEnabled is equal to 1, the following applies:

- If cu_sbt_flag is equal to 1, trTypeHor and trTypeVer are specified in Table 38 depending on cu_sbt_horizontal_flag and cu_sbt_pos_flag.

- Otherwise (cu_sbt_flag is equal to 0), trTypeHor and trTypeVer are derived as follows:

trTypeHor = ( nTbW >= 4  &&  nTbW <= 16 ) ? 1 : 0	(1159)

trTypeVer = ( nTbH >= 4  &&  nTbH <= 16 ) ? 1 : 0	(1160)

- Otherwise, trTypeHor and trTypeVer are specified in Table 37 depending on mts_idx.

The variables nonZeroW and nonZeroH are derived as follows:

- If applyLfnstFlag lfnst_idx[ xTbY ][ yTbY ] is not equal to 0 and both nTbW and nTbH are greater than or equal to 4 is not equal to 0, the following applies:

nonZeroW = ( nTbW = = 4  | |  nTbH = = 4 )  ?  4  :  8	(1161)

nonZeroH = ( nTbW = = 4  | |  nTbH = = 4 )  ?  4  :  8	(1162)

- Otherwise, the following applies:

nonZeroW = Min( nTbW, ( trTypeHor > 0 )  ?  16  :  32 )	(1163)

nonZeroH = Min( nTbH, ( trTypeVer > 0 )  ?  16  :  32 )	(1164)

The (nTbW)x(nTbH) array r of residual samples is derived as follows:

…

# Conclusions

This document proposes to combine JVET-Q0686 with JVET-Q0106 and JVET-Q0133 to (i) reduce the latency in LFNST design, (ii) remove undesired luma-chroma dependency in LFNST signaling, and (iii) includes a conditional check for proper scaling of coefficients. It is recommended to adopt the combined specification text changes in VVC.

# References

- F. Bossen, J. Boyce, X. Li, K. Sühring, V. Seregin, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting, Geneva, CH, 19–27 Mar. 2019.

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7),” JVET-P2001, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 16th Meeting, Geneva, CH, 1–11 Oct. 2019.

- B. Heng, T. Hellman, M. Zhou, W. Wan, “AHG16: Combination of LFNST with transform skip,” JVET-Q0106, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 17th Meeting, Brussels, BE, 7–17 January 2020.

- T. Hashimoto, T. Chujoh, E. Sasaki, T. Ikai, “Fix on LFNST condition,” JVET-Q0133, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 17th Meeting, Brussels, BE, 7–17 January 2020.

- H. E. Egilmez, A. Nalci, M. Coban, V. Seregin, M. Karczewicz, “Chroma LFNST Simplification and Signaling,” JVET-Q0686, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 17th Meeting, Brussels, BE, 7–17 January 2020.

# Patent rights declarations

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).