| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0785-v1 |
| --- | --- |

| Title: | CE3-related: Modification of chroma BDPCM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz, Hongtao Wang, Alican Nalci, Muhammed Coban, 5775 Morehouse Dr., San Diego, CA, 92121, USA Takeshi Tsukuba, Masaru Ikeda, Yoichi Yagasaki, Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +1 858-658-5673 martak@qti.qualcomm.com takeshi.tsukuba@sony.com |
| Source: | Qualcomm Incorporated, Sony |  |  |

_____________________________

# Abstract

In CE3-2.4, it has been studied that chroma BDPCM is beneficial for the lossless coding of YUV420 materials. This contribution studies the performance of chroma BDPCM for the lossy coding of YUV420 test sequences. A modification of the signaling of chroma BDPCM flag is further proposed to improve the performance of chroma BDPCM.

# Introduction

In VVC Draft 7[1], chroma BDPCM is enabled for the YUV444 format but not for the YUV420 format. It has been shown in CE3-2.4[2] that enabling chroma BDPCM is beneficial for lossless coding of YUV420 content. This contribution studies the performance impact of enabling chroma BDPCM for lossy coding of YUV420. It is also proposed to swap the interdependency between CCLM flag and chroma BDPCM flag in this contribution.

# Proposed method & spec changes

## Enable chroma BDPCM for YUV420 format

In this contribution, it is proposed to enable chroma BDPCM for the YUV420 color format as that in CE3-2.4 [2].

## Modification of chroma BDPCM signaling

In the proposed chroma BDPCM signaling, CCLM flag for chroma components is signaled firstly and the chroma BDPCM flag is signaled only when the CCLM flag is 0. The spec changes of the modification are shown in Table 1:

Table 1 Spec changes of the proposed chroma BDPCM signaling

| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) && ChromaArrayType != 0 ) { |  |
| --- | --- |
| if( pred_mode_plt_flag && treeType = = DUAL_TREE_CHROMA ) |  |
| palette_coding( x0, y0, cbWidth / SubWidthC, cbHeight / SubHeightC, treeType ) |  |
| else { |  |
| if( !cu_act_enabled_flag ) { |  |
| if( CclmEnabled ) |  |
| cclm_mode_flag |  |
| if( cclm_mode_flag ) |  |
| cclm_mode_idx |  |
| else { |  |
| if( cbWidth <= MaxTsSize && cbHeight <= MaxTsSize && sps_bdpcm_chroma_enabled_flag ) { |  |
| intra_bdpcm_chroma_flag | ae(v) |
| if( intra_bdpcm_chroma_flag ) |  |
| intra_bdpcm_chroma_dir_flag | ae(v) |
| } else { |  |
| if( CclmEnabled ) |  |
| cclm_mode_flag | ae(v) |
| if( cclm_mode_flag ) |  |
| cclm_mode_idx | ae(v) |
| else |  |
| intra_chroma_pred_mode |  |
| } |  |
| if ( !intra_bdpcm_chroma_flag) | ae(v) |
| intra_chroma_pred_mode |  |
| } |  |
| } |  |
| } |  |
| } |  |

# Simulation Results

## YUV420 results compared with VTM-7.0 with BDPCM=1

The anchor of simulation results provided in this section is generated by running VTM-7.0 with BDPCM=1 for all sequences on top of CTC.

Results of chroma BDPCM with the proposed signaling are provided in Table 2.

Table 2 BDPCM=2 with proposed signaling compared with VTM-7.0 BDPCM=1

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.02% | 0.01% | 96% | 99% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 0.01% | -0.05% | -0.04% | 98% | 100% |
| Class C | 0.00% | -0.01% | -0.05% | 98% | 102% |
| Class E | 0.01% | 0.01% | -0.15% | 102% | 97% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.02% | -0.23% | -0.22% | 101% | 103% |
| Class F | 0.04% | -0.41% | -0.44% | 105% | 98% |
| TGM | 0.00% | -0.95% | -0.85% | 99% | 97% |
|  | Random Access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 0.01% | -0.12% | 0.03% | 99% | 97% |
| Class C | 0.03% | -0.16% | -0.08% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.02% | -0.14% | -0.05% | 100% | 100% |
| Class F | 0.02% | -0.22% | -0.39% | 99% | 101% |
| TGM | -0.06% | -0.82% | -0.68% | 98% | 98% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 0.07% | -0.21% | -0.24% | 104% | 99% |
| Class E | -0.16% | 0.64% | -0.76% | 100% | 97% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.02% | 0.36% | -0.21% | 99% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Results of chroma BDPCM without the proposed signaling are provided in Table 3.

Table 3 BDPCM=2 without the proposed signaling, compared with VTM-7.0 BDPCM=1

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.19% | 0.57% | 0.86% | 95% | 97% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 0.17% | 0.84% | 1.25% | 99% | 100% |
| Class C | 0.17% | 0.70% | 0.99% | 98% | 99% |
| Class E | 0.36% | 0.99% | 1.16% | 100% | 97% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.15% | 0.51% | 0.97% | 100% | 101% |
| Class F | 0.21% | -0.37% | -0.37% | 104% | 95% |
| TGM | 0.21% | -0.91% | -0.72% | 99% | 92% |
|  | Random Access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 0.10% | 0.32% | 0.38% | 99% | 97% |
| Class C | 0.13% | 0.11% | 0.20% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.12% | 0.26% | 0.45% | 100% | 99% |
| Class F | 0.16% | -0.33% | -0.36% | 99% | 101% |
| TGM | 0.09% | -0.77% | -0.58% | 98% | 97% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 0.07% | 0.01% | -0.04% | 104% | 93% |
| Class E | -0.09% | 0.95% | 0.03% | 100% | 95% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.04% | -0.10% | 0.34% | 100% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

## YUV444 results compared with VTM-7.0

In this section, the YUV444 performance of the proposed chroma BDPCM signaling is evaluated according to the CE2 description [3]. The IBC, HashME, BDPCM (=2), PLT are turned on for all Classes. DualITree is set to 1. The results are shown in Table 4.

Table 4 Proposed signaling method compared with VTM-7.0, YUV444, DualITree=1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.20% | -0.02% | -0.04% | 99% | 102% |
| TGM 720p | -0.13% | -0.19% | 0.06% | 100% | 103% |
| Animation | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Mixed content | -0.22% | 0.02% | -0.07% | 100% | 101% |
| Camera-Captured content | -0.07% | -0.37% | -0.45% | 101% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.03% | 0.02% | 0.03% | 101% | 94% |
| TGM 720p | -0.06% | 0.03% | -0.01% | 101% | 96% |
| Animation | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Mixed content | -0.08% | 0.20% | 0.17% | 100% | 94% |
| Camera-Captured content | -0.06% | -0.09% | -0.16% | 101% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| TGM 720p | -0.10% | -0.15% | 0.23% | 102% | 107% |
| Animation | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Mixed content | -0.14% | -0.40% | -0.05% | 106% | 111% |
| Camera-Captured content | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

## Lossless results

Results of the proposed chroma BDPCM signaling according to the CE3 lossless test condition are provided in Table 5.

Table 5 Lossless results of the proposed chroma BDPCM signaling

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | -6.61% | #DIV/0! |  |
| Class A2 | -10.44% | #DIV/0! |  |
| Class B | -6.31% | -4.64% | #DIV/0! |
| Class C | -7.18% | -6.08% | -6.01% |
| Class D | -8.96% | -6.14% | -5.92% |
| Class E | -9.09% |  | -8.08% |
| Class F | -7.61% | -5.30% | #DIV/0! |
| TGM | -4.84% | -1.97% | -0.49% |
| Overall | -7.70% | #DIV/0! | #DIV/0! |
| Enc Time[%] | 106% | #NUM! | #NUM! |
| Dec Time[%] | 95% | #NUM! | #NUM! |

The performance changes of the proposed chroma BDPCM signaling compared with CE3-2.4a are summarized in Table 6.

Table 6 Lossless results of the proposed signaling compared with CE3-2.4a

|  | All Intra | Random Access | Low delay B |
| --- | --- | --- | --- |
|  | bit-rate savings | bit-rate savings | bit-rate savings |
| Class A1 | 0.00% | #DIV/0! |  |
| Class A2 | 0.00% | #DIV/0! |  |
| Class B | 0.00% | -0.11% | #DIV/0! |
| Class C | 0.00% | -0.10% | -0.11% |
| Class D | 0.00% | -0.08% | -0.09% |
| Class E | 0.00% |  | -0.18% |
| Class F | -0.01% | -0.08% | #DIV/0! |
| TGM | -0.06% | -0.08% | -0.44% |
| Overall | 0.00% | #DIV/0! | #DIV/0! |
| Enc Time[%] | #NUM! | #NUM! | #NUM! |
| Dec Time[%] | #NUM! | #NUM! | #NUM! |

## Additional results

### YUV 444 lossy results with DualITree=0

In this section, the YUV444 performance of the proposed chroma BDPCM signaling is evaluated according to the CE2 description [3]. The IBC, HashME, BDPCM (=2), PLT are turned on for all Classes. DualITree is set to 0. The results with are shown in Table 7.

Table 7 Proposed signaling method compared with VTM-7.0, YUV444, DualITree=0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| TGM 720p | -0.33% | 0.24% | 0.42% | 99% | 101% |
| Animation | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Mixed content | -0.63% | 0.16% | 0.05% | 99% | 102% |
| Camera-Captured content | -0.16% | 0.18% | 0.01% | 99% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.04% | 0.07% | 0.02% | 100% | 95% |
| TGM 720p | -0.13% | 0.19% | 0.27% | 100% | 97% |
| Animation | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Mixed content | -0.35% | 0.26% | 0.30% | 101% | 96% |
| Camera-Captured content | -0.07% | -0.04% | 0.05% | 103% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| TGM 720p | -0.40% | -0.26% | -0.18% | 104% | 114% |
| Animation | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Mixed content | -0.19% | 0.78% | 0.76% | 103% | 112% |
| Camera-Captured content | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

### YUV 420 lossy results compared with VTM-7.0 CTC

In this section, results of the same tests that are presented in Section 3.1 are provided with VTM-7.0 CTC as anchor (BDPCM=0 for natural sequences).

The simulation results of the chroma BDPCM with and without the proposed signaling change are shown in Table 8 and Table 9, respectively.

Table 8 BDPCM=2 with proposed signaling compared with VTM-7.0 CTC

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.11% | 0.10% | 98% | 110% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.02% | -0.02% | -0.10% | 108% | 97% |
| Class C | -0.02% | -0.10% | -0.03% | 110% | 102% |
| Class E | -0.05% | -0.14% | -0.05% | 111% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.02% | -0.15% | -0.28% | 110% | 98% |
| Class F | 0.04% | -0.41% | -0.44% | 105% | 98% |
| TGM | 0.00% | -0.95% | -0.85% | 99% | 97% |
|  | Random Access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.03% | -0.01% | 0.08% | 96% | 95% |
| Class C | 0.04% | -0.08% | -0.06% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | -0.23% | -0.13% | 100% | 95% |
| Class F | 0.02% | -0.22% | -0.39% | 99% | 101% |
| TGM | -0.06% | -0.82% | -0.68% | 98% | 98% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.04% | -0.52% | -0.27% | 99% | 99% |
| Class E | -0.23% | 0.16% | -0.53% | 98% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.04% | 0.23% | -0.55% | 99% | 91% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 9 BDPCM=2 without proposed signaling compared with VTM-7.0 CTC

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.17% | 0.45% | 0.95% | 97% | 108% |
| Class A2 | 0.24% | 0.48% | 1.14% | 104% | 94% |
| Class B | 0.19% | 0.88% | 1.18% | 109% | 96% |
| Class C | 0.15% | 0.62% | 1.01% | 110% | 100% |
| Class E | 0.29% | 0.84% | 1.26% | 109% | 99% |
| Overall | 0.20% | 0.68% | 1.11% | 106% | 99% |
| Class D | 0.12% | 0.58% | 0.91% | 108% | 97% |
| Class F | 0.21% | -0.37% | -0.37% | 104% | 95% |
| TGM | 0.21% | -0.91% | -0.72% | 99% | 92% |
|  | Random Access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -0.02% | 0.14% | 100% | 104% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.11% | 0.43% | 0.43% | 96% | 95% |
| Class C | 0.14% | 0.19% | 0.21% | 99% | 97% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.11% | 0.16% | 0.37% | 100% | 95% |
| Class F | 0.16% | -0.33% | -0.36% | 99% | 101% |
| TGM | 0.09% | -0.77% | -0.58% | 98% | 97% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.04% | -0.30% | -0.07% | 98% | 93% |
| Class E | -0.17% | 0.44% | 0.29% | 98% | 98% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.02% | -0.23% | -0.02% | 100% | 93% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Reference

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001, 16th meeting, Geneva, Oct. 2019.

- A. Nalci, H. Wang, M. Coban, M. Karczewicz, T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki, Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, T.-C. Ma, H.-J. Jhu, X. Xiu, Y.-W. Chen, X. Wang, “CE3-2.4: Luma and Chroma BDPCM for Lossless Coding with Regular Residual Coding”, JVET-Q0089, 17th meeting: Brussels, BE, January 2020.

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 2 (CE2): Palette Mode Coding”, JVET-P2022, 16th meeting, Geneva, CH, October 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).