| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0787 |
| --- | --- |

| Title: | AHG9: Subpicture location signalling bugfix |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karsten Suehring, Jingya Li, Ye-Kui-Wang | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI, Panasonic Corporation, Bytedance |  |  |

_____________________________

# Abstract

This contribution proposes to resolve a syntax issues for subpicture location, which was noticed during the implementation of subpicture HLS. The calculation of the size of the syntax elements subpic_ctu_top_left_x, subpic_ctu_top_left_y, subpic_width_minus1 and subpic_height_minus1 fails in case the maximum picture width or height is equal to or smaller than one CTB. It is proposed to signal the syntax elements conditional on the maximum picture width or height being larger than a one CTU.

# Introduction

Currently the subpicture locations are signalled in the SPS as follows:

| if( subpics_present_flag ) { |  |
| --- | --- |
| sps_num_subpics_minus1 | u(8) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) { |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| subpic_ctu_top_left_y[ i ] | u(v) |
| subpic_width_minus1[ i ] | u(v) |
| subpic_height_minus1[ i ] | u(v) |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |

The semantics for subpic_ctu_top_left_x, subpic_ctu_top_left_y, subpic_width_minus1 and subpic_height_minus1 specify the length of the u(v) coded syntax element as follows:

subpic_ctu_top_left_x[ i ] specifies horizontal position of top left CTU of i-th subpicture in unit of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_width_max_in_luma_samples / CtbSizeY ) ) bits. When not present, the value of subpic_ctu_top_left_x[ i ] is inferred to be equal to 0.

subpic_ctu_top_left_y[ i ] specifies vertical position of top left CTU of i-th subpicture in unit of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_height_max_in_luma_samples / CtbSizeY ) ) bits. When not present, the value of subpic_ctu_top_left_y[ i ] is inferred to be equal to 0.

subpic_width_minus1[ i ] plus 1 specifies the width of the i-th subpicture in units of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_width_max_in_luma_samples / CtbSizeY ) ) bits. When not present, the value of subpic_width_minus1[ i ] is inferred to be equal to Ceil( pic_width_max_in_luma_samples / CtbSizeY ) − 1.

subpic_height_minus1[ i ] plus 1 specifies the height of the i-th subpicture in units of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_height_max_in_luma_samples / CtbSizeY ) ) bits. When not present, the value of subpic_height_minus1[ i ] is inferred to be equal to Ceil( pic_height_max_in_luma_samples / CtbSizeY ) − 1.

The syntax element pic_width_max_in_luma_samples is constrained as “samples shall not be equal to 0 and shall be an integer multiple of Max( 8, MinCbSizeY )”

In case pic_width_max_in_luma_samples is smaller than CtbSizeY, the integer division would lead to Log2(0), which is undefined.

A proper non-integer division with rounding up, i.e. Ceil (pic_width_max_in_luma_samples ÷ CtbSizeY ), fixes the calculation of the number of CTBs.

In case that the width/height resolves to one CTB, the remaining Ceil( Log2(1)) resolves to zero, which is the correct number of bits required for signalling the location, but does not match our usual way of specifying syntax elements.

Similar problem descirptions are raised in Ticket #781 [1] and #790 [2].

# Proposed solution

It is proposed signal the syntax elements subpic_ctu_top_left_x, subpic_ctu_top_left_y, subpic_width_minus1 and subpic_height_minus1 conditioned on the maximum picture width or height as follows.

| if( subpics_present_flag ) { |  |
| --- | --- |
| sps_num_subpics_minus1 | u(8) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) { |  |
| if (pic_width_max_in_luma_samples > CtbSizeY){ |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| subpic_width_minus1[ i ] | u(v) |
| } |  |
| if (pic_height_max_in_luma_samples > CtbSizeY){ |  |
| subpic_ctu_top_left_y[ i ] | u(v) |
| subpic_height_minus1[ i ] | u(v) |
| } |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |

The semantics already contain an inference rule for the syntax elements not being present, so that there is no change required in semantics.

# Reference

[1] https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/781

[2] https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/790

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).