| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0791-v1 |
| --- | --- |

| Title: | AHG4: Multiformat Berlin Test Sequences |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | B. Bross H. Kirchhoffer C. Bartnik M. Palkow D. Marpe | Email: | firstname.lastname@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

Eight new test sequences are proposed to be evaluated for potential use in standardization, e.g. for verification testing and conformance bitstreams. All sequences have been shot in Berlin with a RED DSMC2 camera and recorded in 7680x4320 resolution at 60fps. The raw video with Log3G10 transfer characteristics and REDWideGamutRGB color space have been post-processed to obtain TIFF files with SDR BT.709 transfer characteristics and color space using the REDCINE-X PRO software. The TIFF files have been further processed using HDRTools to convert them into YUV420 10bit video for encoding with HM and VTM. Three different resolutions have been produced, 3840x2160 (UHD) and 1920x1080 (HD) by simply downsampling and 3840x2160 (cropped 8k) by cropping an UHD part out of the 7680x4320 video keeping the 8K resolution. From the raw format, HDR gradings can be generated as well. All eight sequences in the three different resolutions have been coded with HM-16.20 and VTM-7.0. For the CTC, the RA BD-rates Y are as follows:

BerlinCrossroads:	-38% (cropped 8K)	-29% (2160p)	-25% (1080p)

ChestnutTree:		-27% (cropped 8K)	-27% (2160p)	-23% (1080p)

March18thSquare:	-31% (cropped 8K)	-23% (2160p)	-24% (1080p)

NeptuneFountain:	-22% (cropped 8K)	-22% (2160p)	-22% (1080p)

Oberbaum:			-36% (cropped 8K)	-32% (2160p)	-31% (1080p)

Quadriga:			-39% (cropped 8K)	-35% (2160p)	-29% (1080p)

ReichstagIntoTree:	-31% (cropped 8K)	-24% (2160p)	-20% (1080p)

Spree:				-30% (cropped 8K)	-31% (2160p)	-27% (1080p)

# Sequences and Output Formats

BerlinCrossroads									ChestnutTree

March18thSquare									NeptuneFountain

Oberbaum											Quadriga

ReichstagIntoTree									Spree

For all sequences:

| Resolution: |  |
| --- | --- |
| cropped 8K | 3840x2160 |
| UHD | 3840x2160 |
| HD | 1920x1080 |
| Framerate: | 60 fps |
| Number of Frames: | 600 (10s) |
| Bitdepth: | 10bit |
| Color format: | YCbCr |
| Chroma subsampling: | 4:2:0 |
| Color gamut: | BT.709 |
| Transfer function: | BT.709 |
| Sample range: | Standard/Limited/Video |

# Technical Information

| Camera: | RED DSMC2 |
| --- | --- |
| Sensor: | Helium 8K S35 35.4 Megapixel CMOS Sensor 29.90 mm x 15.77 mm (Diagonal: 33.80 mm) |
| Lense: | Fujinon XK6x20 (20-120mm) |
| Recording resolution: | 7680x4320 @ 60fps |
| Recording format: | R3D raw (REDCODE 12:1) with RED Log3G10 log curve and REDWideGamutRGB color space |

# Post-Processing Steps

In order to obtain the 4:2:0 YUV 10bit format, the following conversions have been performed which will be further detailed in the next subsections:

- From RED raw R3D to 4:4:4 RGB 12bit TIFF

- From 4:4:4 RGB 12bit TIFF to 4:2:0 YUV 10bit raw

## R3D to 4:4:4 RGB 16bit TIFF

The following steps are performed:

- The R3D raw videos (R3Dfile) have been loaded in the REDCINE-X PRO software, BT.709 grading and TIFF output has been selected and one RMD meta datafile (RMDfile) for each sequence has been saved.

- With the REDline command line tool, the following line generates the TIFF files from the R3D raw video using the grading settings from the RMD file:

REDline -i "$R3Dfile" --loadRMD "$RMDfile" --useRMD 1 -o "$R3Dfile" --format 1 --colorSpace 1 --gammaCurve 32

Please note that an automatic grading algorithm from the REDCINE-X PRO software has been used and no particular emphasis has been put on the grading. Hence there might be still room for improvement in that regard. Also HDR gradings, e.g. for HLG or PQ, can be performed based on the R3D raw data.

## 4:4:4 RGB 12bit TIFF to 4:2:0 YUV 10bit raw

In order to convert the TIF files to YUV, the HDRConvScaler from the HDRTools has been used with the following settings:

|  | TIFF | RAW |
| --- | --- | --- |
| Bitdepth: | 12bit | 10bit |
| Color space: | RGB | YUV |
| Color primaries: | BT.709 | BT.709 |
| Transfer funtion: | BT.709 | BT.709 |
| Sample range: | Full | Standard/Limited/Video |
| Chroma format: | 4:4:4 | 4:2:0, downsampling filter: H={1, 6, 1} and V={1, 1} |
| Resolution: | 7680x4320 | 3840x2160 UHD 1920x1080 HD STM downsampling filters |

The following command lines perform the conversion with HDRConvScaler where the config file HDRConvScalerTIFFBT709ToRAW.cfg is attached to this contribution.

8k -> UHD:

HDRConvScaler -f HDRConvScalerTIFFBT709ToRAW.cfg -p SourceFile=ChestnutTree.%06d.tif -p StartFrame=0 -p NumberOfFrames=600 -p SourceWidth=7680 -p SourceHeight=4320 -p OutputFile=ChestnutTree_3840x2160_60fps_10bit_420.yuv -p OutputWidth=3840 -p OutputHeight=2160 -p OutputChromaFormat=1 -p OutputFourCCCode=0 -p OutputColorSpace=0 -p OutputSampleRange=0 -p OutputBitDepthCmp0=10 -p OutputBitDepthCmp1=10 -p OutputBitDepthCmp2=10

8k -> HD:

HDRConvScaler -f HDRConvScalerTIFFBT709ToRAW.cfg -p SourceFile=ChestnutTree.%06d.tif -p StartFrame=0 -p NumberOfFrames=600 -p SourceWidth=7680 -p SourceHeight=4320 -p OutputFile=ChestnutTree_1920x1080_60fps_10bit_420.yuv -p OutputWidth=1920 -p OutputHeight=1080 -p OutputChromaFormat=1 -p OutputFourCCCode=0 -p OutputColorSpace=0 -p OutputSampleRange=0 -p OutputBitDepthCmp0=10 -p OutputBitDepthCmp1=10 -p OutputBitDepthCmp2=10

# Results

Coding efficiency BD-rate results for HM-16.20 (anchor) vs. VTM-7.0 (test) are summarized in the following. Detailed results can be found in the accompanying Excel files.

## QPs 22, 27, 32, 37

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM-16.20 |  |  |
|  | Y | U | V | EncT | DecT |
| Cropped 8K | -25.47% | -11.43% | -13.55% | 1865% | 180% |
| 4K | -22.74% | -18.26% | -19.41% | 2338% | 180% |
| HD | -20.84% | -25.63% | -25.00% | 3223% | 192% |
| Overall | -23.02% | -18.44% | -19.32% | 2413% | 184% |

|  |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM-16.20 |  |  |
|  | Y | U | V | EncT | DecT |
| Cropped 8K | -31.76% | -35.89% | -39.12% | 578% | 89% |
| 4K | -27.89% | -34.83% | -34.89% | 623% | 151% |
| HD | -25.18% | -37.67% | -35.01% | 612% | 161% |
| Overall | -28.28% | -36.13% | -36.34% | 604% | 130% |

|  |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM-16.20 |  |  |
|  | Y | U | V | EncT | DecT |
| Cropped 8K |  |  |  |  |  |
| 4K |  |  |  |  |  |
| HD | -26.24% | -30.28% | -32.19% | 485% | 142% |
| Overall | -26.24% | -30.28% | -32.19% | 485% | 142% |

|  |  |  | Low delay P |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM-16.20 |  |  |
|  | Y | U | V | EncT | DecT |
| Cropped 8K |  |  |  |  |  |
| 4K |  |  |  |  |  |
| HD | -28.71% | -32.81% | -34.87% | 452% | 146% |
| Overall | -28.71% | -32.81% | -34.87% | 452% | 146% |

## QPs 32, 37, 42, 47

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM-16.20 |  |  |
|  | Y | U | V | EncT | DecT |
| Cropped 8K | -26.22% | -4.15% | -5.30% | 558% | 157% |
| 4K | -24.04% | -15.76% | -12.11% | 793% | 161% |
| HD | -22.63% | -21.14% | -18.13% | 1392% | 174% |
| Overall | -24.30% | -13.68% | -11.85% | 851% | 164% |

|  |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM-16.20 |  |  |
|  | Y | U | V | EncT | DecT |
| Cropped 8K | -32.52% | -25.07% | -25.35% | 239% | 112% |
| 4K | -30.30% | -29.65% | -26.87% | 270% | 141% |
| HD | -27.67% | -35.57% | -34.22% | 301% | 146% |
| Overall | -30.16% | -30.09% | -28.81% | 269% | 132% |

|  |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM-16.20 |  |  |
|  | Y | U | V | EncT | DecT |
| Cropped 8K |  |  |  |  |  |
| 4K |  |  |  |  |  |
| HD | -30.37% | -11.55% | -25.65% | 250% | 121% |
| Overall | -30.37% | -11.55% | -25.65% | 250% | 121% |

|  |  |  | Low delay P |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM-16.20 |  |  |
|  | Y | U | V | EncT | DecT |
| Cropped 8K |  |  |  |  |  |
| 4K |  |  |  |  |  |
| HD | -32.58% | -16.46% | -30.31% | 245% | 125% |
| Overall | -32.58% | -16.46% | -30.31% | 245% | 125% |

## Exemplary RA RD-plots for UHD

QPs 22, 27, 32, 37:

QPs 32, 37, 42, 47: