| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0794-v1 |
| --- | --- |

| Title: | BoG report of contributions on inter prediction and motion vector coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Haitao Yang | Email: | haitao.yang@huawei.com |
| Source: | BoG coordinator |  |  |

_____________________________

Summary

The BoG is set up to review the contributions on inter prediction an motion vector coding, as categorized into section 6.6 of JVET meeting notes.

xxx sessions have been organized for the BoG meeting, 27 contributions have been reviewed.

- Jan. 12, 1800 to 2200, on the category of AFFINE and MER.

Recommendations

Recommended adoptions

Normative change

- Revisits in track

Contributions by category

Affine motion compensation

Q0324, Q0525, Q0431

JVET-Q0324 Simplification of MV derivation for affine chroma [S. H. Wang (PKU), X. Zheng (DJI), S. S. Wang, S. W. Ma (PKU)]

This contribution proposes to simplify the MV derivation process for chroma sub-block of an affine CU. (1) For 4:2:0 format, the motion vector of a chroma 4x4 sub-block is proposed to use only top-left luma sub-block’s MV; (2) For 4:2:2 format, the motion vector of a chroma 4x4 sub-block is proposed to use only the left luma sub-block’s MV.

RD performance is 0.04% for RA and 0.01% for LDB for 4:2:0 content.

On average no performance impact for 4:2:2 content, and the averaging operation of two motion vectors for each 4x4 chroma block is removed.

Because of the performance loss for the modification to 4:2:0 content, it is not considered for recommendation.

It is commented by two experts that the changes to 4:2:2 content dose not bring significant benefits.

JVET-Q0659 Crosscheck of JVET-Q0324 "Simplification of MV derivation for affine chroma" [G. Li (Tencent)] [late]

JVET-Q0525 Non-CE4: Support large rotation and flipping in affine and PROF [A. Aminlou, D. Naik (Nokia)] [late]

Current syntax for affine motion vector supports signalling large rotation (e.g. close to 90/180/270 degree) and flipping, but engine of affine motion compensation and PROF are not working optimally for these cases. In these cases, there is a big discontinuity in borders of predicted subblocks, and PROF increases this discontinuity furthermore. This contribution proposes two solutions to fix these problems. At the first step, large rotation and flipping are detected based on affine motion vectors, and eight processing modes are defined for the current block. Then, based on the processing mode, 1) during PROF, motion vector refinements are modified, and/or 2) after performing subblock-based motion compensation, samples of the motion compensated subblocks are rotated or flipped. Under common test condition, the overall bdrate for RA and LDB is 0.00% and 0.03%, respectively. In point cloud coding (PCC) use case, where YUV sequences are generated from point cloud data, -2.81% and -3.12% overall bdrate is achieved for RA and LDB, respectively. The method in this proposal does not have noticeable effect on encoding and decoding time.

This is a follow-up contribution of JVET-P0600. Notes taken during the BoG discussion in the previous meeting can be found in JVET-P0986.

Use case on point cloud compression is explained, where patches may be rotated and use for composing the picture for coding. The proposed methods is claimed to be able to handle large rotation more efficiently.

It is asked how many orientations in the packed coding picture in point cloud coding. It is said to be two, the original one the 90 degree rotation.

On average no coding gain is shown in JVET CTC test sequences.

On average -2.81% for RA and -3.12% for LDB is achieved using PCC content.

JVET-Q0431 Interaction between Affine and SBT [F. Le Léannec, K. Naser, T. Poirier, P. Bordes (InterDigital)]

This contribution proposed to deactivate the Sub Block Transform (SBT) in case of a CU coded in Affine Merge mode. More precisely, SBT is disallowed in case of sub-block merge mode when the merge candidate used is different from SbTMVP.

The proposed mutual exclusion reportedly leads to an average BD rate change of +0.01% in JVET CTC, in random access and 0.00% in low delay configurations. Encoding time is 98% in random access.

A second proposed method 2 deactivates SBT for all affine modes, leading +0.04% BD rate increase both in RA and LDB.

It is commented that an encoder can choose not checking SBT if the complexity a concern.

The encoder only modification of disabling SBT for affine merge mode is running, and no full results are available by the time of the presentation.

It is commented that the implementation does not cover a case where affine merge mode is the first candidate in the subblock merge list, and as a results SBT is occasionally selected for affine merge mode in method 1.

JVET-Q0666 Cross-check of JVET-Q0431 (Interaction between Affine and SBT) [A. Nalci (Qualcomm)] [late]

Parallel merge region

Q0185s, Q0297, Q0356

JVET-Q0185 AHG16: On merge estimation region for VVC [Y.-L. Hsiao, C.-C. Chen, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek), H. Huang, W.-J. Chien, T. Hsieh, V. Seregin, C.-C. Chen, K. Reuzé, M. Karczewicz (Qualcomm)]

Merge estimation region (MER) is included in HEVC. MERs are non-overlapping square regions, and MER is used at the encoder side to estimate costs of merging candidates in parallel for different coding units (CUs) in one MER. This contribution proposes to add MER to VVC. The MER for VVC is achieved with minor normative changes with proper encoder-only constraints. Results are summarized as follows.

Proposed method with MER size equal to 8x8
VTM7.0-RA: {Y-BD-rate=1.29%, EncT=80%, DecT=98%}
VTM7.0-LB: {Y-BD-rate=1.67%, EncT=81%, DecT=97%}

Proposed method with MER size equal to 16x16
VTM7.0-RA: {Y-BD-rate=3.08%, EncT=62%, DecT=98%}
VTM7.0-LB: {Y-BD-rate=3.82%, EncT=63%, DecT=98%}

Proposed method with MER size equal to 32x32
VTM7.0-RA: {Y-BD-rate=3.23%, EncT=67%, DecT=96%}
VTM7.0-LB: {Y-BD-rate=3.82%, EncT=68%, DecT=97%}

Proposed method with MER size equal to 64x64
VTM7.0-RA: {Y-BD-rate=3.18%, EncT=101%, DecT=95%}
VTM7.0-LB: {Y-BD-rate=3.82%, EncT=99%, DecT=97%}

Additional results:

Encoder-only method of skipping inter merge modes when the current CU is smaller than 32x32 for luma
VTM7.0-RA: {Y-BD-rate=4.69%, EncT=93%, DecT=96%}
VTM7.0-LB: {Y-BD-rate=7.52%, EncT=88%, DecT=95%}

Encoder-only method of skipping inter merge modes when the current CU is smaller than 64x64 for luma
VTM7.0-RA: {Y-BD-rate=7.00%, EncT=96%, DecT=93%}
VTM7.0-LB: {Y-BD-rate=10.55%, EncT=90%, DecT=96%}

HEVC MER is proposed, with additional modifications shown below

- Same constraints applied to subblock merge list construction, spatial motion candidates are marked as unavailable.

Two encoder only constraints

- BT/TT splitting is constrained to make sure one CU is entirely contained in a MER, or one CU contains integer number of MERs.

- HMVP candidates, pairwise candidates and zero candidates are not selected.

It is asked what would happen if an encoder does not follow the proposed encoder-only non-normative constraints. It is claimed by the proponent that the decoder can work properly in this case.

It is claimed by several experts that this feature is important for an encoder for parallel processing.

It is noticed that the RD performance difference between the proposed method and an encoder only implementation is not significant for sequences of large resolution.

It is commented that the performance gap may depend on the level encoder optimization. E.g, the first CU inside a MER can do merge.

It is commented that the performance benefits may be larger if the same encoder constraint is applied. And the encoder only constraint is required for an encoder only parallel merge mechanism.

The parallel merge in HEVC was proposed in JCTVC-H0082.

With the proposed method, HMVP table is updated within a MER, but the HMVP candidates are not selected by the encoder.

It is commented that the cost for supporting the feature is two right shifts and two comparisons for each spatial merge candidate of each coding unit, which is not high.

It is agreed that the feature of MER is desirable.

Comparison of the complexity of four options

Option 1 (Q0185/Q0356 method 1): 2 shifts and 2 comparisons for each spatial merge candidate

Option 2 (Q0297 method 1): 2 shifts and 2 comparisons for each spatial merge candidate, and 2 comparisons for each CU

Option 3 (Q0297 method 2): 2 shifts and 2 comparisons for each spatial merge candidate, 2 additions and 2 comparisons for each CU

Option 4 (Q0356 method 2): 2 shifts and 2 comparisons for each spatial merge candidate, 2 additions and 2 comparisons for each CU

It is commented that the complexity introduced by HMVP update is not substantial.

It is commented that option 3 is slightly preferable by two experts.

Recommendation: adopt option 3.

JVET-Q0640 Crosscheck of JVET-P0185 (AHG16: On merge estimation region for VVC) [S. Esenlik (Huawei)] [late]

JVET-Q0297 AHG16: Merge estimation region [H. Huang, W.-J. Chien, T. Hsieh, V. Seregin, C.-C. Chen, K. Reuze, M. Karczewicz (Qualcomm)]

Based on Q0185, additional normative changes are proposed. When a CU is smaller than the MER,

- Method 1: No update of HMVP table if CU is smaller than MER.

- Method 2: HMVP is updated only for the CU locates at the bottom-right corner of a MER.

Considering the case MER size is 32x32, the performance improvement compared against Q0185 is about 0.6% for method1 and about 0.8% for method 2 for RA configuration.

Spec change for both methods are confirmed.

See notes in Q0185.

JVET-Q0654 Cross-check of JVET-Q0297 (AHG16: Merge estimation region with constrain in HMVP update) [Y.-W. Chen (Kwai Inc.)] [late]

JVET-Q0356 AHG16: Parallel Merge Estimation for VVC [S. Esenlik, H. Gao, B. Wang, A. M. Kotra, E. Alshina (Huawei), Y.-L. Hsiao, C.-C. Chen, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (Mediatek)]

The discussion regarding this proposal is chaired by R.-L. Liao.

Based on Q0185, normative change is made.

The difference between Q0297 and Q0356 is only happened when the non-normative block constraint is not applied.

It is commented that the complexity of these two proposals are the same as each other.

It is confirmed that the results in method 2 in Q0297 is bit exact with method 2 in Q0356. The logics are not the same.

Method 1 in Q0356 is the same as Q0185.

See notes in Q0185.

JVET-Q0583 Crosscheck of JVET-Q0356 (AHG16: Parallel Merge Estimation for VVC) [B. Bross (HHI)] [late]

DMVR and BDOF

Q0128, Q0362, Q0455, Q0522, Q0524

JVET-Q0128 On clarification of applicable conditions of DMVR and BDOF [T. Chujoh, T. Hashimoto, E. Sasaki, T. Ikai (Sharp)]

One of the applicable conditions of DMVR and BDOF is that luminance coefficients and offsets for weighted prediction do not exist for both L0 and L1 reference indexes. However, when any luminance coefficient does not exist and some chroma coefficients exist, the applicable conditions of DMVR and BDOF are claimed to be not clear. It is asserted that there are bugs of both text and software. In this contribution, add a condition that chroma coefficients and offsets for weighted prediction do not exist for both L0 and L1 reference indexes. The changes are to add only one line to the applicable condition of both DMVR and BDOF. There is no change of experimental results on CTC. There were negligible differences for fade sequences when WeightedPredB on VTM-7.0 is set to 1. If the chroma weighted prediction is not constrained, the bugs of text and software should be fixed.

In the current design, DMVR is applicable only if luma WP is not applied. However, chroma WP would be disabled for a DMVR block even it is indicated to do so. It is claimed to be a bug by the proponent.

Revisit

JVET-Q0589 Crosscheck of JVET-Q0128 (On clarification of applicable conditions of DMVR and BDOF) [P. Bordes (InterDigital)] [late]

Merge modifications

SbTMVP Q0313, Q0327, Q0389, Q0390

Miscellaneous Q0105, Q0306, Q0340, Q0350, Q0496, Q0315, Q0337, Q0338, Q0601

Weighted prediction

Q0129, Q0266, Q0349, Q0364

Others

Q0341, Q0368, Q0370, Q0444, Q0483

Contribution list (from section 6.6 of JVET meeting notes)

JVET-Q0105 nonCE4: BCW SIF Derivation for Pairwise Candidate [A. Robert, T. Poirier, F. Le Léannec (InterDigital)]

JVET-Q0558 Crosscheck of JVET-Q0105 "BCW SIF Derivation for Pairwise Candidate" [G. Li (Tencent)] [late]

JVET-Q0128 On clarification of applicable conditions of DMVR and BDOF [T. Chujoh, T. Hashimoto, E. Sasaki, T. Ikai (Sharp)]

JVET-Q0589 Crosscheck of JVET-Q0128 (On clarification of applicable conditions of DMVR and BDOF) [P. Bordes (InterDigital)] [late]

JVET-Q0129 On editorial improvements for specification of explicit weighted prediction [T. Chujoh, T. Hashimoto, E. Sasaki, T. Ikai (Sharp)]

JVET-Q0242 Cleanup for TPM, CIIP, and GEO in the colour format of 4:0:0 [L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, H. Huang, W.-J. Chien, M. Karczewicz (Qualcomm)]

JVET-Q0770 Cross-check of JVET-Q0242 (Cleanup of TPM, CIIP and GEO for 4:0:0 format) [Y.-W. Chen (Kwai Inc.)] [late]

JVET-Q0438 Monochrome processing [A. Browne, K. Sharman, S. Keating (Sony)]

JVET-Q0266 On TPM and GEO merge modes in presence of weighted prediction [A. Filippov, H. Chen, V. Rufitskiy, H. Yang, E. Alshina (Huawei)]

JVET-Q0700 Cross-check of JVET-Q0266 on TPM and GEO merge modes in presence of weighted prediction [P. Onno (Canon)] [late]

JVET-Q0748 Crosscheck of JVET-Q0266 (On TPM and GEO merge modes in presence of weighted prediction) [P. Bordes (InterDigital)] [late]

JVET-Q0306 Non-CE4: Removal of MVD scaling process for 4x8/8x4 blocks in MMVD [N. Zhang, H. Liu, L. Zhang, K. Zhang, Y. Wang (Bytedance)]

JVET-Q0635 Crosscheck of JVET-Q0306 (Non-CE4: Removal of MVD scaling process for 4x8/8x4 blocks in MMVD) [N. Park (LGE)] [late]

JVET-Q0313 Non-CE4: Constraints on block size for ATMVP [Y. Sun, F. Chen, L. Wang (Hikvision)]

JVET-Q0532 Crosscheck of JVET-Q0313 (Non-CE4: Constraints on block size for ATMVP) [H. Chen (Huawei)] [late]

JVET-Q0315 Non-CE4: Triangular prediction mode with motion vector difference [K. Zhang, L. Zhang, H. Liu, Z. Deng, Y. Wang (Bytedance)]

JVET-Q0691 Crosscheck of JVET-Q0315: Non-CE4: Triangular prediction mode with motion vector difference [J. Ström (Ericsson)] [late]

JVET-Q0327 On IBC/ATMVP candidate list construction [X. W. Meng, S. H. Wang (PKU), X. Zheng (DJI), S. S. Wang, S. W. Ma (PKU)]

JVET-Q0612 Crosscheck of JVET-Q0327: On IBC/ATMVP candidate list construction [J. Chen (Alibaba)] [late]

JVET-Q0337 Non-CE4: On weight values of the chroma-component for TPM mode [N. Park, J. Nam, H. Jang, J. Lim, S. Kim (LGE)]

JVET-Q0540 Crosscheck of JVET-Q0337 (Non-CE4: On weight values of the chroma-component for TPM mode) [T. Chujoh (Sharp)] [late]

JVET-Q0340 Non-CE4: Cleanup of the MMVD offset derivation [N. Park, J. Nam, H. Jang, J. Lim, S. Kim (LGE)]

JVET-Q0341 Non-CE4: Range of the motion vector [N. Park, J. Nam, H. Jang, J. Lim, S. Kim (LGE)]

JVET-Q0535 Crosscheck of JVET-Q0341 (Non-CE4: Range of the motion vector) [J. Li (Panasonic)] [late]

JVET-Q0349 Combination of TPM and Weighted-Prediction [P. Bordes, T. Poirier, F. LeLeannec (InterDigital)]

JVET-Q0538 Crosscheck of JVET-Q0349 (Combination of TPM and Weighted-Prediction) [T. Chujoh (Sharp)] [late]

JVET-Q0350 Non-CE: Enable CIIP for 4x8/8x4 block without SCIPU violation [H. Jang, J. Nam, N. Park, S. Kim, J. Lim (LGE)]

JVET-Q0600 Crosscheck of JVET-Q0350 (Non-CE: Enable CIIP for 4x8/8x4 block without SCIPU violation) [K. Panusopone (Nokia)] [late]

JVET-Q0354 Non-CE: Clean-up regarding syntaxes for prediction mode decision [H. Jang, J. Nam, N. Park, S. Kim, J. Lim (LGE)]

JVET-Q0651 Cross-check of JVET-Q0354 (Non-CE: Clean-up regarding syntaxes for prediction mode decision) [T.-C. Ma (Kwai Inc.)] [late]

JVET-Q0362 Non-normative aspects for cleanup of DMVR specification and software [S. Esenlik, B. Wang, A. M. Kotra, H. Gao, E. Alshina (Huawei), J. Arumugam, S. Kotecha, S. Ramamurthy (Ittiam)]

JVET-Q0368 On context model for mvp_flag and ref_idx [H. Chen, H. Yang (Huawei)]

JVET-Q0633 Crosscheck of JVET-Q0368 (On context model for mvp_flag and ref_idx) [X.-W. Li, J.-Y. Huo (Xidian Univ.), S. Wan (NPU)] [late]

JVET-Q0370 On context model for merge indices [H. Chen, H. Yang (Huawei)]

JVET-Q0534 Crosscheck of JVET-Q0370 (On context model for merge indices) [Y. Sun, F. Chen, L. Wang (Hikvision)] [late]

JVET-Q0389 Non-CE4: SbTMVP harmonization [D. Liu, C. Hollmann, R. Yu (Ericsson)]

JVET-Q0562 Crosscheck of JVET-Q0389 (Non-CE4: SbTMVP harmonization) [Y. Sun, F. Chen, L. Wang (Hikvision)] [late]

JVET-Q0390 Non-CE4: SbTMVP simplification [D. Liu, C. Hollmann, R. Yu (Ericsson)]

JVET-Q0455 On CU-level BDOF enable condition [S. Wan, Y. Xue (NPU), J.-Y. Huo, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.), Y.-F. Yu, Y. Liu (OPPO)]

JVET-Q0613 Crosscheck of JVET-O0455: On CU-level BDOF enable condition [J. Chen (Alibaba)] [late]

JVET-Q0483 On avoiding out of range motion vectors [F. Bossen, A. Segall (Sharp)]

JVET-Q0496 AHG16: On motion shift derivation of SbTMVP [Y.-W. Chen, X. Xiu, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai Inc.)]

JVET-Q0522 Non-CE4: Early termination of DMVR and study of its pixel coverage [C.-C. Chen, H. Huang, W.-J. Chien, M. Karczewicz (Qualcomm)]

JVET-Q0705 Crosscheck of JVET-Q0522 (Non-CE4: Early termination of DMVR and study of its pixel coverage) [H.-J. Jhu (Kwai Inc.)] [late]

JVET-Q0524 Non-CE4: DC balancing in DMVR [A. Aminlou, M. Homayouni (Nokia)] [late]

JVET-Q0675 Crosscheck of JVET-Q0524 (Non-CE4: DC balancing in DMVR) [K. Zhang (Bytedance)] [late]

JVET-Q0662 Cross-check of JVET-Q0525 Non-CE4: Support large rotation and flipping in affine and PROF [D. Liu (Ericsson)

JVET-Q0338 CE4-related: Harmonized conditions for CIIP and GEO [N. Park, J. Nam, H. Jang, J. Lim, S. Kim0337 (LGE)]

TBP?

JVET-Q0364 CE4.4-related: Combination of GEO and Weighted-Prediction [P. Bordes, T. Poirier, F. Le Léannec (InterDigital)]

TBP?

JVET-Q0601 Non-CE4: Extension of JVET-P0325 [A. Robert, T. Poirier, F. Le Léannec (InterDigital)] [late]

TBP?