| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0801_v1 |
| --- | --- |

| Title: | CE2/CE3-related : Lossless coding for palette coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Jungah Choi Jaehyun Lim Seunghwan Kim 19 Yangjaedaero11-gil, Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +82-2-6912-6561 sunmi.yoo@lge.com |
| Source: | LG Electronics. Inc |  |  |

_____________________________

- Abstract

In current VVC draft text, Exponential-Golomb with 3rd order (EG3) is utilized for the escape value coding. However, the distribution of the maximum escape value varies significantly over QP values so that it could produce the number of bits more than an input bitdepth of a sequence. Therefore, alternative binarization selection between the fixed length coding (FLC) and Exponential-Golomb with 3rd order (EG3) is proposed, and a high level flag is sent to indicate which binarization to be used. For FLC binarization, the input bit depth is not transmitted but derived using the syntax element, min_qp_prime_ts_minus4. The coding efficiency reportedly shows like below:

- Lossless

  - Dual Tree == 1 : -1.49% (AI) / -0.99% (RA)

  - Dual Tree == 0 : -1.95% (AI) / -1.10% (RA)

- Low Qp (2 ,7, 12, 17)

  - Dual Tree == 1 : -0.55% (AI) / X.XX% (RA)

  - Dual Tree == 0 : -0.95% (AI) / X.XX% (RA)

Proposed method

This contribution proposes an alternative binarization selection between EG3 and fixed length coding. Specifically, a high level flag is transmitted to indicate which binarization to be applied for the escape value. If the flag represents that the fixed length coding is used, the input bitdepth of the sequence is derived using the equation below:

InputBitdepth=BitDepth - min_qp_prime_ts_minus46

where BitDepth indicates the internal bitdepth. Then the input bitdepth is used as the input of the FLC. Otherwise, EG3 is used as the binarization method of the escape value.

Experimental results

To verify the coding efficiency of the proposed method, the test is conducted under the 4:4:4 test conditions in [1] and the lossless coding conditions in [2]. Since the input sources are in 4:4:4 color format, BDPCM=2 is set, and only 100 frames of each sequence are tested.

Lossless coding

For lossless coding, the FLC is used instead of existing binarization for the escape value (EG3). The binarization indication flag is sent in SPS, when sps_palette_enabled_flag is equal to 1.

Table 2 Experimental result for proposed method, lossless configuration, DualTree = 1

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | Bit-rate savings |
|  | VTM-7.0 | Tested |  | VTM-7.0 | Tested |  |
| TGM 1080p | 21.0 | 21.6 | -3.11% | 43.4 | 44.7 | -2.85% |
| TGM 720p | 11.9 | 12.0 | -1.36% | 14.6 | 14.9 | -0.84% |
| Animation | 6.1 | 6.1 | -0.09% | 1.7 | 1.7 | -0.03% |
| Mixed content | 5.6 | 5.7 | -1.81% | 4.6 | 4.6 | -0.30% |
| Camera-captured | 1.9 | 1.9 | -0.16% | 0.3 | 0.3 | -0.01% |
| Overall | 10.6 | 10.9 | -1.49% | 15.7 | 16.1 | -0.99% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

Table 2 Experimental result for proposed method, lossless configuration, DualTree = 0

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | Bit-rate savings |
|  | VTM-7.0 | Tested |  | VTM-7.0 | Tested |  |
| TGM 1080p | 27.6 | 28.9 | -4.94% | 55.1 | 57.2 | -3.54% |
| TGM 720p | 14.4 | 14.6 | -1.74% | 17.7 | 17.9 | -0.70% |
| Animation | 6.2 | 6.2 | -0.02% | 1.7 | 1.7 | -0.03% |
| Mixed content | 6.4 | 6.5 | -1.50% | 4.7 | 4.7 | -0.22% |
| Camera-captured | 1.9 | 1.9 | 0.00% | 0.3 | 0.3 | 0.00% |
| Overall | 13.1 | 13.5 | -1.95% | 19.4 | 20.0 | -1.10% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 96% |  |  | 98% |  |  |

Low QPs

For low Qp configurations, the FLC is used when the Qp is 2 to 17. The binarization indication flag is sent in SPS, when sps_palette_enabled_flag is equal to 1.

DualTree=1

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -1.00% | -1.10% | -1.16% | 99% | 98% |
| TGM 720p | -0.44% | -0.46% | -0.43% | 100% | 98% |
| Animation | -0.21% | -0.10% | -0.12% | 100% | 98% |
| Mixed content | -0.80% | -1.16% | -1.26% | 99% | 96% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 98% | 96% |
| Overall | -0.55% | -0.63% | -0.66% | 99% | 98% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| TGM 720p | -0.62% | -0.23% | -0.18% | 100% | 104% |
| Animation | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Mixed content | -0.67% | 0.05% | -0.01% | 99% | 101% |
| Camera-Captured content | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

DualTree=0

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -2.07% | -2.43% | -2.42% | 99% | 99% |
| TGM 720p | -0.79% | -0.81% | -0.97% | 100% | 101% |
| Animation | -0.12% | -0.15% | -0.13% | 98% | 100% |
| Mixed content | -1.11% | -1.61% | -1.61% | 100% | 97% |
| Camera-Captured content | 0.00% | 0.01% | -0.01% | 98% | 99% |
| Overall | -0.95% | -1.14% | -1.18% | 99% | 99% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| TGM 720p | -0.66% | -0.70% | -0.64% | 100% | 102% |
| Animation | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Mixed content | -0.67% | -0.71% | -0.61% | 99% | 99% |
| Camera-Captured content | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

- Reference

- X. Xu, et al., “Description of Core Experiment 2 (CE2): Palette Mode Coding,” JVET-P2022, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 16th Meeting: Geneva, CH, 1–11 October 2019

- T. Nguyen, et al., “Description of Core Experiment 3 (CE3): Lossless Coding,” JVET-P2023, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 16th Meeting: Geneva, CH, 1–11 October 2019