| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0802 |
| --- | --- |

| Title: | Cross-check of JVET-Q0524 Non-CE4: DC balancing in DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | Du Liu | Email: | du.liu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

JVET-Q0524 proposes to modify the SAD in DMVR by subtracting the difference of DC values between prediction blocks L0 and L1. It is reported that the description in JVET-Q0524 matches the software provided by the proponent. It is also reported that the (available) BD-rate results with JVET-CTC and non-CTC cases match the ones provided in JVET-Q0524.

# Introduction

JVET-P0524 proposes two methods to compute the DC difference (i.e., DeltaDC). Method 1 computes DeltaDC for each search point separately. Method 2 computes DC values for four corner points and estimates the remaining search points based on the four corner points using bi-linear interpolation.

The proponent requested to run the tests for the CTC cases of the two methods and the non-CTC cases with method 2. It is reported that the (available) BD-rate results match with the results in JVET-Q0524. It should be noted that the encoder and decoder run times are not accurate due to different cluster loads.

# Results

Test 1: CTC with method 1

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.10% | -0.08% | 99% | 106% |
| Class A2 | -0.09% | -0.12% | -0.08% | 99% | 99% |
| Class B | -0.09% | -0.03% | -0.06% | 100% | 97% |
| Class C | -0.04% | 0.05% | 0.00% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.04% | -0.05% | 100% | 99% |
| Class D | -0.02% | 0.07% | 0.02% | 100% | 98% |
| Class F | -0.07% | -0.02% | -0.03% | 108% | 95% |

Test 2: CTC with method 2

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.07% | -0.12% | 99% | 108% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.09% | -0.07% | -0.03% | 98% | 96% |
| Class C | -0.05% | -0.04% | -0.01% | 99% | 95% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.02% | -0.05% | 0.02% | 100% | 96% |
| Class F | -0.05% | -0.05% | -0.06%* | 108% | 97% |

*: A minor difference in the second decimal digit of ClassF component V is observed in the proposal JVET-Q0524-v2.docx. However, the results are identical to those in the excel sheet JVET-Q0524-v2_Case2_DC_Interpolation.xlsm. The minor difference is presumed to be a typo.

Test 3: non-CTC with method 2

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -0.84% | -1.05% | -0.98% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.74% | -0.74% | -0.71% | 99% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Patent rights declaration(s)

Ericsson does not have any current or pending patent rights relating to the technology described in this contribution.