| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0808 |
| --- | --- |

| Title: | QP offsets for adaptive color transform |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Wei-Jung Chien, Yung-Hsuan Chao, Adarsh Krishnan Ramasubramonian, Chun-Chi Chen, Vadim Seregin, Ted Hsieh, Geert Van der Auwera, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121 | Email: | {hanhuang, wchien}@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In the 16th JVET meeting, the adaptive color transform (ACT) for 444 video coding was adopted in VVC.  The QP adjustments of (-5, -5, -3) are applied to the transform residuals to compensate the dynamic range change of residuals signals before and after color transform. In this contribution, it’s proposed to signal additional ACT QP offsets at picture parameter set and slice header to provide flexibility of QP control for ACT coded block.

# Introduction

In JVET-P0517 [1], the adaptive color transform (ACT) tool in HEVC was implemented into the VVC framework and it was adopted in the 16th JVET meeting. The color space conversion in JVET-P0517 is carried out in the residual domain. Two additional changes were proposed for implementation in VVC: 1) move the ACT enabling/disabling flag from TU-level to CU-level; 2) disable the ACT when the separate-tree partition or intra sub-partition prediction (ISP) mode is applied.

The forward and inverse ACT are described as follows:

| C0'C1'C2'=2112-1-10-22C0C1C2/4YCoCg=12120-2-12-1RGBYCoCg=12120-2-12-1RGB C0C1C2=1101-1-11-11C0'C1'C2' | (1) |
| --- | --- |

Fixed QP adjustments of (-5, -5, -3) are applied to the transform residuals if ACT is applied. The QP adjustments of (-5, -5, -3) are applied to the transform residuals to compensate the dynamic range change of residuals signals before and after color transform.

In this contribution, the QP is clipped after adding the QP adjustments in the scaling process of transform coefficients to avoid out of range. It’s also proposed to signal additional ACT QP offsets at picture parameter set and slice header to provide flexibility of QP control for ACT coded block, which has different characteristics versus non-ACT block.  The proposed method doesn’t change the performance under common test conditions.

# Proposed Text Change

Syntax change in PPS:

| pps_cb_qp_offset | se(v) |
| --- | --- |
| pps_cr_qp_offset | se(v) |
| pps_joint_cbcr_qp_offset_present_flag | u(1) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| pps_joint_cbcr_qp_offset_value | se(v) |
| pps_slice_chroma_qp_offsets_present_flag | u(1) |
| pps_cu_chroma_qp_offset_list_enabled_flag | u(1) |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) { |  |
| chroma_qp_offset_list_len_minus1 | ue(v) |
| for( i = 0; i <= chroma_qp_offset_list_len_minus1; i++ ) { |  |
| cb_qp_offset_list[ i ] | se(v) |
| cr_qp_offset_list[ i ] | se(v) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| joint_cbcr_qp_offset_list[ i ] | se(v) |
| } |  |
| } |  |
| pps_act_y_qp_offset | se(v) |
| pps_act_cb_qp_offset | se(v) |
| pps_act_cr_qp_offset | se(v) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| pps_act_cbcr_qp_offset | se(v) |
| pps_slice_act_qp_offset_present_flag | u(1) |

Syntax change in slice header:

| if( pps_cu_chroma_qp_offset_list_enabled_flag ) |  |
| --- | --- |
| cu_chroma_qp_offset_enabled_flag | u(1) |
| if(pps_slice_act_qp_offset_present_flag && !( slice_type == I && qtbtt_dual_tree_intra_flag) ) { |  |
| slice_act_y_qp_offset | se(v) |
| slice_act_cb_qp_offset | se(v) |
| slice_act_cr_qp_offset | se(v) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| slice_act_cbcr_qp_offset | se(v) |
| } |  |

pps_act_y_qp_offset, pps_act_cb_qp_offset, pps_act_cr_qp_offset and pps_act_cbcr_qp_offset specify the offsets to quantization parameter Qp′Y, Qp′Cb, Qp′Cr and Qp′CbCr, respectively, when cu_act_enabled_flag is 1. The values of pps_act_y_qp_offset, pps_act_cb_qp_offset and pps_act_cr_qp_offset shall be in the range of −12 to +12, inclusive. When not present, the values of pps_act_y_qp_offset, pps_act_cb_qp_offset and pps_act_cr_qp_offset are inferred to be equal to 0.

pps_slice_act_qp_offsets_present_flag equal to 1 specifies that slice_act_y_qp_offset , slice_ act _cb_qp_offset, slice_act_cr_qp_offset, slice_act_cbcr_qp_offset are present in the slice header. pps_slice_act_qp_offsets_present_flag equal to 0 specifies that slice_act_y_qp_offset , slice_ act _cb_qp_offset, slice_ act _cr_qp_offset, slice_act_cbcr_qp_offset are not present in the slice header. When not present, the value of pps_slice_act_qp_offsets_present_flag is inferred to be equal to 0.

slice_act_y_qp_offset, slice_act_cb_qp_offset, slice_act_cr_qp_offset and slice_act_cbcr_qp_offset specify the offsets when determining the value of the Qp′Y, Qp′Cb, Qp′Cr and Qp′CbCr quantization parameter. The values of slice_act_y_qp_offset, slice_act_cb_qp_offset, slice_act_cr_qp_offset and slice_act_cbcr_qp_offset shall be in the range of −12 to +12, inclusive. When not present, the values are inferred to be equal to 0.

8.7.1	Derivation process for quantization parameters

…

The variable QpY is derived as follows:

QpY = ( ( qPY_PRED + CuQpDeltaVal +( cu_act_enabled_flag[ xTbY ][ yTbY ] ?  pps_act_y_qp_offset+ slice_act_y_qp_offset : 0) + 64 + 2 * QpBdOffset ) % ( 64 + QpBdOffset ) ) − QpBdOffset	(1116)

The luma quantization parameter Qp′Y is derived as follows:

Qp′Y = QpY + QpBdOffset		(1117)

When ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the following applies:

–	When treeType is equal to DUAL_TREE_CHROMA, the variable QpY is set equal to the luma quantization parameter QpY of the luma coding unit that covers the luma location ( xCb + cbWidth / 2, yCb + cbHeight / 2 ).

–	The variables qPCb, qPCr and qPCbCr are derived as follows:

qPChroma = Clip3( −QpBdOffset, 63, QpY )		(1118)

qPCb = ChromaQpTable[ 0 ][ qPChroma ]		(1119)

qPCr = ChromaQpTable[ 1 ][ qPChroma ]		(1120)

qPCbCr = ChromaQpTable[ 2 ][ qPChroma ]		(1121)

–	The chroma quantization parameters for the Cb and Cr components, Qp′Cb and Qp′Cr, and joint Cb-Cr coding Qp′CbCr are derived as follows:

- If cu_act_enabled_flag[ xTbY ][ yTbY ] is equal to 1, the following applies

Qp′Cb =  Clip3( −QpBdOffset, 63, qPCb + pps_cb_qp_offset + slice_cb_qp_offset + pps_act_cb_qp_offset+ slice_act_cb_qp_offset + CuQpOffsetCb )
	 + QpBdOffset

Qp′Cr = Clip3( −QpBdOffset, 63, qPCr + pps_cr_qp_offset + slice_cr_qp_offset+ pps_act_cr_qp_offset+ slice_act_cr_qp_offset + CuQpOffsetCr )
	 + QpBdOffset

Qp′CbCr = Clip3( −QpBdOffset, 63, qPCbCr + pps_joint_cbcr_qp_offset + 
	slice_joint_cbcr_qp_offset+ pps_act_cbcr_qp_offset+ slice_act_cbcr_qp_offset +CuQpOffsetCbCr ) + QpBdOffset

- Otherwise, the following applies:

Qp′Cb =  Clip3( −QpBdOffset, 63, qPCb + pps_cb_qp_offset + slice_cb_qp_offset + CuQpOffsetCb )
	 + QpBdOffset		(1122)

Qp′Cr = Clip3( −QpBdOffset, 63, qPCr + pps_cr_qp_offset + slice_cr_qp_offset + CuQpOffsetCr )
	 + QpBdOffset		(1123)

Qp′CbCr = Clip3( −QpBdOffset, 63, qPCbCr + pps_joint_cbcr_qp_offset + 
	slice_joint_cbcr_qp_offset +CuQpOffsetCbCr ) + QpBdOffset

8.7.3	Scaling process for transform coefficients

…

The variable QpActOffset is set equal to 5.

The quantization parameter qP is derived as follows:

- If cIdx is equal to 0, the following applies:

qP = Qp′Y		(1129)

- Otherwise, if TuCResMode[ xTbY ][ yTbY ] is equal to 2, the following applies:

qP = Qp′CbCr		(1130)

- Otherwise, if cIdx is equal to 1, the following applies:

qP = Qp′Cb		(1131)

- Otherwise (cIdx is equal to 2), the following applies:

qP = Qp′Cr		(1132)

QpActOffset = 3

The quantization parameter qP is modified and the variables rectNonTsFlag and bdShift are derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0, the following applies:

qP = Clip3(0, 63+ QpBdOffset, qP − ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? QpActOffset   5 : 0 ))	(1133)

rectNonTsFlag = ( ( ( Log2( nTbW ) + Log2( nTbH ) ) & 1 )  = =  1 )  ?  1 : 0	(1134)

bdShift = BitDepth + rectNonTsFlag +		(1135)
		( ( Log2( nTbW ) + Log2( nTbH ) ) / 2 ) − 5 + pic_dep_quant_enabled_flag

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1), the following applies:

qP = Max( QpPrimeTsMin, qP ) − ( cu_act_enabled_flag[ xTbY ][ yTbY ]  ?  5  :  0 )	(1136)

qP = Max( QpPrimeTsMin, qP− ( cu_act_enabled_flag[ xTbY ][ yTbY ]  ?  QpActOffset  :  0 ))	(1136)

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang, “Support of adaptive color transform for 444 video coding in VVC”, JVET-P0517, 16th Meeting: Geneva, CH, 1–11 October 2019