| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0814 |
| --- | --- |

| Title: | AHG8/AHG9: DPB structure and picture output |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin, Adarsh K. Ramasubramonian, Muhammed Coban Marta Karczewicz (Qualcomm) Hendry (LGE) Ye-Kui Wang (Bytedance) | Tel: Email: | vseregin@qti.qualcomm.com dr.hendry@lge.com yekui.wang@bytedance.com |
| Source: | Qualcomm Incorporated, LG Electronics, Bytedance |  |  |

_____________________________

# Abstract

This contribution proposes text implementation for cleaning-up the DPB structure and picture output process suggested based on the discussion from the first review of JVET-Q0158 and JVET-Q0308 in Track A. It includes the following aspects:

- DPB capacity is shared across all layers.

- Sub-DPB concept is removed.

- Pictures are counted in units of pictures.

- The conceptual picture size is the largest value of max_width * max_height among all the SPSs in the OLS, it is signalled in VPS.

- When the decoder needs to bump a picture, it bumps the lowest-layer picture with the oldest POC that is waiting for output. This way the pictures will always be output in the correct order across all layers.

- Max_num_reorder_pics[ i ] and max_latency_increase_plus1[ i ] are defined in unit of AUs rather than in unit of pictures.

Specification modification reflecting the above items is attached to this contribution.

# Introduction

The following is the excerpt from meeting notes after the first review of JVET-Q0158 and JVET-Q0308:

The intent is that the DPB capacity is shared across all layers.

It was also discussed whether we count pictures or bytes. It was agreed that we always count in units of pictures.

It was also agreed that (e.g., as has been the case for single-layer AVC and HEVC bitstreams), that the DPB capacity in units of pictures can be larger when the picture size is smaller.

The conceptual picture size is the largest value of max_width * max_height among all the SPSs in the OLS. This needs to be signalled in the VPS (and this isn’t currently in the pictures).

It is currently specified that the DPB capacity can never be higher than 16 pictures.

It was discussed whether we need a “sub-DPB” concept. Perhaps not.

It was agreed that when the decoder needs to bump a picture, it bumps the lowest-layer picture with the oldest POC that is waiting for output. This way the pictures will always be output in the correct order across all layers.

This contribution implements the above with the following addition:

- max_num_reorder_pics[ i ] and max_latency_increase_plus1[ i ] are defined in unit of AUs rather than in unit of pictures.

# Patent rights declaration(s)

Qualcomm Incorporated, LG Electronics, Bytedance may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).