| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0817-v |
| --- | --- |

| Title: | AHG12: On single_slice_per_subpic_flag constraint |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry, Seethal Paluri (LGE) Lulin Chen, Chun-Chia Chen, Chih-Wei Hsu, Yu-Ling Hsiao, Yu-Wen Huang, Shaw-Min Lei (MediaTek) | Email: | dr.hendry@lge.com lulin.chen@mediatek.com |
| Source: | LG Electronics and MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution provides agreed text for implementation of the agreed aspect after review of JVET-Q0210 aspect #5 and JVET-Q0224 proposal 2.

The agreed point was to remove the constraint such that When subpics_present_flag is equal to 0, single_slice_per_subpic_flag shall be equal to 0.

# Introduction and proposal

The following is the excerpt from meeting notes after the first review of JVET-Q0210 aspect 5 and JVET-Q0224:

- Remove the following constraint: When subpics_present_flag is equal to 0, single_slice_per_subpic_flag shall be equal to 0, and use the flag to change some derivation equations suitably for this case. (JVET-Q0210, a similar proposal is said to be in Q0224)

It was commented (by those who wrote that sentence) that this sentence had been put into the text by mistake.

Removing the constraint would skip some unnecessary slice syntax elements by setting single_slice_per_subpic_flag equal to 1.

Decision (cleanup): Adopt. Hendry is responsible for the software. Revisit for text to be prepared (coordinated by Hendry). Editorially, it was suggested to also consider renaming the flag to make it clear that that the flag is not irrelevant when subpics_present_flag is equal to 0.

The text for implementing the above agreed point is shown below.

...

single_slice_per_pic_or_subpic_flag equal to 1 specifies that either one of the following applies:

- when present, each subpicture in the picture referring to the PPS consists of one and only one rectangular slice

- when no subpicture is present, each picture referring to the PPS consists of one and only one rectangular slice.

single_slice_per_pic_or_subpic_flag equal to 0 specifies that each subpicture in the picture, when present, or each picture referring to the PPS may consist one or more rectangular slices. When subpics_present_flag is equal to 0, single_slice_per_subpic_flag shall be equal to 0. When single_slice_per_pic_or_subpic_flag is equal to 1, num_slices_in_pic_minus1 is inferred to be equal to sps_num_subpics_minus1.

num_slices_in_pic_minus1 plus 1 specifies the number of rectangular slices in each picture referring to the PPS. The value of num_slices_in_pic_minus1 shall be in the range of 0 to MaxSlicesPerPicture − 1, inclusive, where MaxSlicesPerPicture is specified in Annex A. When no_pic_partition_flag is equal to 1, the value of num_slices_in_pic_minus1 is inferred to be equal to 0.

...

For rectangular slices, the list NumCtuInSlice[ i ] for i ranging from 0 to num_slices_in_pic_minus1, inclusive, specifying the number of CTU in the i-th slice, and the matrix CtbAddrInSlice[ i ][ j ] for i ranging from 0 to num_slices_in_pic_minus1, inclusive, and j ranging from 0 to NumCtuInSlice[ i ] − 1, inclusive, specifying the picture raster scan address of the j-th CTB within the i-th slice, are derived as follows:

if( subpics_present_flag  &&  single_slice_per_pic_or_subpic_flag ) {
	for( i = 0; i  <=   sps_num_subpics_minus1; i++ )
		NumCtuInSlice[ i ] = 0
	for( i = 0; i < PicSizeInCtbsY; i ++ ) {
		sliceIdx = subpics_present_flag ? 0 : CtbToSubPicIdx[ i ]
		CtbAddrInSlice[ sliceIdx ][ NumCtuInSlice[ sliceIdx ] ] = i
		NumCtuInSlice[ sliceIdx ]++
	}
} else {
	…
}

...

# Patent rights declaration(s)

LG Electronics and MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).