| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0818 |
| --- | --- |

| Title: | Moving the frame-field information SEI message to the SEI specification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Virginie Drugeon Ye-Kui Wang | Tel: Email: | virginie.drugeon@eu.panasonic.com yekui.wang@bytedance.com |
| Source: | Panasonic, Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a solution to move the frame-field information SEI message from the VVC specification to the SEI specification, as requested during the discussions related to JVET-Q0042.

# Background

During the discussions related to JVET-Q0042, the group expressed the desire to move the frame-field information SEI message from the VVC specification to the SEI specification. The frame-field information SEI message is currently specified within the VVC specification because the syntax element display_elemental_periods_minus1, which is currently signalled within the frame-field information SEI message, is used within the semantics of the HRD parameters.

This contribution suggests the following changes to both the VVC and SEI specifications to solve this issue:

- Repeat the syntax element display_elemental_periods_minus1 in the picture timing SEI message.

- Require that the value of this syntax element shall be the same in the picture timing SEI message and in the frame-field information SEI message if both are present.

- Specify the variables FieldSeqFlag, FixedPicRateWithinCvsFlag, GeneralProgressiveSourceFlag and GeneralInterlacedSourceFlag within the SEI specification to take care of the dependency on the VVC specification within the semantics of the frame-field information SEI message.

This contribution provides text for both the VVC and SEI specifications implementing the modifications suggested above.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).