| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0822 |
| --- | --- |

| Title: | Reference picture resampling functionality confirmation test |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jiancong Luo Vadim Seregin | Email: | jiancong.luo@alibaba-inc.com vseregin@qti.qualcomm.com |
| Source: | Alibaba Group US, Qualcomm |  |  |

_____________________________

# Abstract

This document describes reference picture resampling functionality confirmation test (FCT) guidelines and test verification criteria.

# Test guidelines

VVC support resampling of the reference pictures with 1/8 (upsampling) to 2 (downsampling).

Low delay B (LB) CTC test configuration with the following modifications is used for the confirmation tests:

- only 5 second length (half of the CTC frame count)

- 0.5 second switch points

- 2:1 and 1.5:1 with both dimensions changing together

The following encoder parameters should be set for 2:1 and 1.5:1 scaling ratios as follows

| 2:1 test | 1.5:1 test |
| --- | --- |
| --ScalingRatioHor=2 --ScalingRatioVer=2 --FractionNumFrames=0.5 --UpscaledOutput=0 | --ScalingRatioHor=1.5 --ScalingRatioVer=1.5 --FractionNumFrames=0.5 --UpscaledOutput=0 |

Configuration files for 1.5x and 2.0x scaling ratios can be found in the cfg/rpr directory of the VTM software package.

In the VTM software, there are three output modes, which can be used for visual verification:

--UpscaledOutput=0: pictures are output after applying cropping window specified in the referred PPS (conformant output).

--UpscaledOutput=1: pictures are padded with zeros if the picture size is smaller than the max picture size specified in SPS.

--UpscaledOutput=2: pictures are upsampled if the picture size is smaller than the max picture size specified in SPS.

Figure 1. Decoder output of down-sampled image with option --UpscaledOutput=1

If it is desired to specify other than 0.5 second resolution switching interval, it can be changed by modifying SwitchPocPeriod configuration parameter, which by default indicates 0.5s. It sets the number of pictures after which the resolution switch happens. For example, --SwitchPocPeriod=2 means that after every second picture there is a resolution switch.

VTM software outputs two PSNR values when reference picture resampling is enabled:

- PSNR1: it is measured on the decoded picture size by calculating MSE for each picture, accumulating MSEs for all pictures and calculating average PSNR from the accumulated MSE.

- PSNR2: it is measured after upsampling of the decoded picture if the decoded picture size is in smaller resolution, the existed VTM interpolation filters are used for upsampling.

# Test pass criteria

To ensure that test results are correct the following criteria may be used:

- The bitstreams should be decodable without mismatch between encoder reconstruction and decoder reconstruction. For encoder/decoder picture mismatch detection, the MD5 sum hash can be enabled by setting --SEIDecodedPictureHash=1.

- Visual confirmation, --UpscaledOutput=1 or 2 can be set for this purpose. In one example, it can be set only for the decoder.

The decoded output sequence should consist of interleaved chunks as shown in Figure 1.

Figure 2. RPR decoded output