| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0003-v1 |
| --- | --- |

| Title: | AHG report: Test model software development (AHG3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Frank Bossen Xiang Li Karsten Suehring | Tel: Email: | frank@bossentech.com xlxiangli@tencent.com karsten.suehring@hhi.fraunhofer.de |
| Source: | AhG3 on Test model software development |  |  |

_____________________________

# Abstract

This report summarises the activities of the AhG3 on Test model software development that has taken place between the 17th and 18th JVET meetings.

# Introduction

The mandates given to the AHG are:

- Coordinate development of test model (VTM) software and associated configuration files.

- Produce documentation of software usage for distribution with the software.

- Discuss and make recommendations on the software development process.

- Propose improvements to the guideline document for developments of the test model software.

- Perform tests of VTM behaviour relative to HEVC and previous VTM using the VTM common test conditions.

- Coordinate with AHG on Draft text and test model algorithm description editing (AHG2) to identify any mismatches between software and text, and make further updates and cleanups to the software as appropriate.

- Coordinate with AHG6 for integration with 360lib software.

The software development continued on the GitLab server. VTM version 7.2 was tagged on Jan. 17, 7.3 on Jan. 20, VTM 8.0 on Feb. 22. VTM 8.1 is expected during the 18th JVET meeting.

# VTM software development

Development was continued on the GitLab server, which allows participants to register accounts and use a distributed development workflow based on git.

The server is located at:

https://vcgit.hhi.fraunhofer.de

The registration and development workflow is documented at:

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/wikis/VVC-Software-Development-Workflow

The VTM software can be found at

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/

VTM 7.2 was tagged on Jan. 17, 2020.

Changes relative to VTM 7.1 include:

- JVET-P1004: Removal of Bricks

- JVET-P0590, JVET-P0592: Scaling window and chroma phase

- JVET-P0126: SubPicture ID in SliceHeader

- JVET-P0181: Modifications to HRD information signalling

- JVET-P0462: add 360 related SEI messages (from HEVC)

- JVET-P0218 set temporal id and layer id for access unit delimiter

- JVET-P0450: Sample Aspect Ratio Information SEI

- JVET-P0257: Decoder speed-up for handling scaling matrices

- JVET-P0337: porting several HEVC sei messages and add encoder control and cfg files

- JVET-P0478: allow multiple PTL in DPS

- JVET-P0984: Subpicture Level Information SEI

- JVET-P1038: ALF padding - support for raster scan slice

- JVET-P0171 subpicture layout

- JVET-P0446: add concatenation related parameters to buffering period SEI / alternative CPB parameters

- JVET-P1024: single slice per Sub Picture flag

- JVET-P0185: Inference rule for vps_max_layers_minus1 when not signaled and...

- JVET-P0135: add constraint on vps_direct_dependency_flag[ i ][ j ]

- JVET-P0182: Add check to verify inter_layer_pics_present_flag

- JVET-P0184: Decoding process for generating unavailable reference pictures

- JVET-P0597: Generalized cubemap projection SEI message

- JVET-P0894: Added profiles as described in JVET-P2001-E

- various bug fixes and cleanups

VTM 7.3 was tagged on Jan. 20, 2020.

Changes relative to VTM 7.2 include:

- removal of previous cycle macros

After one release candidate, VTM 8.0 was tagged on Feb. 22, 2020.

Changes related to VTM 7.3 include:

- JVET-Q0433: Modified chroma distortion weight

- JVET-Q0054: Fix long luma deblocking decision

- AHG14: QP derivation fix for usability

- JVET-Q0150: Fix for ALF virtual horizontal CTU boundary processing

- JVET-Q0249: Cleanup of chroma clipping flags for ALF

- JVET-Q0495: Cleanup of nonlinear ALF clipping table

- JVET-Q0517: Affine down-sampling filters for RPR

- JVET-Q0147: Conditional signaling of sps_joint_cbcr_enabled_flag based on ChromaArrayType.

- JVET-Q0516/Q0685: disable MTS when there is only DC coefficient

- JVET-Q0695: Enabling the RD checking of chroma transform-skip mode for JCCR at encoder

- JVET-Q0795: Cross-component ALF

- JVET-Q0820: ACT bug fixes and reversible ACT transform

- JVET-Q0501: Palette predictor initialization from above CTU row in WPP

- JVET_Q0297: Merge estimation region

- JVET-Q0775: Enable signalling of picture header in slice header

- JVET-Q0156: Enable inter-layer prediction for STSA pictures

- JVET-Q0806: Geometric partitioning mode

- JVET-Q0503/JVET-Q0712: Encoder only improvement and fix for palette mode

- JVET-Q0787: Bug fix for subpicture width and height signalling

- JVET-Q0446: MIP with constant shift and offset

- JVET-Q0121: Deblocking control parameters

- JVET-Q0512: encoder-side improvement on enabling chroma transform-skip for ACT

- JVET-Q0447: Encoder estimation of weighted prediction parameters

- JVET-Q0110/Q0785: Enable Chroma BDPCM for 420 and Chroma BDPCM Fixes

- JVET-Q0438: Monochrome bug fixes

- JVET-Q0115: Move POC to Picture Header

- JVET-Q0169: AH9/AH12: Bitstream conformance requirements on subpicture ID

- JVET-Q0781: use two flags to signal the use of intra/inter slice(s) in the PH

- JVET-Q0436: Cabac Zero Word modification and JVET-P0188: MinCr encoder check

- JVET-Q0444: Conditional signaling of sps_affine_amvr_enabled_flag

- JVET-Q0414 method2: a constraint on gdr_pic_flag based on gdr_enabled_flag

- JVET-Q0265: Cleanup for monochrome and independently coded color planes in SPS

- JVET-Q0420: Add pps_chroma_tool_offsets_present_flag in PPS

- JVET-Q0200: Six flags in PPS for indication if signalling is in PH or in SH (RPL, ALF, SAO, deblocking, WP, QP delta)

- JVET-Q0183: Signal the max block size in SPS and conditionally signal min_qp_prime_ts_minus4

- JVET-P0097: Removing dependencies on VPS in single layer bit-streams

- JVET-O1143: Motion compensation and loop filters at sub-picture boundaries

- JVET-Q0814: Decoded Picture Buffer

- JVET-Q2014: additional cfg files for lossless ctc

- JVET-Q0629: Removal of 4x4 blocks in palette mode

- JVET-Q0114: AHG9: A few more general constraints flags

- JVET-Q0493: Palette encoder improvements for lossless coding

- JVET-Q0449: Disable smoothing half-sample interpolation filter in conjunction with RPR

- JVET-Q0210 (aspect 8): removal of ue(k) signalling in the alf_data()

- JVET-Q0468/Q0469: support of min Luma coding block size, and intra chroma QT bug fix

- Restructuring according to JVET-Q0819

- JVET-Q0786: PTL part of VPS syntax

- JEVT-Q0505: Disabling chroma scaling list signaling for 400 color format

- JVET-Q0200: Update location for signalling WP and add checks for enforcing conformance constraints

- JVET-Q0416 aspect 1: on wraparound offset signaling

- JVET-P0117: Scalability - PTL and decoder capability

- JVET-Q0471: chroma QT split based on block height

- JVET-Q0044: slice index with subpictures

- JVET-Q0379: Move the aps_id related syntax elements in the PH and SH.

- JVET-Q0400: extra SH/PH bits for future extensions

- various subpicture related adoptions (P meeting related)

- CABAC initial values retraining based on VTM-8.0rc1

- Add support for writing conformance log file

- Software manual updates

- various bug fixes and cleanups

VTM 8.1 is expected to be tagged during the 18th JVET meeting. Changes include so far:

- JVET-P0190: Scalable nesting SEI message

- JVET-Q0151-Q0205: Move WPP signalling to SPS and mandate tile entry points

- JVET-P0125: Specify EOS NAL units to be layer specific

- JVET-P0125: Aspects of constraints on TemporalId and nuh_layer_id

- JVET-Q0260: Conformance cropping window in the SPS that applies to the max picture size

- JVET-Q0118: AHG8/AHG9: Scalability HLS cleanups

- JVET-Q0259 aspect 5: Include collocated pic in PH when TMVP enabled and rpl_info_in_ph_flag is 1

- JVET-Q0117: Parameter sets cleanup

- JVET-Q0244 (aspect 3): Signal the slice width(height) in tiles when the number of tile columns(rows) is greater than 1

- JVET-Q0355: Constraint on level in DCI

- JVET-P0115: Constraint on LayerId and TemporalId

- JVET-Q0317: Cleanups on deblocking across boundaries between subpictures

- JVET-Q0359: Tile size constraint

- JVET-Q0203: Signalling multiple rectangular slices within a tile

- JVET-Q0210 (aspect 8): removal of ue(k) signalling in the alf_data()

- JVET-Q0246: Virtual Boundary Enable Flag (Rebase to master)

- JVET-Q0221: Decoding Unit Parameter Signalling

- JVET-Q0343: add guard band type in GCMP SEI message

- JVET-P0124/P0095/P0222: mixed Nal Unit type within a picture

- JVET-Q0216: Syntax modification to picture timing SEI message

- JVET-Q0044: slice index with subpictures

- JVET-Q0157: implementation of IRAP NUT constraint for reordered sub-pictures

- JVET-Q0210 (aspect 7): Constraint Virtual Boundaries (if present) to be signalled in the SPS when Subpicture info is present.

- JVET-Q0358: TemporalId constraint between ALF_APS NALU and the pic associated with PH

- JVET-P0125: the 4th aspect on SEI message constraints

- JVET-Q0798: Signal the number of merge candidates in SPS

- JVET-Q0219: Signal separate set of alternative buffering delay parameters

- JVET-Q0218 (Proposal 3): Fix missing inference rule for single_slice_per_subpic_flag

- JVET-Q0257: Constraint expression to limit actual scaling ratios rather than the picture sizes

- JVET-Q0751: Constraints and properties of mixed nal unit types

- JVET-Q0218 Proposal1 : conditionally signal tile_idx_delta_present_flag in slice signalling

- JVET-Q0346 aspect 2: scaling list used in sh

- JVET-Q0222: Modifications to subpicture information signalling

- JVET-P0118: OLS bit stream extraction

- JVET-Q0346 aspect 1: LMCS enabled flag in SH

- JVET-P0118: HRD aspects

- JVET-P0978: RPL constraint checks

- JVET-Q0289: rect_slice_flag signaling

- JVET-Q0630: Parsing dependency fix for subpicture level info SEI

- JVET-Q0399: Allow inference of scaling window from conformance window

- JVET-Q0237: STSA with TID Zero in Dependent Layer

- JVET-Q0397: Subpicture extraction process

- JVET-Q0217 Proposal 5: Modfications to reference picture information signalling

- New bitstream extractor app (stub to be extended)

- Remove HEVC_SEI macro

- Refactor: Move ParameterSetManager class into own source files

- Cleanup: Remove PPS RExt extension (and only remaining parameter cross-component prediction)

- Respect cropping window on YUV output

- Update cfg files to support the parameter name change

- various bug fixes and cleanups

# CTC Performance

The following tables show VTM 8.0 performance over HM 16.20:

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM-16.20 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -27.89% | -39.05% | -39.30% | 1865% | 221% |
| Class A2 | -27.49% | -29.04% | -26.48% | 2997% | 238% |
| Class B | -20.99% | -32.87% | -37.24% | 3361% | 227% |
| Class C | -21.74% | -25.20% | -28.82% | 4692% | 224% |
| Class E | -25.16% | -31.46% | -30.65% | 2667% | 206% |
| Overall | -24.09% | -31.32% | -32.82% | 3097% | 224% |
| Class D | -17.64% | -19.95% | -20.14% | 5303% | 217% |
| Class F | -38.68% | -43.85% | -46.26% | 5898% | 215% |
|  |  |  | Random access |  |  |
|  |  |  | Over HM-16.20 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -37.28% | -44.18% | -49.59% | 951% | 204% |
| Class A2 | -41.45% | -46.23% | -44.78% | 1080% | 222% |
| Class B | -34.02% | -53.72% | -51.85% | 991% | 192% |
| Class C | -29.08% | -38.81% | -40.31% | 1280% | 200% |
| Class E |  |  |  |  |  |
| Overall | -34.84% | -46.33% | -46.91% | 1070% | 202% |
| Class D | -26.89% | -35.50% | -34.96% | 1411% | 203% |
| Class F | -40.62% | -49.10% | -50.32% | 789% | 167% |
|  |  |  | Low delay B |  |  |
|  |  |  | Over HM-16.20 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -30.05% | -46.77% | -45.16% | 882% | 192% |
| Class C | -28.10% | -33.08% | -33.34% | 1015% | 178% |
| Class E | -32.53% | -48.40% | -44.05% | 423% | 138% |
| Overall | -30.02% | -42.62% | -40.94% | 769% | 172% |
| Class D | -25.19% | -28.47% | -28.37% | 1050% | 189% |
| Class F | -41.83% | -49.84% | -49.94% | 569% | 140% |
|  |  |  | Low delay P |  |  |
|  |  |  | Over HM-16.20 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -34.44% | -49.05% | -47.56% | 792% | 199% |
| Class C | -29.84% | -33.27% | -33.61% | 906% | 187% |
| Class E | -35.30% | -51.00% | -47.14% | 404% | 143% |
| Overall | -33.12% | -44.28% | -42.80% | 700% | 179% |
| Class D | -26.69% | -28.64% | -28.40% | 949% | 194% |
| Class F | -41.32% | -48.83% | -49.08% | 593% | 147% |

The following tables show VTM 8.0 performance compared to VTM 7.0:

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.50% | -8.28% | -8.23% | 114% | 128% |
| Class A2 | 0.67% | -11.87% | -15.48% | 112% | 130% |
| Class B | 0.35% | -16.13% | -13.68% | 115% | 123% |
| Class C | 0.33% | -6.85% | -6.57% | 114% | 122% |
| Class E | 0.32% | -11.88% | -5.54% | 113% | 125% |
| Overall | 0.42% | -11.34% | -10.14% | 114% | 125% |
| Class D | 0.22% | -6.67% | -5.31% | 117% | 116% |
| Class F | 0.44% | -7.96% | -8.06% | 122% | 119% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | -6.93% | -6.95% | 115% | 118% |
| Class A2 | 0.08% | -12.25% | -17.23% | 115% | 116% |
| Class B | -0.04% | -18.85% | -15.02% | 112% | 113% |
| Class C | -0.66% | -6.25% | -5.64% | 111% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.16% | -11.79% | -11.35% | 113% | 112% |
| Class D | -0.76% | -6.76% | -5.30% | 115% | 88% |
| Class F | -0.30% | -5.29% | -5.32% | 122% | 106% |
|  |  |  | Low delay B |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.32% | -20.85% | -18.23% | 114% | 115% |
| Class C | -0.78% | -7.15% | -6.08% | 111% | 94% |
| Class E | -1.87% | -15.88% | -6.66% | 105% | 88% |
| Overall | -0.86% | -15.04% | -11.29% | 111% | 100% |
| Class D | -0.98% | -7.37% | -5.90% | 108% | 77% |
| Class F | -0.70% | -9.68% | -6.98% | 110% | 90% |
|  |  |  | Low delay P |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -19.62% | -17.65% | 113% | 117% |
| Class C | 0.11% | -5.40% | -4.86% | 108% | 96% |
| Class E | -0.72% | -14.74% | -5.41% | 106% | 90% |
| Overall | -0.16% | -13.66% | -10.33% | 110% | 103% |
| Class D | -0.28% | -6.00% | -4.10% | 106% | 78% |
| Class F | 0.07% | -8.01% | -5.66% | 110% | 92% |

Full results are attached to this AHG report as Excel files.

# Issues encountered during the software implementation process

Several issues were encountered during software development:

- It was noticed that the UseIdentityTableForNon420Chroma configuration parameter does not work as intended. This malfunction can greatly impact results encodings done with 4:2:2 and 4:4:4 chroma format. The issue was fixed shortly before the 18th JVET meeting.

# Status of implementation of proposals of previous JVET meetings

All previously open implementation issues from the 15th meeting were resolved:

JVET-O1159 on scalable coding was reported to be resolved during the meeting 17th meeting.

The software for JVET-O1143 on subpictures was submitted on Jan. 21 2020 including updates for adoptions of the 16th meeting.

Open issues from the 16th meeting were resolved except for two issues:

JVET-P0116: Each IRAP AU is complete (i.e., there is a picture in each layer present in the CVS) and all pictures in an IRAP AU are IRAP pictures with the same NAL unit type.

A decoder for checking completeness of access units was expected to be implemented. Proponents requested to further delay the implementation because a related specification issue was found. It was stated that a related proposal would be submitted to the 18th JVET meeting.

JVET-P0359: Add an SEI message that contains only a flag self_contained_cvs_flag in its syntax.

The proponents confirmed that the implementation was still open.

# Status of proposals of the 17th JVET meeting (Brussels)

At the beginning of the 18th meeting the software AHG tracking list contains a number of meeting decisions that were not marked as implemented. With the arrangements for changing the meeting into an teleconference meeting and related AHG meetings starting earlier, the software coordinators did not find the time to contact proponents to check the status, e.g. whether the implementation was included with a different merge request, or if no implementation is required.

The following table contains all adoptions that were not marked as merged or specification only change:

| JVET-Q0112 | It is asserted that the first item (using the global maximum picture size to determine the DPB size) is required following the adoption of JVET-Q0814. Agreed. |
| --- | --- |
| JVET-Q0112 | It is asserted that the first part of the third item (using the global maximum picture size in computing limits instead of current picture size) is required following the adoption of JVET-Q0814. Agreed. |
| JVET-Q0112 | The fourth item proposes to use the cumulative worst-case picture size for all pictures in an AU to derive constraints on CPB removal time, etc. Agreed. |
| JVET-Q0398 | Sublayer wise dependency in multi-layer: when there is a dependent layer, there is an indication of the max_tid_il_ref_pics_plus1 that the layer depends on, and if that value is 0, inter-layer prediction uses only IRAP pictures. |
| JVET-Q0247 | Make the prediction weight table a fifth type of data that can be signalled either in the PH or SH (like ALF, deblocking, RPL, and SAO). |
| JVET-Q0154 | Disallow mixing of GDR and IRAP (Disallow mixing of GDR with any non-GDR). |
| JVET-Q0270 | Add a PPS flag to determine whether qp delta is sent in the PH or SH, like other things (e.g., ALF, deblocking, SAO). |
| JVET-Q0217 | The condition for calculation of AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] is modified to signal the value as a “minus1” for the 0-th entry. In the reference picture list, the short-term 0th entry cannot have a zero-valued delta POC, so the proposal this in a current condition check the semantics. (The other case is already there, so this is using the same equation as for when weighted prediction is not used.) |
| JVET-Q0404 | It proposes a way to associate filler data NAL units and filler payload SEI messages with subpictures. It was noted that these already have an association defined for association with VCL NAL units, and this should be sufficient to associate them with subpicture regions. Using this association and adding a CBR flag for the subpicture level information SEI message should be sufficient. The extraction process should account for the association. Add a CBR flag to the subpicture level info SEI message, and change the semantics and extraction process as described. |
| JVET-Q0113 | The general editorial changes regarding the specifictaion of NAL unit decoding order. When rect_slice_flag is equal to 1, the decoding order of VCL NAL units within a subpicture is specified to be in increasing order of their subpicture-level slice index values, i.e., the slice_address values. |
| JVET-Q0271 | To add a syntax element sps_independent_subpics_flag in the SPS. When equal to 1 it specifies that all subpicture boundaries in the CLVS are treated as picture boundaries and there is no loop filtering across the subpicture boundaries. subpic_treated_as_pic_flag[ i ] and loop_filter_across_subpic_enabled_flag[ i ] are signalled only when sps_independent_subpics_flag is equal to 0. |
| JVET-Q0164 | When single_slice_per_subpic_flag is equal to 1, each subpicture should contain only one slice and the vertical slice boundaries shall also be tile boundaries. |
| JVET-Q0119 | When rect_slice_flag is equal to 1, the length of slice_address is specified to be Max( Ceil( Log2( NumSlicesInSubpic[ SubPicIdx ] ) ), 1 ) bits, as opposed to be Ceil( Log2( NumSlicesInSubpic[ SubPicIdx ] ) ) bits. Instead, condition the presence of the slice_address on NumSlicesInSubpic[ SubPicIdx ] being greater than 1. |
| JVET-Q0786 | Not to repeat HRD parameters info of OLSs containing only one layer in the VPS (in addition to signalling them in the SPS). |
| JVET-Q0277 | Only allow references to SPSs/PPSs/APSs that are in the current or lower layer that is in an OLS that includes the VCL NAL unit. Ye-Kui Wang is responsible for the providing text. B. Choi is to provide the conformance check for the decoder software. |
| JVET-Q0764: | Move ref wraparound offset syntax to the PPS and add a ref wraparound enable flag in the PPS, while maintaining the ref wraparound enable flag in the SPS, and introduce a variable to disable the ref wraparound operation when ref pic scaling is enabled for the current picture relative to the reference picture. |
| JVET-Q0280 | SPS constraint on VPS id: “The value of sps_video_parameter_set_id shall be the same in all SPSs that are referred to by CLVSs in a CVS.” |
| JVET-Q0402 | Establish the semantics of subpic_treated_as_pic_flag[ ] to allow SNR scalability with independent subpictures when subpictures are aligned. |
| JVET-Q0406 | Add a constraint on cabac_zero_word for subpictures treated as pictures to obey the bin-to-bit ratio on a subpicture basis. |
| JVET-Q0443 | Modification of the subpicture level SEI message semantics to impose a constraint on MinCR. |
| JVET-Q0395 | Add constraints for BitRate and number of tiles to the subpicture level SEI message. |

# Bug tracking

The bug tracker for VTM and specification text is located at:

https://jvet.hhi.fraunhofer.de/trac/vvc

The bug tracker uses the same accounts as the HM software bug tracker. Users may need to log in again due to the different sub-domain. For spam fighting reasons account registration is only possible at the HM software bug tracker at

https://hevc.hhi.fraunhofer.de/trac/hevc

Please file all issues related to the VVC reference software into the bug tracker. Try to provide all the details, which are necessary to reproduce the issue. Patches for solving issues and improving the software are always appreciated.

# Recommendations

The AHG recommends to:

- Continue to develop the VTM reference software

- Improve documentation, especially the software manual

- Resolve any normative issues resulting from the large number of integrations in the most recent development cycle

- Encourage people to test VTM software more extensively outside of common test conditions.

- Encourage people to report all (potential) bugs that they are finding.

- Encourage people to submit bit-streams/test cases that trigger bugs in VTM.

- Encourage people to submit non-normative changes that reduce encoder run time without significantly sacrificing compression performance

- Make sure that contributions considered for adoption in the future are subject to adequate text and software review by the JVET at large

- Design and add configuration files to the VTM software for testing of HLS features