| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0013 |
| --- | --- |

| Title: | JVET AHG report: Tool reporting procedure (AHG13) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien, Jill Boyce (co-chairs) Wei Chen, Yi-Wen Chen, Roman Chernyak, Kiho Choi, Ryoji Hashimoto, Yu-Wen Huang, Hyeongmun Jang, Ru-Ling Liao, Shan Liu (vice-chairs) | Email: | wchien@qti.qualcomm.com jill.boyce@intel.com wei.chen@interdigital.com yiwenchen@kwai.com chernyak.roman@huawei.com kiho14.choi@samsung.com ryoji.hashimoto.te@renesas.com yuwen.huang@mediatek.com hm.jang@lge.com ruling.lrl@alibaba-inc.com shanl@tencent.com |
| Source: | AHG13 |  |  |

_____________________________

# Abstract

This document summarizes the activity of AHG13: “Tool reporting procedure” between the 17th meeting in Brussels, BE (7–17 Jan. 2020) and the 18th Meeting by teleconference (15–24 April 2020). Tool on/off experimental results vs. VTM anchor are provided for the tools specified in JVET-Q2005.

# Mandates

The AHG was established with the following mandates:

- Prepare output document JVET-Q2005, which describes the methodology of tool-off testing and a list of tools to be tested by identified testers, including non-CTC configurations as appropriate.

- Produce, study and develop improvements of the JVET-Q2013 testing condition description for non-4:2:0 colour format coding.

- Provide configurations files, bitstreams, and results of tool-on/tool-off testing.

- Develop and collect test results for additional testing of VVC capabilities.

- Maintain VTM software aspects for memory bandwidth analysis in coordination with AHG3.

- Use the tool usage counts and memory bandwidth usage to study the decoder complexity of features in on/off testing.

- Prepare a report with results of the tests.

# Activities

The initial version of JVET-Q2005 “Methodology and reporting template for tool testing” was provided on February 25th.

All tests described in JVET-Q2005 were conducted. VTM tool tests were conducted on VTM-8.0 software with VTM configuration by switching off or on specific tool either in configuration files or macros.

The tested tools, testers, and cross-checkers are listed in the tables below.

## Tools included in VTM (Tool off test vs VTM Anchor)

Table 1 List of adoptions included in VTM (Tool off test (unless specified) vs VTM anchor)

| Tool Name | Acronym | Document reference(s) | AI | RA | LD | Tester | Crosscheck |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Chroma separate tree | CST | JVET-P0063, JVET-P0406 | X | X | X | Tzu-Der Chuang (peter.chuang@mediatek.com) | Wei-Jung Chien (wchien@qti.qualcomm.com) |
| Dependent quantization | DQ | JVET-P0170, JVET-Q0433, | X | X | X | Tzu-Der Chuang (peter.chuang@mediatek.com) | Shan Liu (shanl; xinzzhao@ tencent.com) |
| Cross-component linear model | CCLM | JVET-O1124, JVET-Q0500, JVET-Q0194, | X | X | X | Roman Chernyak (chernyak.roman@huawei.com) | Shan Liu (shanl; leolzhao@ tencent.com) |
| multiple transform set | MTS | JVET-O0294, JVET-O0474, JVET-O0541, JVET-Q0055, JVET-Q0516, | X | X | X | Kiho Choi (kiho14.choi@samsung.com) | Shan Liu (shanl; xinzzhao@ tencent.com) |
| Adaptive loop filter | ALF | JVET-P0162, JVET-P0164, JVET-P0505, JVET-P0554, JVET-P0665, JVET-P1038, JVET-Q0249, JVET-Q0150, JVET-Q0495, JVET-Q0795, | X | X | X | Wei-Jung Chien (wchien@qti.qualcomm.com) | Tzu-Der Chuang (peter.chuang@mediatek.com) |
| Affine motion model | AFF | JVET-O0070 |  | X | X | Roman Chernyak (chernyak.roman@huawei.com) | Shan Liu (shanl; guichunli@ tencent.com) |
| subblock-based temporal merging candidates | SbTMVP | JVET-P0385 |  | X | X | Shan Liu (shanl; guichunli@ tencent.com) | Wei-Jung Chien (wchien@qti.qualcomm.com) |
| Adaptive motion vector resolution | AMVR | JVET-O0057 |  | X | X | Shan Liu (shanl; guichunli@ tencent.com) | Wei-Jung Chien (wchien@qti.qualcomm.com) |
| Geometric partition mode | GPM | JVET-Q0806 |  | X | X | Kiho Choi (kiho14.choi@samsung.com) | Shan Liu (shanl; leolzhao@ tencent.com) |
| Bi-directional optical flow | BDOF | JVET-P0091, JVET-P0519, JVET-P1023, JVET-Q0128, |  | X |  | Kiho Choi (kiho14.choi@samsung.com) | Tzu-Der Chuang (peter.chuang@mediatek.com) |
| Combined intra/inter prediction | CIIP | JVET-O0108, JVET-O0681 |  | X | X | Kiho Choi (kiho14.choi@samsung.com) | Tzu-Der Chuang (peter.chuang@mediatek.com) |
| Merge with MVD | MMVD | JVET-P1023 |  | X | X | Kiho Choi (kiho14.choi@samsung.com) | Hyeongmun Jang (hm.jang@lge.com) |
| Bi-predictive with CU weights | BCW | JVET-P0280 |  | X | X | Wei-Jung Chien (wchien@qti.qualcomm.com) | Tzu-Der Chuang (peter.chuang@mediatek.com) |
| Multi-reference line prediction | MRLP | JVET-P0418 | X | X | X | Shan Liu (shanl; leolzhao@ tencent.com) | Hyeongmun Jang (hm.jang@lge.com) |
| Intra block copy mode | IBC | JVET-P0400, JVET-P0457, JVET-P1018 | X | X | X | Shan Liu (shanl; xiaozhongxu@ tencent.com) | Wei-Jung Chien (wchien@qti.qualcomm.com) |
| Intra sub-partitioning | ISP | JVET-O0106, JVET-O0341, JVET-O0502 | X | X | X | Roman Chernyak (chernyak.roman@huawei.com) | Hyeongmun Jang (hm.jang@lge.com) |
| Decoder motion vector refinement | DMVR | JVET-O0297, JVET-O0590, JVET-O0634 |  | X |  | Hyeongmun Jang (hm.jang@lge.com) | Roman Chernyak (chernyak.roman@huawei.com) |
| Sub-block transform | SBT | JVET-P1026 |  | X | X | Roman Chernyak (chernyak.roman@huawei.com) | Shan Liu (shanl; xinzzhao@ tencent.com) |
| Luma mapping with chroma scaling | LMCS | JVET-P0254, JVET-P0371 | X | X | X | Taoran Lu (tlu@dolby.com) | Hyeongmun Jang (hm.jang@lge.com) |
| Symmetric motion vector difference | SMVD | JVET-O0284, JVET-O0414, JVET-O0567, JVET-O0572 |  | X |  | Yi-Wen Chen(yiwenchen@kwai.com) | Hyeongmun Jang (hm.jang@lge.com) |
| Quantized residual DPCM | BDPCM | JVET-P0059, JVET-Q0089, JVET-Q0110, JVET-Q0785, | X | X | X | Ru-Ling Liao (ruling.lrl@alibaba-inc.com) | Yi-Wen Chen(yiwenchen@kwai.com) |
| Matrix based intra prediction | MIP | JVET-P0054, JVET-P0199, JVET-P0803, JVET-Q0446 | X | X | X | Ru-Ling Liao (ruling.lrl@alibaba-inc.com) | Yi-Wen Chen(yiwenchen@kwai.com) |
| Low frequency non-separable transform | LFNST | JVET-P1026, JVET-P0350, JVET-Q0784, | X | X | X | Ru-Ling Liao (ruling.lrl@alibaba-inc.com) | Yi-Wen Chen(yiwenchen@kwai.com) |
| Joint coding of chrominance residuals | JCCR | JVET-N0054 | X | X | X | Ru-Ling Liao (ruling.lrl@alibaba-inc.com) | Yi-Wen Chen(yiwenchen@kwai.com) |
| Sampled-adaptive offset | SAO | JVET-Q0441, | X | X | X | Tzu-Der Chuang (peter.chuang@mediatek.com) | Ru-Ling Liao (ruling.lrl@alibaba-inc.com) |
| Prediction refinement using optical flow | PROF | JVET-P0409, JVET-P0057, JVET-P0154, JVET-P0491, JVET-P0653 |  | X | X | Yi-Wen Chen(yiwenchen@kwai.com) | Ru-Ling Liao (ruling.lrl@alibaba-inc.com) |
| Cross component adaptive loop filter | CCALF | JVET-Q0795 | X | X | X | Shan Liu (shanl; xinzzhao@ tencent.com) | Hyeongmun Jang (hm.jang@lge.com) |
| Palette coding mode** | PLT | JVET-P0077, JVET-Q0291, JVET-Q0491, JVET-Q0501, JVET-Q0504, JVET-Q0629, JVET-Q0447, JVET-Q0493, JVET-Q0503, JVET-Q0712, | X | X | X | Yung-Hsuan Chao (yunghsua@qti.qualcomm.com) | Yi-Wen Chen(yiwenchen@kwai.com) |
| Adaptive colour transform** | ACT | JVET-P0517, JVET-Q0820, JVET-Q0353, JVET-Q0512, | X | X | X | Xiaoyu Xiu (xiaoyuxiu@kwai.com) | Shan Liu (shanl; xinzzhao@ tencent.com) |

* Test was conducted by disabling DQ and enabling Sign Data Hiding.

** Test was conducted with test sequences and test condition defined in [1].

*** Test was conducted Test sequences and test condition are defined in [2] while coding parameters are set as the same as RGB SCC, i.e. --IBC=1 --HashME=1 --BDPCM=1 --PLT=1 --ColorTransform=0 --DualITree=0.

# Results

The results of the tests are summarized in Table 2-6 below. The attached spreadsheet provides additional data. Table 7 shows tool test results across several VTM versions. The method of computing combined BD-Rate_YUV is similar to the suggested method in [1]. Instead of computing PSNR_YUV for each frame and then averaging frame PSNR_YUVs for a sequence, PSNR_YUV is directly calculated from average PSNR_Y, PSNR_U, and PSNR_V. The difference of the two methods is due to neglectable rounding error. Scatter plots are also provided for the tested tools in random access configuration, comparing PSNR-Y based bd-rate on the Y axis vs. each of Enc runtime ratio, Dec runtime ratio, and a weighted average of Enc and Dec runtime ratio, (Enc + a*Dec)/(a+1), with a configurable weight, a. The exemplary weighting is set to 6 and can be adjusted in the spreadsheet attached to this report.

Full experimental results and configuration files can be found at the link below:

https://hevc.hhi.fraunhofer.de/svn/svn_VVCTestConfig/branches/VTM-8.0/

There was no bitrate or PSNR differences between testers and cross-checkers.

Encoder and Decoder runtime ratios provided by both the testers and cross-checkers are included in the reporting template, to identify if there were significant runtime differences.

Table 3 Simulation results in all intra configuration (AI) of VTM tool tests. (VTM anchor)

|  |  |  |  | AI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acronym | BDR-Y | BDR-U | BDR-V | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| CST | 0.42% | 8.85% | 8.57% | 149% | 101% | 152% | 102% |
| DQ | 1.71% | 1.40% | 1.29% | 96% | 103% | 93% | 100% |
| CCLM | 1.66% | 13.54% | 14.02% | 100% | 100% | 99% | 98% |
| MTS | 1.32% | 0.96% | 1.02% | 86% | 101% | 85% | 99% |
| ALF | 2.20% | 12.23% | 11.98% | 90% | 91% | 98% | 92% |
| MRLP | 0.32% | 0.15% | 0.11% | 100% | 100% | 98% | 100% |
| IBC | 0.63% | 0.69% | 0.72% | 52% | 100% | 55% | 100% |
| ISP | 0.50% | 0.30% | 0.29% | 85% | 98% | 85% | 97% |
| LMCS | 0.95% | 0.54% | 0.87% | 99% | 98% | 98% | 97% |
| MIP | 0.63% | 0.19% | 0.17% | 90% | 102% | 90% | 101% |
| LFNST | 0.99% | 1.98% | 2.21% | 110% | 100% | 110% | 100% |
| JCCR | 0.63% | 0.41% | 0.51% | 97% | 101% | 99% | 102% |
| SAO | 0.00% | 0.14% | 0.19% | 101% | 98% | 100% | 98% |
| CCALF | -0.14% | 9.13% | 8.15% | 99% | 97% | 100% | 98% |

Table 4 Simulation results in random access configuration (RA) of VTM tool tests. (VTM anchor)

|  |  |  |  | RA |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acronym | BDR-Y | BDR-U | BDR-V | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| CST | 0.12% | 3.35% | 3.79% | 101% | 100% | 102% | 100% |
| DQ | 1.57% | 1.03% | 0.67% | 100% | 102% | 93% | 95% |
| CCLM | 1.06% | 10.61% | 11.45% | 99% | 100% | 99% | 100% |
| MTS | 0.75% | 0.60% | 0.53% | 94% | 99% | 89% | 95% |
| ALF | 4.40% | 18.37% | 18.15% | 96% | 87% | 98% | 89% |
| AFF | 2.99% | 2.15% | 2.08% | 82% | 97% | 82% | 98% |
| SbTMC | 0.44% | 0.37% | 0.37% | 101% | 99% | 102% | 99% |
| AMVR | 1.40% | 2.08% | 2.23% | 84% | 102% | 84% | 100% |
| TPM | 0.66% | 1.04% | 1.10% | 97% | 102% | 97% | 101% |
| BDOF | 0.76% | 0.37% | 0.28% | 98% | 98% | 98% | 98% |
| CIIP | 0.27% | 0.02% | 0.00% | 98% | 100% | 98% | 100% |
| MMVD | 0.52% | 0.45% | 0.51% | 94% | 101% | 93% | 100% |
| BCW | 0.39% | 0.42% | 0.42% | 93% | 98% | 95% | 101% |
| MRLP | 0.14% | 0.00% | 0.02% | 100% | 100% | 100% | 100% |
| IBC | -0.03% | 0.12% | 0.14% | 91% | 100% | 92% | 101% |
| ISP | 0.28% | 0.27% | 0.28% | 96% | 100% | 96% | 99% |
| DMVR | 0.80% | 1.14% | 1.13% | 100% | 96% | 100% | 96% |
| SBT | 0.41% | -0.08% | -0.03% | 95% | 100% | 90% | 96% |
| LMCS | 1.41% | 1.09% | 0.91% | 95% | 100% | 94% | 98% |
| SMVD | 0.26% | 0.27% | 0.24% | 97% | 101% | 96% | 101% |
| MIP | 0.33% | 0.35% | 0.38% | 96% | 101% | 97% | 100% |
| LFNST | 0.70% | 0.78% | 1.08% | 95% | 100% | 96% | 100% |
| JCCR | 0.59% | 0.29% | -0.07% | 98% | 100% | 99% | 100% |
| SAO | 0.07% | 0.17% | 0.26% | 100% | 98% | 100% | 99% |
| PROF | 0.45% | 0.15% | 0.11% | 99% | 100% | 96% | 101% |
| CCALF | -0.14% | 12.54% | 12.60% | 94% | 93% | 100% | 99% |

Table 5 Simulation results in low delay B configuration (LDB) of VTM tool tests. (VTM anchor)

|  |  |  |  | LDB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acronym | BDR-Y | BDR-U | BDR-V | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| CST | 0.01% | 1.93% | 1.96% | 100% | 99% | 99% | 99% |
| DQ | 1.60% | 0.60% | 0.16% | 100% | 104% | 93% | 97% |
| CCLM | 0.00% | 3.15% | 2.81% | 100% | 101% | 99% | 96% |
| MTS | 0.53% | 0.26% | 0.04% | 98% | 100% | 93% | 96% |
| ALF | 4.06% | 23.59% | 18.29% | 95% | 89% | 95% | 88% |
| AFF | 2.95% | 2.25% | 2.39% | 73% | 92% | 74% | 91% |
| SbTMC | 0.69% | 1.07% | 0.76% | 101% | 94% | 100% | 95% |
| AMVR | 0.59% | 1.06% | 0.86% | 86% | 99% | 86% | 101% |
| GPM | 1.50% | 1.83% | 1.64% | 94% | 103% | 95% | 102% |
| CIIP | 0.37% | 0.54% | 0.57% | 98% | 101% | 97% | 99% |
| MMVD | 0.46% | 0.52% | 0.49% | 95% | 98% | 96% | 100% |
| BCW | 0.24% | 0.37% | 0.37% | 96% | 97% | 97% | 100% |
| MRLP | 0.03% | 0.05% | 0.03% | 99% | 98% | 100% | 100% |
| IBC | -0.01% | -0.20% | -0.09% | 85% | 97% | 87% | 101% |
| ISP | 0.04% | 0.15% | -0.08% | 100% | 101% | 99% | 100% |
| SBT | 0.58% | -0.07% | -0.20% | 92% | 100% | 87% | 95% |
| LMCS | 0.88% | -0.07% | -0.08% | 94% | 97% | 94% | 98% |
| MIP | 0.18% | 0.40% | 0.53% | 95% | 100% | 104% | 98% |
| LFNST | 0.33% | 0.88% | 0.96% | 92% | 100% | 107% | 98% |
| JCCR | 0.13% | 1.81% | 2.12% | 100% | 102% | 99% | 98% |
| SAO | 0.07% | 0.71% | 1.04% | 100% | 98% | 101% | 100% |
| PROF | 0.27% | 0.35% | 0.26% | 99% | 100% | 98% | 99% |
| CCALF | -0.16% | 16.94% | 13.04% | 94% | 94% | 100% | 99% |

Table 7 Simulation results for screen coding tools for ClassF and ClassTGM (VTM anchor)

|  |  |  |  | AI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acronym | BDR-Y | BDR-U | BDR-V | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| IBC Class F | 15.11% | 15.04% | 15.03% | 54% | 98% | 54% | 99% |
| IBC Class TGM | 46.91% | 44.57% | 44.58% | 63% | 99% | 62% | 99% |
| BDPCM ClassF | 0.92% | 0.81% | 0.76% | 98% | 101% | 106% | 102% |
| BDPCM ClassTGM | 1.39% | 1.58% | 1.54% | 100% | 102% | 97% | 100% |
|  |  |  |  | RA |  |  |  |
| IBC Class F | 12.30% | 12.36% | 12.51% | 87% | 98% | 86% | 100% |
| IBC Class TGM | 22.21% | 21.63% | 22.04% | 90% | 102% | 89% | 103% |
| BDPCM ClassF | 0.72% | 0.63% | 0.60% | 99% | 101% | 103% | 103% |
| BDPCM ClassTGM | 0.82% | 0.99% | 0.99% | 100% | 101% | 101% | 102% |
|  |  |  |  | LD |  |  |  |
| IBC Class F | 6.19% | 5.98% | 6.41% | 87% | 192% | 85% | 99% |
| IBC Class TGM | 11.45% | 11.79% | 12.06% | 87% | 104% | 86% | 103% |
| BDPCM ClassF | 0.48% | 0.77% | -0.39% | 99% | 100% | 102% | 104% |
| BDPCM ClassTGM | 0.44% | 0.35% | 0.34% | 99% | 102% | 102% | 101% |

Table 8 Simulation results of coding tools for color space 4:4:4 (VTM anchor)

|  |  |  |  | AI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acronym | BDR-Y | BDR-U | BDR-V | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| PLT | 14.12% | 15.57% | 16.78% | 98% | 108% | 99% | 111% |
| ACT on, RGB SCC | -8.24% | -2.47% | -3.32% | 104% | 99% | 103% | 98% |
| ACT on, RGB Natural | -13.34% | -0.71% | -5.13% | 118% | 103% | 130% |  |
|  |  |  |  | RA |  |  |  |
| PLT | 9.76% | 12.12% | 13.54% | 99% | 102% | 99% | 102% |
| ACT on, RGB SCC | -11.79% | -4.49% | -5.36% | 106% | 101% | 107% | 99% |
| ACT on, RGB Natural | -21.00% | -3.02% | -8.37% | 111% | 101% | 115% |  |
|  |  |  |  | LD |  |  |  |
| PLT | 5.13% | 7.52% | 8.51% | 96% | 100% | 98% | 103% |
| ACT on, RGB SCC | -15.95% | -6.51% | -7.23% | 105% | 98% |  |  |
| ACT on, RGB Natural | -28.08% | -5.51% | -12.56% | 108% | 100% |  |  |

Table 9 Luma sample usage and memory bandwidth results of VTM tool “off” test. (VTM anchor)

|  | AI |  | RA |  |  | LDB |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acronym | Sample usage | Sample usage | Ave mem BW | Max mem BW | Sample usage | Ave mem BW | Max mem BW |
| CCLM | 48.21% | 3.73% |  |  | 0.78% |  |  |
| ALF | 99.00% | 70.60% |  |  | 67.26% |  |  |
| AFF |  | 18.28% |  |  | 27.56% |  |  |
| SBTMC |  | 10.75% |  |  | 13.38% |  |  |
| AMVR |  | 5.45% |  |  | 2.56% |  |  |
| GPM |  | 2.50% |  |  | 6.15% |  |  |
| BDOF |  | 44.75% |  |  |  |  |  |
| CIIP |  | 0.88% |  |  | 1.48% |  |  |
| MMVD |  | 7.05% |  |  | 8.61% |  |  |
| BCW |  | 9.91% |  |  | 8.25% |  |  |
| MRLP | 6.40% | 0.58% |  |  | 0.23% |  |  |
| DMVR |  | 39.97% |  |  |  |  |  |
| SBT |  | 2.60% |  |  | 4.28% |  |  |
| SMVD |  | 2.83% |  |  |  |  |  |
| MIP | 23.64% | 5.14% |  |  | 2.40% |  |  |
| LFNST | 9.79% | 0.78% |  |  | 0.35% |  |  |
| JCCR | 11.02% | 0.53% |  |  | 0.12% |  |  |
| SAO | 31.67% | 7.16% |  |  | 8.08% |  |  |

Table 7 test results of VTM tool “off” test on various VTM versions

|  |  |  | VTM RA |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Abbreviation | VTM3 | VTM4 | VTM5 | VTM6 | VTM7 | VTM8 |
| CST | 0.72% | 1.08% | 1.22% | 0.96% | 0.88% | 0.80% |
| DQ | 1.41% | 1.39% | 1.27% | 1.27% | 1.28% | 1.41% |
| CCLM | 3.94% | 4.01% | 3.84% | 3.57% | 3.60% | 3.26% |
| MTS | 1.25% | 0.82% | 0.37% | 0.68% | 0.70% | 0.71% |
| ALF | 3.61% | 3.71% | 4.78% | 4.65% | 4.63% | 7.06% |
| AFF | 2.43% | 2.47% | 2.39% | 2.82% | 2.80% | 2.80% |
| SbTMC | 0.52% | 0.43% | 0.40% | 0.48% | 0.43% | 0.43% |
| AMVR | 0.97% | 1.11% | 1.13% | 1.60% | 1.59% | 1.56% |
| GPM | 0.43% | 0.43% | 0.40% | 0.39% | 0.44% | 0.74% |
| BDOF | 1.02% | 0.63% | 0.67% | 0.67% | 0.66% | 0.66% |
| CIIP | 0.43% | 0.51% | 0.32% | 0.24% | 0.23% | 0.22% |
| MMVD | 0.81% | 0.52% | 0.59% | 0.52% | 0.51% | 0.51% |
| BCW | 0.48% | 0.45% | 0.46% | 0.43% | 0.41% | 0.40% |
| MRLP | 0.24% | 0.18% | 0.17% | 0.18% | 0.14% | 0.12% |
| ISP |  | 0.24% | 0.12% | 0.20% | 0.30% | 0.28% |
| DMVR |  | 0.80% | 0.87% | 0.87% | 0.89% | 0.88% |
| SBT |  | 0.33% | 0.34% | 0.31% | 0.31% | 0.31% |
| LMCS |  | 0.64% | 0.61% | 0.97% | 1.36% | 1.32% |
| SMVD |  | 0.26% | 0.24% | 0.27% | 0.26% | 0.26% |
| MIP |  |  | 0.28% | 0.32% | 0.37% | 0.34% |
| LFNST |  |  | 0.75% | 0.60% | 0.74% | 0.75% |
| JCCR |  |  | 0.28% | 0.35% | 0.32% | 0.41% |
| SAO | 0.80% | 0.63% | 0.16% | 0.13% | 0.12% | 0.10% |
| PROF |  |  |  | 0.41% | 0.39% | 0.38% |
| CCALF |  |  |  |  |  | 2.30% |

Figure 1. PSNR-Y vs encoding runtime ratio of VTM with VTM tool tests (VTM anchor)

Figure 2. PSNR-Y vs decoding runtime ratio of VTM with VTM tool tests (VTM anchor)

Figure 3. PSNR-Y vs weighted runtime ratio (a = 6) of VTM with VTM tool tests (VTM anchor)

# Related contributions

| JVET-R0468 | AHG13: On RGB common test condition | Y.-H. Chao, W.-J. Chien, M. Karczewicz (Qualcomm), X. Xiu, Y.-W. Chen, X. Wang (Kwai) |
| --- | --- | --- |

# Recommendations

The AHG recommends the following:

- Consider the reported tool test results during tool adoption decision making

- Review non-420 common test condition for RGB contents

- Refine list of tested tools and test methodology for the next meeting cycle

# Reference

[1] K. Andersson, F. Bossen, J.-R. Ohm, A. Segall, R. Sjöberg, J. Ström, G. J. Sullivan, “Summary information on BD-rate experiment evaluation practices”, JVET-Q2016, Brussels, BE, 7–17 January 2020