| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0014 |
| --- | --- |

| Title: | JVET AHG report: Lossless and near-lossless coding (AHG14) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | T. Nguyen and T.-C. Ma (co-chairs), M. Ikeda, H. Jang, X. Zhao (vice-chairs) | Email: | tung.nguyen@hhi.fraunhofer.de tsung-chuanma@kwai.com masaru.ikeda@sony.com hm.jang@lge.com xinzzhao@tencent.com |
| Source: | AHG14 on lossless and near-lossless coding tools |  |  |

_____________________________

# Abstract

This document reports the activity of AHG 14 on lossless and near-lossless coding tools between the 17th JVET meeting in Brussels and the 18th Meeting via Teleconference.

# Introduction

The JVET established the AHG 14 at the Brussels meeting with the following mandates:

- Produce, study and develop improvements of the JVET-Q2014 testing condition description.

- Study coding tools for lossless and near-lossless coding, including transform skip, BDPCM, and other potential technologies.

- Consider the interaction between coding tools and other processing such as loop filtering and LMCS for lossless and near-lossless coding.

- Consider throughput bottlenecks for lossless and near-lossless coding at high resolutions and frame rates.

# Activities

Discussions related to AHG14 used the JVET email reflector (jvet@lists.rwth-aachen.de), and the AHG chairs sent a kick-off message on 3th February 2020. No technical emails have been exchanged related to the AHG. The output document JVET-Q0214 was produced that specify the lossless CTC.

The results for VTM-8.0 against VTM-7.0 for the 420 test set are as follows.

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM7 | VTM8 |  | VTM7 | VTM8 |  |
| Class A1 | 2.2 | 2.4 | -6.62% | 2.2 | 2.4 | -7.20% |
| Class A2 | 1.6 | 1.8 | -10.44% | 1.7 | 1.9 | -8.52% |
| Class B | 2.2 | 2.3 | -6.31% | 2.3 | 2.5 | -4.65% |
| Class C | 1.9 | 2.1 | -7.17% | 2.4 | 2.6 | -6.12% |
| Class D | 1.9 | 2.1 | -8.96% | 2.8 | 2.9 | -6.18% |
| Class E | 2.8 | 3.1 | -9.15% |  |  |  |
| Class F | 5.3 | 5.8 | -7.57% | 33.7 | 35.4 | -5.45% |
| TGM | 11.8 | 12.4 | -4.63% | 107.1 | 109.0 | -1.85% |
| Overall | 2.1 | 2.3 | -7.71% | 2.2 | 2.4 | -6.33% |
| Enc Time[%] | 95% |  |  | 109% |  |  |
| Dec Time[%] | 98% |  |  | 105% |  |  |

The results for HEVC RExt relative to HEVC Main/Main10 are as follows using HM-16.20.

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | HM-16.20 | HM-16.20 Rext |  | HM-16.20 | HM-16.20 Rext |  |
| Class A1 | 2,2 | 2,3 | -4,50% | 2,3 | 2,4 | -3,88% |
| Class A2 | 1,7 | 1,8 | -5,88% | 1,8 | 1,9 | -4,52% |
| Class B | 2,2 | 2,3 | -5,06% | 2,3 | 2,4 | -2,59% |
| Class C | 1,9 | 2,0 | -5,42% | 2,5 | 2,5 | -2,22% |
| Class D | 1,9 | 2,1 | -7,85% | 2,8 | 2,9 | -2,56% |
| Class E | 2,7 | 3,0 | -8,22% |  |  |  |
| Class F | 4,5 | 5,2 | -12,17% | 26,6 | 30,6 | -8,54% |
| TGM | 6,1 | 8,1 | -22,91% | 74,4 | 99,5 | -20,65% |
| Overall | 2,1 | 2,3 | -5,71% | 2,3 | 2,3 | -3,14% |
| Enc Time[%] | 95% |  |  | 105% |  |  |
| Dec Time[%] | 93% |  |  | 90% |  |  |

The results for VTM-7.0 relative to HEVC Main/Main10 are as follows.

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | HM-16.20 | VTM-7.0 |  | HM-16.20 | VTM-7.0 |  |
| Class A1 | 2,2 | 2,2 | -0,24% | 2,3 | 2,2 | 1,49% |
| Class A2 | 1,7 | 1,6 | 5,96% | 1,8 | 1,7 | 5,45% |
| Class B | 2,2 | 2,2 | -0,30% | 2,3 | 2,3 | 0,11% |
| Class C | 1,9 | 1,9 | -0,36% | 2,5 | 2,4 | 1,55% |
| Class D | 1,9 | 1,9 | -0,82% | 2,8 | 2,8 | 1,20% |
| Class E | 2,7 | 2,8 | -2,18% |  |  |  |
| Class F | 4,5 | 5,3 | -13,28% | 26,6 | 33,7 | -10,55% |
| TGM | 6,1 | 11,8 | -44,31% | 74,4 | 107,1 | -30,87% |
| Overall | 2,1 | 2,1 | 0,43% | 2,3 | 2,2 | 1,84% |
| Enc Time[%] | 3133% |  |  | 1339% |  |  |
| Dec Time[%] | 172% |  |  | 136% |  |  |

The results for VTM-7.0 relative to HEVC RExt are as follows.

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | HM-16.20 Rext | VTM-7.0 |  | HM-16.20 Rext | VTM-7.0 |  |
| Class A1 | 2,3 | 2,2 | 4,47% | 2,4 | 2,2 | 5,59% |
| Class A2 | 1,8 | 1,6 | 12,66% | 1,9 | 1,7 | 10,50% |
| Class B | 2,3 | 2,2 | 5,06% | 2,4 | 2,3 | 2,81% |
| Class C | 2,0 | 1,9 | 5,37% | 2,5 | 2,4 | 3,86% |
| Class D | 2,1 | 1,9 | 7,65% | 2,9 | 2,8 | 3,86% |
| Class E | 3,0 | 2,8 | 6,59% |  |  |  |
| Class F | 5,2 | 5,3 | -1,51% | 30,6 | 33,7 | -2,52% |
| TGM | 8,1 | 11,8 | -28,31% | 99,5 | 107,1 | -12,06% |
| Overall | 2,3 | 2,1 | 6,55% | 2,3 | 2,2 | 5,18% |
| Enc Time[%] | 3285% |  |  | 1270% |  |  |
| Dec Time[%] | 185% |  |  | 151% |  |  |

# Related contributions

JVET-R0083 AHG14: Residual coding constraints for transform skip blocks

JVET-R0084 AHG14: On signalling for lossless coding

JVET-R0110 AHG14: Mixed lossy/lossless coding of VTM reference software

JVET-R0116 AHG11/AHG14: On sign data hiding of transform skip block

JVET-R0140 AHG14: Max BT/TT size restriction for lossless coding encoder configuration

JVET-R0143 AHG14: Configuration parameter to enable TSRC for lossless coding

JVET-R0144 AHG14: On lossless operation with RRC

JVET-R0169 AHG14: Report of CABAC skip mode results on VTM-8.0

JVET-R0271 AHG9/AHG14: High-level constraints of dependent quantization and sign data hiding

JVET-R0325 AHG14: Disabling dependent quantization and sign bit hiding for transform skip mode

JVET-R0353 AHG14: On Interaction between ACT and BDPCM

JVET-R0354 AHG14: BDPCM for Inter/IBC-predicted residuals

# Recommendations

The AHG recommends:

- To review all related contributions

- Discussion on mixed lossy/lossless conditions