| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0044 |
| --- | --- |

| Title: | AHG9: On subpicture boundary handling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jingya Li, Kiyofumi Abe | Tel: Email: | Jingya.li@sg.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to unify the behavior of loop filter on subpicture boundary when subpicture is treated as picture, i.e., to always disable loop filter across subpicture boundaries between independent subpictures and enable loop filter across subpicture boundaries between dependent subpictures.

# Introduction

From previous discussion, subpicture targets at (1) viewport dependent decoding devices, like a head-mount-display (HMD), (2) 360 equirectangular and cubemap projection formatted video (3) E-sports.[1]-[4]

Figure 1: example of subpicture use cases: 360 viewport dependent streaming

It is noted that for the known use cases as stated above, it is beneficial for subjective quality to disable loop filter between independent subpictures and enable loop filter between dependent subpictures.

# Proposed methods

## To implicitly derive loop_filter_across_subpic_enabled_flag using subpic_treated_as_pic_flag.

It is proposed to always disable loop filter across subpicture boundaries between independent subpictures and enable loop filter across subpicture boundaries between dependent subpictures.

The proposed syntax change is as below:

| if( subpic_info_present_flag ) { |  |
| --- | --- |
| sps_num_subpics_minus1 | ue(v) |
| sps_independent_subpics_flag | u(1) |
| for( i = 0; sps_num_subpics_minus1 > 0 && i <= sps_num_subpics_minus1; i++ ) { |  |
| if( i > 0 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| if( i > 0 && pic_height_max_in_luma_samples > CtbSizeY ) { |  |
| subpic_ctu_top_left_y[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_width_minus1[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_height_max_in_luma_samples > CtbSizeY ) |  |
| subpic_height_minus1[ i ] | u(v) |
| if( !sps_independent_subpics_flag) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |

loop_filter_across_subpic_enabled_flagLoopFilterAcrossSubpicEnabledFlag[ i ] equal to 1 specifies that in-loop filtering operations may be performed across the boundaries of the i-th subpicture in each coded picture in the CLVS. loop_filter_across_subpic_enabled_flagLoopFilterAcrossSubpicEnabledFlag[ i ] equal to 0 specifies that in-loop filtering operations are not performed across the boundaries of the i-th subpicture in each coded picture in the CLVS. When not present, theThe value of loop_filter_across_subpic_enabled_pic_flagLoopFilterAcrossSubpicEnabledFlag[ i ] is inferredset to be equal to 1 − sps_independent_subpics_flagsubpic_treated_as_pic_flag[ i ].

An editorial improvement can be made to rename subpicture syntax can below:

| if( subpic_info_present_flag ) { |  |
| --- | --- |
| sps_num_subpics_minus1 | ue(v) |
| sps_independent_subpics_flag sps_all_independent_subpics_flag | u(1) |
| for( i = 0; sps_num_subpics_minus1 > 0 && i <= sps_num_subpics_minus1; i++ ) { |  |
| if( i > 0 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| if( i > 0 && pic_height_max_in_luma_samples > CtbSizeY ) { |  |
| subpic_ctu_top_left_y[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_width_minus1[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_height_max_in_luma_samples > CtbSizeY ) |  |
| subpic_height_minus1[ i ] | u(v) |
| if( !sps_independent_subpics_flag sps_all_independent_subpics_flag) { |  |
| subpic_treated_as_pic_flag independent_subpic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |

loop_filter_across_subpic_enabled_flagLoopFilterAcrossSubpicEnabledFlag[ i ] equal to 1 specifies that in-loop filtering operations may be performed across the boundaries of the i-th subpicture in each coded picture in the CLVS. loop_filter_across_subpic_enabled_flagLoopFilterAcrossSubpicEnabledFlag[ i ] equal to 0 specifies that in-loop filtering operations are not performed across the boundaries of the i-th subpicture in each coded picture in the CLVS. When not present, theThe value of loop_filter_across_subpic_enabled_pic_flagLoopFilterAcrossSubpicEnabledFlag[ i ] is inferredset to be equal to 1 – sps_independent_subpics_flagindependent_subpic_flag[ i ].

## To implicitly derive loop_filter_across_subpic_enabled_flag  using subpic_treated_as_pic_flag when subpic_treated_as_pic_flag is false.

A less aggressive approach over 2.1 is to constraint the signaling of loop_filter_across_subpic_enabled_flag only when subpic_treated_as_pic_flag is false.

Syntax change is as below:

| if( subpic_info_present_flag ) { |  |
| --- | --- |
| sps_num_subpics_minus1 | ue(v) |
| sps_independent_subpics_flag | u(1) |
| for( i = 0; sps_num_subpics_minus1 > 0 && i <= sps_num_subpics_minus1; i++ ) { |  |
| if( i > 0 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| if( i > 0 && pic_height_max_in_luma_samples > CtbSizeY ) { |  |
| subpic_ctu_top_left_y[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_width_minus1[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_height_max_in_luma_samples > CtbSizeY ) |  |
| subpic_height_minus1[ i ] | u(v) |
| if( !sps_independent_subpics_flag) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| if(!subpic_treated_as_pic_flag [i]){ |  |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |

loop_filter_across_subpic_enabled_flag[ i ] equal to 1 specifies that in-loop filtering operations may be performed across the boundaries of the i-th subpicture in each coded picture in the CLVS. loop_filter_across_subpic_enabled_flag[ i ] equal to 0 specifies that in-loop filtering operations are not performed across the boundaries of the i-th subpicture in each coded picture in the CLVS. When not present, the value of loop_filter_across_subpic_enabled_pic_flag[ i ] is inferred to be equal to 1 – sps_independent_subpics_flag0.

## To remove signalling of loop_filter_across_subpic_enabled_flag and subpic_treated_as_pic_flag.

It is asserted that it further simplifies current design without known side effect on subpicture use cases by implicitly setting values of loop_filter_across_subpic_enabled_flag same as 1-sps_independent_subpics_flag and subpic_treated_as_pic_flag same as sps_independent_subpics_flag.

| if( subpic_info_present_flag ) { |  |
| --- | --- |
| sps_num_subpics_minus1 | ue(v) |
| sps_independent_subpics_flag | u(1) |
| for( i = 0; sps_num_subpics_minus1 > 0 && i <= sps_num_subpics_minus1; i++ ) { |  |
| if( i > 0 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| if( i > 0 && pic_height_max_in_luma_samples > CtbSizeY ) { |  |
| subpic_ctu_top_left_y[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_width_minus1[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_height_max_in_luma_samples > CtbSizeY ) |  |
| subpic_height_minus1[ i ] | u(v) |
| if( !sps_independent_subpics_flag) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |

# Reference

[1] B. Choi, "AHG12: On sub-picture design", JVET-N0055, March 2019

[2] M. M. Hannuksela, "Use cases and proposed design choices for adaptive resolution changing (ARC)", JVET-M0259, January 2019

[3] Sachin Deshpande, "WD6 of ISO/IEC 23090-2 OMAF 2nd edition", w18587, OMAF, July Gothenburg,

http://wg11.sc29.org/doc_end_user/documents/127_Gothenburg/wg11/w18587.zip

[4] Prakash Kolan, "Streaming E-Sports Event", m46551, MPEG-I Systems, January 2019

http://phenix.it-sudparis.eu/mpeg/doc_end_user/current_document.php?id=66208&id_meeting=177

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).