| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0045 |
| --- | --- |

| Title: | AHG15: cleanup for signaling of minimum QP of transform skip |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jingya Li, Kiyofumi Abe | Tel: Email: | Jingya.li@sg.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to replace syntax “min_qp_prime_ts_minus4” (in range of [0-48]) with “internal_minus_input_bit_depth” (in range of[0-8]) to remove redundancy of signalling and make the design more compact.

# Introduction

In Gothenburg meeting, syntax “min_qp_prime_ts_minus4” is adopted to avoid smaller valued qp used for lossless[1]-[2]. And the intended value for min_qp_prime_ts_minus4 depending on the values of internal and input bit depth is in Table 1.

Table 1: Example value of “min_qp_prime_ts_minus4” given internal and input bit depth

| Input bit depth | Internal bit depth | Value of min_qp_prime_ts_minus4 |
| --- | --- | --- |
| 8 | 8 | 0 |
| 8 | 9 | 6 |
| 8 | 10 | 12 |
| 8 | 11 | 18 |
| 8 | 12 | 24 |
| 8 | 13 | 24 |
| 8 | 14 | 30 |
| 8 | 15 | 36 |
| 8 | 16 | 48 |
| 10 | 8 | 0 |
| 10 | 9 | 4 |
| 10 | 10 | 4 |
| 10 | 11 | 6 |
| 10 | 12 | 12 |
| 10 | 13 | 18 |
| 10 | 14 | 24 |
| 10 | 15 | 30 |
| 10 | 16 | 36 |
| 12 | 8 | 0 |
| 12 | 9 | 0 |
| 12 | 10 | 0 |
| 12 | 11 | 0 |
| 12 | 12 | 0 |
| 12 | 13 | 6 |
| 12 | 14 | 12 |
| 12 | 15 | 18 |
| 12 | 16 | 24 |

It is noted that the values of “min_qp_prime_ts_minus4” signalled can be any value in range of [0-48], which is unnecessarily redundant. It is proposed to replace “min_qp_prime_ts_minus4” with “internal_minus_input_bit_depth” in range of [0, 8] with every value used for an existing case of input and internal bit depth to make the design more compact.

The proposed syntax and semantics changes are as below:

| if( sps_transform_skip_enabled_flag ¦ ¦ sps_palette_enabled_flag ) |  |
| --- | --- |
| min_qp_prime_ts_minus4 internal_minus_input_bit_depth | ue(v) |

min_qp_prime_ts_minus4 specifies the minimum allowed quantization parameter for transform skip mode as follows:

QpPrimeTsMin = 4 + min_qp_prime_ts_minus4		(64)

The value of min_qp_prime_ts_minus4 shall be in the range of 0 to 48, inclusive.

internal_minus_input_bit_depth specifies the minimum allowed quantization parameter for transform skip mode as follows:

QpPrimeTsMin = 6 * internal_minus_input_bit_depth + 4	(64)

The value of internal_minus_input_bit_depth shall be in the range of 0 to 8, inclusive.

An example of value of “internal_minus_input_bit_depth” given internal and input bit depth is given in Table 2:

Table 2: Example value of “internal_minus_input_bit_depth” given internal and input bit depth

| Input bit depth | Internal bit depth | Value of internal_minus_input_bit_depth |
| --- | --- | --- |
| 8 | 8 | 0 |
| 8 | 9 | 1 |
| 8 | 10 | 2 |
| 8 | 11 | 3 |
| 8 | 12 | 4 |
| 8 | 13 | 5 |
| 8 | 14 | 6 |
| 8 | 15 | 7 |
| 8 | 16 | 8 |
| 10 | 8 | 0 |
| 10 | 9 | 0 |
| 10 | 10 | 0 |
| 10 | 11 | 1 |
| 10 | 12 | 2 |
| 10 | 13 | 3 |
| 10 | 14 | 4 |
| 10 | 15 | 5 |
| 10 | 16 | 6 |
| 12 | 8 | 0 |
| 12 | 9 | 0 |
| 12 | 10 | 0 |
| 12 | 11 | 0 |
| 12 | 12 | 0 |
| 12 | 13 | 1 |
| 12 | 14 | 2 |
| 12 | 15 | 3 |
| 12 | 16 | 4 |

# Reference

[1] T. Nguyen, B. Bross, H. Schwarz, D. Marpe, T. Wiegand, “Non-CE8: Minimum Allowed QP for Transform Skip Mode”, JVET-O0405, July 2019

[2] M. Karczewicz, Y.-H. Chao, H. Wang, M. Coban, “Non-CE8: Minimum QP for Transform Skip Mode”, JVET-O0919, July 2019

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).