| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0048-v1 |
| --- | --- |

| Title: | AHG9: On chroma deblocking parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai, Chih-Wei Hsu, Shih-Ta Hsiang, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chia-ming.tsai, cw.hsu, shih-ta.hsiang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to signal chroma tc and β deblocking offset syntax elements in picture header (PH) and slice header (SH) depending on ChromaArrayType. When ChromaArrayType is equal to 0 (monochrome), chroma tc and β deblocking offsets syntax elements are unnecessary and proposed to be skipped and not signalled. Note that, the chroma deblocking syntax elements in picture parameter set (PPS) are not changed and still signalled for 4:0:0. It is because: 1) unlike PH, PPS is not frequently signalled in a bitstream; 2) currently PPS parsing is independent of sequence parameter set (SPS) parsing and does not need ChromaArrayType, which is derived from the syntax element chroma_format_idc signalled in SPS.

# Introduction

In VVC Draft 8 [1], JVET-Q0121 [2] is adopted for supporting chroma tc and β deblocking offset syntax elements in picture header (PH). However, in the current design, these chroma deblocking syntax elements will be unnecessarily signalled when coding monochrome videos. Hence, when the current picture is  monochrome (i.e., ChromaArrayType is equal to 0), chroma tc and β deblocking offsets syntax elements are not used and therefore should not be signalled neither in PH not in SH.

# Proposed method

The related syntax modifications of the proposed method are highlighted in yellow in Table 1 and Table 2. In the proposed design, chroma deblocking offset syntax elements in PH and SH are signalled only when they are used, namely when ChromaArrayType is not 0.

Table 1.  Picture header RBSP syntax

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( deblocking_filter_override_enabled_flag && dbf_info_in_ph_flag ) { |  |
| ph_deblocking_filter_override_flag | u(1) |
| if( ph_deblocking_filter_override_flag ) { |  |
| ph_deblocking_filter_disabled_flag | u(1) |
| if( !ph_deblocking_filter_disabled_flag ) { |  |
| ph_beta_offset_div2 | se(v) |
| ph_tc_offset_div2 | se(v) |
| if( ChromaArrayType != 0 ) { |  |
| ph_cb_beta_offset_div2 | se(v) |
| ph_cb_tc_offset_div2 | se(v) |
| ph_cr_beta_offset_div2 | se(v) |
| ph_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |

Table 2.  General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( deblocking_filter_override_enabled_flag && !dbf_info_in_ph_flag ) |  |
| slice_deblocking_filter_override_flag | u(1) |
| if( slice_deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| if( ChromaArrayType != 0 ) { |  |
| slice_cb_beta_offset_div2 | se(v) |
| slice_cb_tc_offset_div2 | se(v) |
| slice_cr_beta_offset_div2 | se(v) |
| slice_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| … |  |

In PPS, chroma deblocking offset syntax elements are not changed and still signalled for 4:0:0 because PPS is not frequently signalled in a bitstream. In addition, currently PPS parsing is independent of SPS parsing and does not have access to ChromaArrayType, which is derived from the signalled chroma_format_idc in SPS.

# References

- B. Bross, J. Chen, S. Liu, and Y.-K. Wang, “Versatile Video Coding (Draft 8),” Document of Joint Video Experts Team, JVET-Q2001.

- J. Xu, Y.-K. Wang, L. Zhang, W. Zhu, K. Zhang, and H. Liu, “AHG9: On deblocking control parameters,” Document of Joint Video Experts Team, JVET-Q0121.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).