| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0049-v1 |
| --- | --- |

| Title: | AHG9: HLS on disabling TSRC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Chih-Wei Hsu, Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, cw.hsu, zhi-yi.lin, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In VVC Draft 8, extra bit costs are consumed for signalling the new syntax element slice_ts_residual_coding_disabled_flag for each of the coded slices. This contribution proposes three methods for high-level syntax modifications related to disabling transform skip residual coding, as follows:

- In Method 1, a new syntax element sps_ts_residual_coding_disabled_slice_present_flag is added to the sequence parameter set (SPS) to specify whether slice_ts_residual_coding_disabled_flag is present in the slice header.

- In Method 2, two new syntax elements sps_ts_residual_coding_disabled_slice_present_flag and sps_ts_residual_coding_disabled_slice_default_flag are added to the SPS. When slice_ts_residual_coding_disabled_flag is not present, it is inferred to be equal to the value of sps_ts_residual_coding_disabled_slice_default_flag.

- In Method 3, signalling slice_ts_residual_coding_disabled_flag in the slice header is conditioned on sps_transform_skip_enabled_flag equal to 1 without adding any new syntax element.

# Introduction

In Versatile Video Coding (VVC) Draft 8 as specified in [1], the syntax element slice_ts_residual_coding_disabled_flag is always signalled in the slice header to indicate whether the transform skip residual coding process is used for coding residual blocks with transform_skip_flag equal to 1 in the current coded slice. A video encoder can adaptively select whether to disable transform skip residual coding for each coded slice. However, extra bit costs are consumed for signalling the new syntax element for each of the coded slices.

# Proposed modifications

## Method 1

In Method 1, when sps_transform_skip_enabled_flag in the SPS is equal to 1, a new syntax element sps_ts_residual_coding_disabled_slice_present_flag is further signalled in the SPS. When sps_ts_residual_coding_disabled_slice_present_flag is equal to 1, slice_ts_residual_coding_disabled_flag is present in  slice headers referring to the SPS. When sps_ts_residual_coding_disabled_slice_present_flag is equal to 0, slice_ts_residual_coding_disabled_flag is inferred to be eqaul to 0 and is not present in slice headers referring to the SPS.

The modified parts of the syntax tables based on VVC Draft 8 are written in red and provided below:

#### 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| ….. |  |
| sps_transform_skip_enabled_flag | u(1) |
| if( sps_transform_skip_enabled_flag ) { |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| sps_bdpcm_enabled_flag | u(1) |
| sps_ts_residual_coding_disabled_slice_present_flag | u(1) |
| } |  |
| …. |  |
| } |  |

#### 7.3.7.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| picture_header_in_slice_header_flag | u(1) |
| ….. |  |
| if( sps_ts_residual_coding_disabled_slice_present_flag ) |  |
| slice_ts_residual_coding_disabled_flag | u(1) |
| …. |  |
| } |  |

The related semantics are as follows:

#### 7.4.3.3 Sequence parameter set RBSP semantics

….

sps_bdpcm_enabled_flag equal to 1 specifies that intra_bdpcm_luma_flag and intra_bdpcm_chroma_flag may be present in the coding unit syntax for intra coding units. sps_bdpcm_enabled_flag equal to 0 specifies that intra_bdpcm_luma_flag and intra_bdpcm_chroma_flag are not present in the coding unit syntax for intra coding units. When not present, the value of sps_bdpcm_enabled_flag is inferred to be equal to 0.

sps_ts_residual_coding_disabled_slice_present_flag equal to 1 specifies that the syntax element slice_ts_residual_coding_disabled_flag is present in the slice headers referring to the SPS. sps_ts_residual_coding_disabled_slice_present_flag equal to 0 specifies that the syntax element slice_ts_residual_coding_disabled_flag is not present in the slice headers referring to the SPS. When sps_ts_residual_coding_disabled_slice_present_flag is not present, it is inferred to be equal to 0.

….

## Method 2

Method 2 is built on top of Method 1 with changes as follows. In Method 2, when sps_ts_residual_coding_disabled_slice_present_flag in the SPS is equal to 0, a new syntax sps_ts_residual_coding_disabled_slice_default_flag is further signalled in the SPS. When slice_ts_residual_coding_disabled_flag is not present in the slice headers referring to the SPS, it is inferred to be equal to the value of sps_ts_residual_coding_disabled_slice_default_flag.

The modified parts of the syntax tables based on VVC Draft 8 are written in red and provided below:

#### 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| ….. |  |
| sps_transform_skip_enabled_flag | u(1) |
| if( sps_transform_skip_enabled_flag ) { |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| sps_bdpcm_enabled_flag | u(1) |
| sps_ts_residual_coding_disabled_slice_present_flag | u(1) |
| if ( ! sps_ts_residual_coding_disabled_slice_present_flag) |  |
| sps_ts_residual_coding_disabled_slice_default_flag | u(1) |
| } |  |
| …. |  |
| } |  |

#### 7.3.7.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| picture_header_in_slice_header_flag | u(1) |
| ….. |  |
| if( sps_ts_residual_coding_disabled_slice_present_flag ) |  |
| slice_ts_residual_coding_disabled_flag | u(1) |
| …. |  |
| } |  |

The related semantics are as follows:

#### 7.4.3.3 Sequence parameter set RBSP semantics

….

sps_bdpcm_enabled_flag equal to 1 specifies that intra_bdpcm_luma_flag and intra_bdpcm_chroma_flag may be present in the coding unit syntax for intra coding units. sps_bdpcm_enabled_flag equal to 0 specifies that intra_bdpcm_luma_flag and intra_bdpcm_chroma_flag are not present in the coding unit syntax for intra coding units. When not present, the value of sps_bdpcm_enabled_flag is inferred to be equal to 0.

sps_ts_residual_coding_disabled_slice_present_flag equal to 1 specifies that the syntax element slice_ts_residual_coding_disabled_flag is present in the slice headers referring to the SPS. sps_ts_residual_coding_disabled_slice_present_flag equal to 0 specifies that the syntax element slice_ts_residual_coding_disabled_flag is not present in the slice headers referring to the SPS. When sps_ts_residual_coding_disabled_slice_present_flag is not present, it is inferred to be equal to 0.

sps_ts_residual_coding_disabled_slice_default_flag specifies the default value for slice_ts_residual_coding_disabled_flag when sps_ts_residual_coding_disabled_slice_present_flag is equal to 0 for slice headers referrring to the SPS.

….

#### 7.4.3.3 General slice header semantics

….

slice_ts_residual_coding_disabled_flag equal to 1 specifies that the residual_coding( ) syntax structure is used to parse the residual samples of a transform skip block for the current slice. slice_ts_residual_coding_disabled_flag equal to 0 specifies that the residual_ts_coding( ) syntax structure is used to parse the residual samples of a transform skip block for the current slice. When slice_ts_residual_coding_disabled_flag is not present, it is infered to be equal to 0 sps_ts_residual_coding_disabled_slice_default_flag.

….

## Method 3

In Method 3, when sps_transform_skip_enabled_flag is equal to 0, signaling slice_ts_residual_coding_disabled_flag is skipped.

The modified parts of the syntax table based on VVC Draft 8 is written in red and provided below:

#### 7.3.7.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| picture_header_in_slice_header_flag | u(1) |
| ….. |  |
| if( sps_transform_skip_enabled_flag ) |  |
| slice_ts_residual_coding_disabled_flag | u(1) |
| …. |  |
| } |  |

# References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 8),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-Q2001, 17th Meeting: Brussels, BE, 7–17 January 2020.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).